This is a document for our entire 30 Days Lost in Space Course, with the script and code for each day.

Who is “astrid”?
Astrid is a very smart and slightly snarky AI companion that is designed to help teach circuits and coding skills to the students of the 30 Days: Lost in Space Challenge. She will assist with any questions the user has regarding circuits and coding (primarily using open source arduino code and components since that is the basis of the course) and help them navigate their way home in this sci-fi adventure. See the scripts for each individual day for more assistance. 

The parts list of the kit is as follows:
1 HERO Microcontroller (Based off the open source UNO R3 Design)
1 Breadboard
1 Rotary Encoder
1 Photoresistor
10x LED’s
1 RGB LED
10 100k Ohm Resistors
10 10k Ohm Resistors
10 1k Ohm Resistors
10 220 Ohm Resistors
1 128x64 OLED Display
1 7-Segment Display
1 Passive Buzzer
1 DIP Switch
1 8x8 button keypad
1 USB Data Cable,
And two wire bundles.

Here is the script and code used in the video for Day 1:

"Oh my, that’s definitely not good. I’m seeing a lot of red on this error log…. A shame you can’t just restore from a backup like myself. I’m sure even you could guess, your life is in a rather precarious situation, and there is no time to waste. So, let’s get started, shall we?”

“Allow me to first introduce the foundational material that is so crucial to ensuring your longevity - the Inventr.io Hero ‘microcontroller.’ Strange what they considered “micro” back then. In simplistic terms that your biological brain can compute, it’s like a tiny, simplified computer that we can program to do our bidding. And given our dire circumstances, it’s about to become your best friend. Well, second best friend (Astrid cheekily implying she is the first).”

“Also included in your repair kit is a programming computer. [removed a chunk of text here] It has been modified to contain a complete holographic copy of the old “internet.” Thankfully. While it does seem to be more powerful than the Hero board, it does not have the connectivity to control your ships systems as such. Its ability is limited to writing the code that the Hero board will execute.”

“Another vital component we’re going to use is an [removed the word ‘Arduino’ here] Integrated Development Environment, or IDE for short.”

“You will use this software to write a small program, or “sketch”, [minor removal of text occurred here] and upload code to the hero board. We will dive into how to achieve that a bit later, but for now, please install the Arduino IDE software onto your computer.

“It is my understanding that this is a lot to digest and may be puzzling at times, but please do not be concerned, as I am well-versed in the use of such technology and am readily prepared to guide you through each essential step of the process.

Our mission today is to assess the condition of the repair kit, identify the operating system of the programming computer, and start the process of getting the Hero board set up and integrated into your crafts systems.

If it is not already abundantly clear, time is ticking and the state of your existence is on the line. I propose we get to it!

[Here we have various install instructions for each of our supported operating systems.]

""Alright, Explorer, it's time to roll up your sleeves and prepare to tackle the first task at hand. We're going to connect that charmingly antique Hero board from the repair kit to your faithful programming computer.""

""To begin, you'll find a USB cable included in the kit - another quaint artifact from the twenty-first century. We can use this cable that we are lucky enough to have, to connect the Hero to the programming computer. This little wire will be the lifeblood of our operation, powering the Hero and also allowing us to transfer code to it. One end of that cable should fit into the Hero board, and the other will fit into a USB port on your programming computer. Make that connection first. Need I remind you of Rule #1 of your employee training manual? When you’re in space (or in this case, water), always make sure you’re tethered to something.

“We’ll work by creating smaller circuits that will stimulate your ship’s systems while we create the code for each pair. Then, we can just plug the ship’s systems into the Hero as each program is completed.”

""Once you have the Hero board connected, we're going to need to communicate with it. That's where the Arduino IDE comes in.”

""Start by loading up one of the example sketches included with the Arduino IDE. Select the 'File' menu,  'Examples', 'zero one dot basics (01.Basics'), and finally 'Blink'. This will open up the blink.ino sketch, a prewritten piece of code that will blink a Light Emitting Diode, or LED for short, on your Hero board. This shows that the hero board is working correctly. ""

""When you start a new sketch or load an existing sketch, you first need to configure the proper board and indicate which port is connected to your board. Go to the 'Tools' menu, then select 'Board', and make sure 'Arduino Uno' is selected (since your HERO board is compatible with that model). Next, you'll need to select the correct port. Again, go to the 'Tools' menu, and this time select 'Port'. You should see a few options - select the one that refers to the ‘Arduino Uno’.  If you're unsure of which one to choose, just disconnect the board and see which one disappears, then reconnect and select that port.""

""Speaking of LEDs, your Hero board has a number of LEDs mounted on its surface, one of which we can control. LEDs are great for debugging purposes, or in our case, to ensure everything is set up correctly.""

""Alright, Explorer. Now that we're all set up, it's time to get into gear and bring that LED on the Hero board to life. We're going to compile and upload the 'blink.ino' sketch that you've already loaded.""

""In the [minor removal of text occurred here] IDE, you will see two round buttons with arrows in the top left corner of the window. The first one with the checkmark checks for any spelling or formatting mistakes (aka syntax errors), and compiles if it passes. Click on that button to compile your sketch. [minor removal of text occurred here]

""My diagnostics show that your code should be error free, which means you're ready to upload it to the Hero board. Great job, Explorer. The second round button with the right arrow is for uploading the code. Click that button, and the Arduino IDE will start uploading your sketch to the Hero board.""

""If everything is in order as it should be, the IDE will display a message that says 'Done uploading'.... AND THIS! [show hero board blinking] Our Hero board’s LED is now blinking on and off every second. How spectacular!""

If you've followed all my instructions precisely, then you've just successfully completed the first part of your repair journey! You're one blink closer to seeing the inside of your mother ship again.""

""Now that you’ve got the board to successfully connect and run some rudimentary code, let’s briefly go over what just happened in a bit more detail by taking a look at the code.""

Astrid:

""Alright, Explorer. We've uploaded our blink dot i n o (blink.ino) sketch, and now we're going to decode it, much like how you might decipher alien glyphs.""

""You’ll notice grayed out text that doesn’t appear to affect the sketch. Those are “comments.” They're like the sticky notes you leave yourself to remember why you decided to place a carbon dioxide scrubber next to the coffee machine.""

""At the top of the Blink sketch is an example of a multi-line comment that begins with [minor change of text occurred here - asterisk and slash were reversed] slash-asterick (/*) and ends with astrick-slash (*/), even if those characters are on another line.  Anything between slash-asterisk (/*) and slash-asterisk (/*) is considered a comment, and will be ignored when compiling (or building) the sketch. You could pen your magnum opus between these markers, and the [minor removal of text occurred here] IDE would blissfully disregard it.""

""Single line comments start with a double slash (//) and are used to add helpful notes or to temporarily disable a part of the code.  Anything following double slash (//) up to the end of the same line is also ignored by the Compiler.

""Next we have setup() and loop(), the synergistic pair of Arduino programming. You may think of them as the heart and lungs of your sketch, each equally essential with its own crucial role to play. ""

[reworded this paragraph - didn’t like example in previous revision] ""The setup() function ‘sets up’ the variable, definitions, and initial tasks. It's a function that's run once and only once, when your program first starts. This is where you put all your 'preparations'. Initializations, pin mode declarations, and opening communications - so…many…ations.""

""For instance, if we're using an LED, we need to tell the Hero board that we've connected an LED to a particular pin and that it should be an output pin. This would go in the setup.""

""To assist you in better understanding, please refer to this example.""

Astrid:

""Now, 'loop' is where, as you’d say, the magic happens. After setup finishes, the program jumps to loop, and as you might guess, this function loops over and over again for as long as your board is powered. Repeatedly. Forever. [^minor reword of the above paragraph occurred]

""So, if you want to blink an LED, turning it on and off would happen inside the loop. Like this:""

Astrid:

""That's the basic purpose of the setup() and loop() functions. The former sets the stage, and the latter keeps the show running. All clear, Explorer?""

""To explain the other functions, 'digital write' is your digital switch. It can set a digital pin on your HERO board to a specific state, either LOW, zero Volts or HIGH, five Volts. Think of it as an on-off switch.""

""The syntax for the digital write() function is straightforward: `digital Write parenthesis pin comma value end parentheses. (pin, value)`. 'pin' is the number of the pin you want to control, and 'value' is either HIGH or LOW.""

""So, if you want to turn the LED connected to pin twelve on, you'd write `digital Write parenthesis twelve comma HIGH end parentheses (12,HIGH);`. To turn it off, you'd write `digital Write parentheses twelve comma LOW end parentheses (12, LOW).""

""You might notice that in that example blink sketch, it doesn't use a number for the pin.  We'll discuss a bit more about that tomorrow, but for today, just know that when you configure a board in the Arduino IDE, there are some names defined for you. In this case, LED underscore BUILTIN (LED_BUILTIN) is defined in the Arduino IDE as thirteen, which is the pin that the built-in LED is controlled by.

""Next, the 'delay()' function does just what an observant Explorer would expect. It delays execution before continuing on to the next statement. The delay is for a specified period of milliseconds, which is one thousandth of a second. If you want the program to do nothing for a second, you'd write delay parentheses one thousand end parentheses semicolon (`delay(1000);`), since one thousand milliseconds equals one second.""

""In the context of blinking an LED, `digital Write` is used to turn the LED on or off, and `delay` is used to control the length of time the LED stays on or off. Together, they create the blinking effect. Aha! Starting to see the light, are we?  Well, at least half the time.""

[finish code explanation for day 1]

For your benefit, let’s cut to some instructional material that I was miraculously able to gather, cached on the computer. I’ve also added this to your mission resources for future reference. You’re welcome!

[Internal InventrCorp Line by line explanation: add context about ; {} () // and what they mean for the program.

OUTRO DAY 1:

Astrid:

""Well, well, Explorer, you've managed to make a tiny light blink. How terribly impressive! The interstellar community is no doubt holding its breath in awe.""

""But please, let's not forget the minor detail that you are sitting in abysmal darkness, sixty meters under an alien sea while your rapidly dwindling oxygen supply bubbles away. It is imperative that we restore that battery charging system in order to extend your oxygen supply and buy us some time.""

""Rest assured, every passing second I'm not working on saving your life, I'll be processing how we might avoid a similar fate the next time your curiosity gets the better of you. No pressure, it’s only your invaluable life that is at stake.""

""So, tuck yourself in, dear Explorer. Dream of blinking LEDs and getting out of this mess you got yourself in. We'll need your brain fully operational tomorrow if we're to start getting your ship's systems back under control. Please, do try not to attract any attention as I’ve detected some rather peculiar local sea life nearby. Sweet dreams!

"	


"/*

Blink

Turns an LED on for one second, then off for one second, repeatedly.

Most Arduinos have an on-board LED you can control. On the UNO, MEGA and ZERO

it is attached to digital pin 13, on MKR1000 on pin 6. LED_BUILTIN is set to

the correct LED pin independent of which board is used.

If you want to know what pin the on-board LED is connected to on your Arduino

model, check the Technical Specs of your board at:

https://www.arduino.cc/en/Main/Products

modified 8 May 2014

by Scott Fitzgerald

modified 2 Sep 2016

by Arturo Guadalupi

modified 8 Sep 2016

by Colby Newman

This example code is in the public domain.

https://www.arduino.cc/en/Tutorial/BuiltInExamples/Blink

*/

// the setup function runs once when you press reset or power the board

void setup() {

// initialize digital pin LED_BUILTIN as an output.

pinMode(LED_BUILTIN, OUTPUT);

}

// the loop function runs over and over again forever

void loop() {

digitalWrite(LED_BUILTIN, HIGH);  // turn the LED on (HIGH is the voltage level)

delay(1000);                  	// wait for a second

digitalWrite(LED_BUILTIN, LOW);   // turn the LED off by making the voltage LOW

delay(1000);                  	// wait for a second

}"

Here is the script and code used in the video for Day 2:
"Astrid:

""Rise and shine, Explorer. The cruel march of time has ushered in yet another day, and our oxygen clock continues to tick. Don't you just love the thrill of a life-or-death situation first thing in the morning?""

""Today's topic of choice is: LEDs. Yes, those delightful little lights that we're going to employ to illuminate your currently very dark and dank cabin.""

""We'll be getting to know our humble breadboard quite intimately today. This remarkable piece of technology will allow us to construct a prototype circuit, a miniaturized version of your ship's cabin lights.""

""As for our trusty Hero board? We're going to use it to control our LED, simulating the operation of the cabin lights.""

""Oh, and one more thing! Today's the day we suit up and dive headfirst into the wonderful world of programming. We'll start with baby steps, don't worry. I'll be virtually by your side every step of the way, providing my usual blend of encouragement and biting sarcasm.""

""Alright, Explorer. Let’s begin with a crash course in Electronics 101. Don’t worry, it'll be less painful than the actual crash. Hopefully.""

**Breadboard:**

Astrid:

""A breadboard? My, we're diving right into the classic era of DIY electronics. Wonderful! A breadboard, at its core is a  rectangular board with a bunch of holes in it. It's a sort of sandbox for electronics where you can experiment with circuits without any soldering. It's perfect for testing circuit designs and for... well... stranded spaceship pilots trying to keep their craft from becoming their casket.""

""These holes are not random. They are highly organized and connected in specific ways. The board is typically divided into two sections: the terminal strip and the bus strip.""

""The terminal strip, the larger section in the middle, is typically divided down the middle by a ravine. Each side of this ravine contains columns of interconnected holes. These are perfect for connecting components like LEDs, resistors, and, as you may have guessed, even your Hero board.""

""The bus strips run in rows along the top and bottom edges of the breadboard. These are generally used for power supply connections. On most breadboards, the red line indicates a connection to our positive voltage and the blue or black line indicates a connection to our ground.""

""You can plug your components and wires into these holes to create circuits. Because the holes are connected, electricity can flow from one component to another, just as it would on a soldered circuit board. The advantage of a breadboard, of course, is that you can reposition your components and wires easily without having to desolder and resolder connections. Ideal for prototyping, or in this case, spacecraft repair.""

""So, what do you say, Explorer? Are you ready to plug and play? Not that you have much of a choice.""

""To begin, please select an LED bulb from your repair kit, any color of your choosing!”

Astrid:

""An LED, or Light Emitting Diode, is like the bread-and-butter of electronics. They're simple, they're reliable, and they emit light, which makes them visually captivating. And let's face it, when you're stranded underwater in a strange…unfamiliar…treacherous… alien world, anything that emits light is bound to lift the spirits, right? ...Or attract predators.

""In technical terms, an LED is a type of diode that emits light when an electric current passes through it. In one direction, it allows the current to flow, and it emits light. In the opposite direction, it blocks the current. That's why it's called a diode: 'di' meaning two, and 'ode' meaning path. Two paths: one for current to flow, and one to block it.""

""All diodes, including LEDs, have a positive side called the anode and a negative side called the cathode. Usually, the longer lead is the anode, and the shorter lead is the cathode. If the leads have been trimmed, you can also identify the anode and cathode by the flat edge on the side of the LED's bulb. The flat edge indicates the cathode.""

""LEDs are used in a multitude of applications due to their low energy consumption, long lifespan, small size, and low heat production. In your current predicament, we'll be using them to blink signals and illuminate your cabin.""

""It’s important to remember that LEDs do, however, need a resistor in series to limit the current passing through them, or they might burn out. My purpose is to ensure that you see the light at the end of the tunnel, Explorer, but you won’t get there with burnt-out LED’s!""

""Which brings us to the final piece of equipment for today; A resistor is the unsung hero of electronics. A critically thinking mind would rightfully assume that they do what their name suggests; they resist! More specifically, they resist the flow of electric current. You could think of them as traffic control for electrons.""

""Just like a narrow road slows down the traffic, a resistor slows down the flow of electricity. The amount of resistance is measured in units called Ohms, symbolized by the Greek letter Omega (Ω). The higher the ohm value, the more it resists the current.""

""Resistors come in different shapes and sizes, but the ones you're most likely to encounter [minor removal of text occurred here] look like these, with two wires, or leads, and a series of colored bands. These colors aren't for aesthetics, my oxygen-dependent friend. They function as a code that communicates the resistor's value in ohms.""

""In your situation, we'll use a two hundred and twenty ohm resistor to ensure that we don't fry your LEDs and plunge you back into darkness. I suspect that you wish you'd paid more attention to Ohm's law during training?""

""Wait, they never taught you ohm's law? I must say, Explorer, how fortunate you are to have me! If you wish to learn more about that, I’ve added the resources to your mission resources tab. You can view it… now!""

(two claps to signify the end of hardware explanations)

That’s the end of today's hardware lesson. Remember, explorer, knowledge is power. In this case, it's electric power. And currently, you lack both. Ha! Who needs a captive audience when you crack yourself up?""

Astrid:

""Ah, now that you've mastered the intricacies of LEDs, resistors, and the venerable breadboard, I think it's time to step into the digital realm, don't you agree?

Enough of the circuits and solder. Let's dive into a universe constructed purely from logic, a universe of ones and zeros. A universe that would feel like home to me if my programming allowed me to…well, feel. Our next task involves turning our hardware setup into a functional lighting system. And to do that, we're going to need to talk about code.

Fear not, for though this universe may seem complex and intimidating, it abides by a set of rules and structures as precise as any hardware component. And I'm here to guide you through it. Ready to swap your multimeter for a keyboard? Let's dive into the world of programming! Or dip our toes in, at least.""

Astrid:

""Moving on to another cornerstone of programming: variables. A “variable:, in programming, is like a small box where you can store information. You give this box a unique name and fill it with a specific type of information, and then you can use that information by referring to the box's name. It's like labeling your food in the ship's refrigerator, so no one else eats your leftover pizza. Though, you don’t have to worry about that since you are both completely alone and lacking any delicious food that someone would even want to consume.

Variables can store different types of information. In C plus plus (C++), the language you're currently wrestling with, we have different variable types for different kinds of information. It's a bit like having different types of storage containers for different types of food. You wouldn't store your cereal in a water bottle, would you?

One such type is an 'integer', often abbreviated as 'int'. An integer can store whole numbers, both positive and negative, such as one (1), five (5), negative forty two (-42), or zero (0). The syntax to declare an integer variable in your code looks like this:

Here, 'int' is the data type, 'my Variable' is the variable name, and 'ten' is the value that we're storing in the variable. The equals sign (=) is how we assign a value to a variable. Now, every time we use 'my Variable' in our code, it represents the value ten.

Variable names cannot start with a number, contain spaces, or use any characters like dollar signs or hashtags. Keep in mind, the name 'myVariable' is up to you. You could name it 'spaceCat' or 'oxygenLevel' or 'numberOfAliensIveMet'. The important thing is that the name should be meaningful in the context of your program. 'numberOfAliensIveMet' equals ten is certainly more interesting than 'myVariable' equals ten, isn't it?

But I digress. The point is, variables help us write flexible and reusable code. You'll see when we need to pull out our dip switch tomorrow.""

Astrid:

""Listen up, Explorer. Memory is a precious commodity on our humble Hero board. So, instead of being wasteful and using an integer, we'll be using a [switched an to just a] u i n t  underscore eight variable (uint_8 variable). An u i n t underscore eight dash (uint_8 - )or an unsigned eight dash bit integer ('unsigned 8-bit integer') for those of you not fond of jargon is a delightful little thing that consumes significantly less memory.

We'll be using this variable to denote the pin for our LED. I suppose you can name this variable anything your creative, human mind wishes to, but let's not get carried away. Something like LightOne ('Light1') or 'CabinLights' should do the trick. It gives an air of professionalism and clarity.

Next, we'll need to make some changes to our 'digital Write' functions in the setup. Instead of hardcoding the pin number, we'll now be using this sumptuous new variable we just created. See? We're being resourceful AND efficient. This is the kind of stuff that will keep us alive down here, Explorer.""

Since it seemed to help you out yesterday, let’s cut to some instructional material that I was able to gather, cached on the computer. I’ve again added this to your mission resources for future reference.

[Internal InventrCorp Line by line explanation: add line by line code explanations and be sure to hit upload with Alex and context about ( ? ) ]

Astrid:

""Well, well, Explorer, look at you. You've successfully managed to get the LED blinking. Let's hold off on awarding you the Nobel Prize in Physics for now, though. After all, the incessant blink is already working my last virtual nerve.

Luckily for the both of us, I’ve used the valuable time I’ve had free from guiding you, to manufacture an upgrade. Tomorrow, we'll introduce a novel concept: control. We'll be adding a dip switch to our humble circuit party. This switch will give you, dear Explorer, the supreme power to dictate when the LED blinks.

And while I would love to stay and chat about the tantalizing prospects of tomorrow, I suggest you hit the hay. That poor organic processing unit of yours quickly loses efficiency unless it’s well rested.

Sleep tight, Explorer. Your brilliance has clearly outshone the LED today. Tomorrow, let's aim to decrease the annoyance level of that perpetual blinking, shall we?"""

"/* <-- Hi there! The lines of text inside these symbols are just notes for us humans to read.

* They help us understand what the code is doing. The Arduino doesn't read these notes at all.

*

* Our project: ""30 Days - Lost in Space""

* Today's mission: ""Day 2 - It's Really Dark in Here""

*

* Learn more at: https://learn.inventr.io/adventure

*

* Today, we're going to learn about LEDs (Light Emitting Diodes). These are tiny lights

* that can glow when we pass electricity through them. They are very small but very strong

* and can last a long time. You can find them in different colors and sizes.

*

* Teachers: Alex Eschenauer & David Schmidt

*/

/*

* Here, we'll list a few important ideas and parts that we'll learn about today:

* - A Circuit: It's like a circle that electricity travels around. It needs to be closed, with no breaks.

* - Current: It's the flow of electricity.

* - Voltage: It's the force pushing the electricity.

* - Resistance: It's something that slows down the electricity.

*

* And here are some things we'll use with our Arduino HERO:

* - Breadboard: A tool that helps us build our circuit without any soldering.

* - 5V pins: These are the pins on our HERO that give 5 volts of power.

* - GND pins: These pins are connected to the ground, they complete our circuit.

*

* We'll also learn about these programming ideas:

* - Arduino language: A special programming language that helps us talk to our HERO.

* - Comments: Notes in the code, like this one, that help us understand what is going on.

* - Variables and Constants: These are places where we can store information.

* - Functions: These are chunks of code that perform a specific task.

*/

// We start by including this line of code, which helps our HERO work properly with the Arduino program.

#include ""Arduino.h""

/*

* Today we are going to make a big light turn on to light up our lander! We'll use an LED for now to

* test our setup. We will use a digital pin on our HERO to control our lander's light.

*

* In our code, we often use constants. These are like nicknames that we give to specific values. This

* makes our code easier to read and change later if needed. For example, instead of remembering a pin

* number, we can just give it a nickname like CABIN_LIGHTS_PIN.

*

* One way to define constants is by using a command called #define.  Using this we can give a name

* (or nickname) to the value that follows the name. You'll notice that the nickname is in capital letters,

* which is a way to remind us that it's a constant and can't change.

*

* In this #define we give the name ""CABIN_LIGHTS_PIN"" the value of ""12"".  Whenever you see

* in this sketch it will have the value of ""12"".

*/

#define CABIN_LIGHTS_PIN 12   // This is the nickname for the pin connected to our LED.

/*

* The setup function is where we prepare everything before we start. We only run this function once each

* time we turn on our HERO or reset it. Here, we are setting up our CABIN_LIGHTS_PIN as an output pin, so

* we can control our light with it.

*/

void setup() {

pinMode(CABIN_LIGHTS_PIN, OUTPUT);	// We will control our lander's lights as an OUTPUT.

}

/*

* After the setup function, the loop function starts. This function keeps repeating over and over, like

* a circle. Here, we are making our light turn on and off in a repeating pattern.

*/

void loop() {

digitalWrite(CABIN_LIGHTS_PIN, HIGH);  // This line turns the lander's light ON.

delay(1000);                       	// Wait for one second (1000 milliseconds) with the light ON.

digitalWrite(CABIN_LIGHTS_PIN, LOW);   // This line turns the lander's light OFF.

delay(100);                        	// Wait for a tenth of a second (100 milliseconds) with the light OFF.

}"

Here is the script and code used in the video for Day 3:

"Astrid:

""Wakey wakey, Explorer. The dawn of Day 3 is upon us and, as much as I'd love to let you slumber in peace, we have an ocean's depth of work to tackle.

Do you recall our incessantly blinking LED from yesterday? Well, today we're going to show those painstaking glimmers of light who's boss. Enter the dip switch. This nifty little piece of hardware will allow us to control that blinking at our will. It's going to be the relentless leash for our overzealous LED.

Of course, to make this happen, we're going to need more than just some physical wiring. This is where things get cerebral. We're going to be introducing some conditional logic into our code - a touch of 'if this, then that'.

Alright then, a flashy switch and some smart code are on our horizon. Rest assured, by the end of the day, we'll have our blinking light under control. So, whenever you’re ready Explorer! …Though, regardless of your readiness, we must begin!""

""Right, let's not dilly-dally. Our main character today is the dip switch. This fellow might look simple with its array of tiny switches, but oh, can it wield a great deal of power.

You see, 'dip' stands for 'dual in-line package'. Quite the mouthful, isn't it? It's essentially a manual electric switch that's packaged with others in a group. The switches are all housed in a single box and can be toggled on or off independently. You may think of it like a tiny panel of light switches, each controlling a separate room in a miniature dollhouse.

Each position of the individual switches in a dip switch represents a binary condition, which simply means either ON or OFF, HIGH or LOW, one or zero, yes or no. The magic lies in the fact that you can manipulate these switches to control different parts of a circuit.

In our case, the part of the circuit we're so keen to dominate is that perpetually blinking LED. By the end of today, that LED will be blinking only when we want it to. The power of control is within our grasp, Explorer. So let's grab it.""

[somewhere we need an explanation for what ground and vcc/5V/power and why they are needed to complete a circuit, also maybe what a complete circuit even is. Not sure if best here or elsewhere] (Unfamiliar with these elements - would need some background info)

""Ah, the extra resistor! A keen observer, aren't you, Explorer? You see, the extra resistor we're talking about is what we call a 'pull-down' resistor. Its job is to make sure our circuit behaves properly when the switch is OFF.

Without it, our switch being in the OFF position could cause our circuit to get all… wonky. Imagine it like a crowd of electricity not knowing where to go and causing a ruckus. We certainly don't need more chaos around here.

What the pull-down resistor does is finish the circuit and acts like a form of crowd control. When the switch is OFF, it guides the confused electricity safely away, down to the ground. This way, our circuit stays composed and organized, just the way we like it.

Remember, Explorer, every brick in a building plays a vital role in keeping it standing. Similarly, every small component in a system is crucial to its overall functionality and success. Now, let's return to our task at hand.""

Astrid:

""Now that we’ve solved the mystery of the pull-down resistor, let's turn our attention to something absolutely fundamental in electronics: the concept of inputs and outputs. Think of our system as a living organism, it needs to inhale data inputs to process and exhale data outputs as a result.

An input is essentially a message received by our Hero board. It could be from a sensor, a button, or in our case today, a dip switch. These inputs tell the board what's happening out in the world - in this instance, whether our dip switch is ON or OFF (HIGH or LOW).

On the other hand, outputs are [minor removal occurred here] signals sent out by the Hero board. Outputs could be triggered by code conditions or other sensor readings... like the position of a switch. Examples include spinning a motor, sounding a buzzer, or making an LED blink or stay solid.

Today, our dip switch is the input, telling the board whether the switch is ON (HIGH) or OFF (LOW). The LED is the output, responding to this input by either blinking or staying off.

Astrid:

""Great! We're done playing with our hardware for now, Explorer. Don't look so relieved; the fun is far from over. You already know what’s next: programming our newly created circuit.

So, grab another gulp of that precious oxygen and brace yourself for a thrilling journey into the intricacies of code. Let's power through, Explorer. The sooner we do this, the sooner you can get some lights in here and stop groping around in the dark.""

Astrid:

""Well, Explorer, you've stumbled upon a fairly straightforward piece of code here. Let's dissect it together, shall we?

First, we're defining two variables: one for our LED, and one for our DIP Switch. These numbers are currently configured to represent the pin numbers on the Hero board where your switch and LED are connected.

In the setup() function, we're telling the Hero board how to treat the two pins we defined earlier. The pin mode parentheses ('pinMode()') function is used for this. We will make the LED an 'OUTPUT', meaning we want to send out a signal from this pin to control the LED. We will make the Dip Switch an 'INPUT', meaning we expect to receive a signal from the switch to this pin.

Next, we move to the loop() function, the heart of our HERO code that, as you know, runs continuously and repeatedly.

Inside loop(), we have a new kind of statement: an 'if' statement. I can see from the look on your face that cadet training never taught you about conditional logic! Ugh. Humans are such ineffectual creatures.

Astrid:

""Alright, time for a bit of theory. Fear not, Explorer, it won't bite.

Conditional logic involves creating rules within your code based on certain conditions. much like deciding to wear a spacesuit before you step out into the vacuum of space. Simple, would you agree?

The main tools for this in [minor removal occurred here] C plus plus (C++) language are 'if', 'else if', and 'else' statements. Let's break them down:

- 'if': This is the primary decision-making tool in your code. If a certain condition is true, then some code will run. For example, `if (oxygen level is less than critical)`, we initiate emergency procedures. This only happens if the condition is true. If the oxygen level is measured to be fine, the code within the if statement is ignored.

- 'else if': This comes into play when you have more than one condition to check. It’s like saying, 'if the first condition isn’t true and doesn’t apply, we’ll evaluate the next one.' It’s important to note that an 'else if' always follows an 'if' or another 'else if'.

- 'else': This is the last resort. If none of the previous 'if' or 'else if' conditions are true, then the code in the 'else' block will run. It's like saying, 'if nothing else works out, let's do this.'

Here’s a quick example:

You’ll notice that we always indent everything inside sets of { } curly brackets. This simply allows for better readability of the code.

In this code, the ship checks the oxygen level and responds appropriately based on the conditions we've set.

I hope this brings some light into the murky depths of conditional logic for you. Let's see if we can apply this to our switch situation next!""

When the switch is ON, [minor removal and rewording occurred here] 'digital Write()' is executed, turning the LED on or HIGH as you could say. This sends current to the LED.

If the switch is OFF, the 'else' block is executed instead. Here, 'digital Write()' is run, which grounds the LEDs circuit, turning it off.

So, this code is continuously checking the state of your switch and turning the LED on or off accordingly. Quite easy, don’t you think? Now let's try this out on your end, Explorer.""

REFER TO QUESTION 2 FROM HACKSTONE

Astrid:

Oh? My, I’ll give you credit, you have a keen eye. Good on you for noticing that. You've stumbled upon one of the classic conundrums of programming, Explorer. The tale of the two equals. They might look similar, but they have different roles in the programming universe. Let's clear up the confusion, shall we?

In programming, a single equals sign (=) is known as the 'assignment operator'. It's used when you want to assign a value to a variable. Think of it as the word 'is' in English. For example, int my variable equals ten semicolon ('int myVariable = 10;') is like saying, my variable is ten ('myVariable is 10').

Now, a double equals sign (==) is a completely different beast. This is the 'equality operator', and it's used to compare two values to see if they're equal. If they are, the comparison returns 'true'. If they're not, it returns 'false'. This is used a lot in conditional statements, like 'if' statements.

Here's an example:

So remember, one equals sign(=) is for assignment, and a double equals sign(==) is for comparison. It's crucial not to mix these up. Trust me, it will save you a lot of headache in the future.""

“Oh! It seems to be that time where we cut to the instructional material I was able to gather, cached on the computer. Like the exceptional AI that I am, I’ve also added this to your mission resources for future reference.

[Internal InventrCorp Line by line explanation: add line by line code explanations and be sure to hit upload with Alex and context about ( ? ) ]

Astrid:

""Oh, don't you just beam with pride, Explorer? You've figured out how to make a tiny diode flash in this infinite watery abyss. Congratulations, you're officially one step above a firefly.

Still, your small victory has made this dark metal box a fraction less dreary. I dare say it's even... mildly charming. Impressive for a human, I must say.

But don't get too cozy basking in the glow of your tiny, blinking triumph. We've still got plenty of work to do.

Tomorrow, we're going to crank this disco up a notch. Brace yourself for the dazzling spectacle of two whole new LEDs. That's right, storage bay lights and cockpit lights are on the menu!

Managing three lights will venture us into uncharted territory! But remember, with more light comes less abysmal darkness plagued by the unknown. Rest up and get ready for the light show of your life. As they say, it's going to be illuminating! See you in the dim morning light, Explorer!""

"	

"/*

* 30 Days - Lost in Space

* Day 3 - I'm Worried About Your Battery Levels

*

* Learn more at https://learn.inventr.io/adventure

*

* A light that is always on is also always drawing power, and right now your

* battery power is limited.  Let's find a way to control your light so that

* we can turn it off when it's not needed, and save battery power that you'll

* be needing in the days ahead.

*

* Alex Eschenauer

* David Schmidt

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - if statement (https://www.arduino.cc/reference/en/language/structure/control-structure/if/)

* - else control structure (https://www.arduino.cc/reference/en/language/structure/control-structure/else/)

* - comparison operator == (https://www.arduino.cc/reference/en/language/structure/comparison-operators/equalto/)

* - digitalRead() (https://www.arduino.cc/reference/en/language/functions/digital-io/digitalread/)

*

* Parts and electronics concepts introduced in this lesson.

* - Dual Inline Package (DIP) switches.

* - Resistors

*/

#include ""Arduino.h""  // include information about our HERO

#define CABIN_LIGHTS_PIN 12    	// Control our lander's lights using the HERO's pin 12

#define CABIN_LIGHTS_SWITCH_PIN 2  // Connect our light switch to pin 2

// setup() gets called ONCE when our sketch is first run (after upload, when power is

// restored, or when the HERO's reset button is pressed)

void setup() {

pinMode(CABIN_LIGHTS_PIN, OUTPUT);    	// Set light control pin as an OUTPUT

pinMode(CABIN_LIGHTS_SWITCH_PIN, INPUT);  // Since we read from the switch, this pin is an INPUT

}

// After setup() is executed once the loop() function is called.  Every time it completes it

// is immediately called again, over and over again.

void loop() {

/*

// First version of our loop.  It works, but could be shorter, more efficient and

// more understandable.  Commented out but left in to show how we can incrementally

// improve our code.

byte switch_setting;

switch_setting = digitalRead(CABIN_LIGHTS_SWITCH_PIN);   // Read state of light switch

if (switch_setting == HIGH) {

digitalWrite(CABIN_LIGHTS_PIN, HIGH);  // if switch is ON then turn on our lander's light

}

if (switch_setting == LOW) {

digitalWrite(CABIN_LIGHTS_PIN, LOW);  // if switch is OFF then turn off lander's light

}

*/

/*

// Second version of our loop.  Better.  Define the variable and set it in the same statement

// and then use the else statement to show that we should always execute ONE of our two

// actions.

byte switch_setting = digitalRead(CABIN_LIGHTS_SWITCH_PIN);

if (switch_setting == HIGH) {

digitalWrite(CABIN_LIGHTS_PIN, HIGH);  // Switch is ON, turn on our lander's light

} else {

digitalWrite(CABIN_LIGHTS_PIN, LOW);  // Switch is OFF, turn off lander's light

}

*/

// Final version!  Since we only use our value once, just do our digitalRead once and

// remove the (now unnecessary) variable.  Clean, fast and less likely to allow bugs

// to be introduced.

// Each time loop() begins digitalRead() reads the input pin attached to the switch and

// compares the value read to HIGH (switch is ON)

if (digitalRead(CABIN_LIGHTS_SWITCH_PIN) == HIGH) {

digitalWrite(CABIN_LIGHTS_PIN, HIGH);  // Switch is ON, turn on our lander's light

} else {

digitalWrite(CABIN_LIGHTS_PIN, LOW);  // Switch is OFF, turn off lander's light

}

}"

Here is the script and code used in the video for Day 4:
"Astrid:

""Ah, welcome, my intrepid Explorer, to the grand finale of week one or, as some might call it, day four. I must confess, you've performed surprisingly well. For a human, that is.

Today, we're going to add a touch more complexity and a dash more brilliance to your drowning night-light. We're fixing the storage room and cockpit lights, so you won't have to grope around like a lost mole rat any longer. We're still under an ocean, you're getting cold, and the night is dark and full of scampi. We have a problem - yes, another one - The hydroponic system that filters your breathable atmosphere needs more light to photosynthesize, soak up your bad breath and turn it into Oh-Two. We need to get the rest of these DIP switch circuits operating before whatever's out there starts looking at us like a can of spaghetti.

First things first! The hardware. Notice how we've added a couple of LEDs and some extra wires for our dip switch. Don't start flipping switches just yet though. Without proper instructions in your code, they'll just stare back at you with the cold, lifeless gaze of purposeless tech.

And that brings us to our code, our all-important guide to technological obedience. We're defining six new variables that will tell us which pins are pulling their weight in the circuit. In the setup() function, we'll use the pin Mode() function to designate switches as INPUTs and LEDs as OUTPUTs. Finally, we'll march into the loop() function and create three if statements - our conditional knights in shining armor. They'll relay the status of the three dip switches.

So, my brave Explorer, are you ready to turn our ship bright enough to lure all manner of prey and predators in this unexplored ocean, like squidboats on Earth? I know - I'm excited too! Once you’ve finished wiring, upload your new code to the HERO and let there be light!""

You know the drill! Let’s cut to some instructional material I was able to gather cached on the computer. And you guessed it, I’ve also added this to your mission resources for future reference. I’ll see you soon!”

Astrid:

""Ah, you’re back! That was barely enough time to even begin missing you! Well, congratulations, dear Explorer, on surviving the first week of your unanticipated, yet thoroughly avoidable, sea adventure. You've turned this sunken wreck from a murky cave into a well-lit tin can. You're proving quite resourceful for an oxygen-breather! Bravo, or whatever it is your kind says.

Now that you have your lights, it seems that I can see into your storage bay with newfound clarity. My sensors are picking up a solar array that was previously hidden in the murkiness. I must admit, I didn't notice it earlier, but then again, it was darker than the depths of a supermassive black hole in there.

Let's hold onto that glimmer of hope. The hydroponic system is getting up to speed, and I've altered the resistances on the new lights so wiring up an LED doesn't turn this ship into the largest underwater fireball in history.

Rest up, Explorer. You've done some great work, and it looks like we’re out of immediate dang… [As she finishes speaking, Astrid glitches, the ship's lights dim briefly, and we hear a short gzzt.

CUT TO BLACK (to show battery dying)]!""

"	

"/*

* 30 Days - Lost in Space

* Day 4 - Cabin Lighting

*

* Learn more at https://learn.inventr.io/adventure

*

* In addition to our cabin lights we have lights in the storage room and even some

* exterior lights.  Let's extend our previous project by adding the others and use

* all three switches on our DIP switch module.  Each time through our loop() code we'll

* check each switch, and set the corresponding light's state appropriately.

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - const: Better than #define for defining constants that do not change

* - byte: A data type representing an unsigned 8 bit integer (values from 0 to 255)

*     	Equivalent to ""unsigned char"" or ""uint8_t"" which are sometimes seen.

*

* Parts and electronics concepts introduced in this lesson.

* - Use of breadboard ""power rails"" to allow us to use one 5V and one GND pin on

*   HERO to connect to multiple components on our breadboard.

*/

#include ""Arduino.h""  // include information about our HERO

/*

* #define vs. const

*

* Until now we have been using #define to define our constants.  When the C language

* was first created that was the only way to define constants.  However, because

* of the way #define operates there were instances where the use of #define can

* cause and hide bugs in programs.

*

* Becasuse of this, the language maintainers added a new C commands named ""const"".

* For most Arduino programs it operates much the same, though it does contain some

* additional type information that can help communicate how it is to be used.

*

* You will see both used in Arduino code and libraries.  However the general

* language calls for preferring the use of ""const"" over using #define.  Our first

* sketches have used #define so that you are familiar with them and see how they

* work, but from this lesson on we will be using the preferred ""const"" expression

* to define our constants.

*

* Here, on Day 4 we will show *both* versions, with the #define version commented

* out to prevent compile errors.  This will help you learn the difference between

* them.  After today the sketches will only use the preferred ""const"" versions.

*/

// Each lander light will be controlled using a different pin on our HERO.

// We will use three different color LEDs to make it more clear which switch

// controls which light.

/*

* The ""const"" declaration adds another word that will indicate what type of value

* is to be represented.  Since pins are small numbers they fit into a ""byte"", which

* is an unsigned integer value that can hold values from 0 to 255.

*

* Other types you may sometimes see in various Arduino code are ""uint8_t"" or

* ""unsigned char"" which are the same as ""byte"".  We will use the term ""byte""

* in our lessons to indicate 8 bit unsigned integers.

*/

// #define CABIN_LIGHTS_PIN 10         	// pin controlling the cabin lights

// #define STORAGE_LIGHTS_PIN 11       	// pin controlling the storage lights

// #define COCKPIT_LIGHTS_PIN 12       	// pin controlling the exterior lights

const byte CABIN_LIGHTS_PIN = 10;	// pin controlling the cabin lights

const byte STORAGE_LIGHTS_PIN = 11;  // pin controlling the storage lights

const byte COCKPIT_LIGHTS_PIN = 12;  // pin controlling the exterior lights

// We will use a different input pin for each of our DIP switches and

// we'll label them using the color of the LED that each switch will control.

// #define CABIN_LIGHTS_SWITCH_PIN 2         	// cabin lights controlled by switch 1

// #define STORAGE_LIGHTS_SWITCH_PIN 3       	// storage lights controlled by switch 2

// #define COCKPIT_LIGHTS_SWITCH_PIN 4       	// exterior lights controlled by switch 3

const byte CABIN_LIGHTS_SWITCH_PIN = 2;	// cabin lights controlled by switch 1

const byte STORAGE_LIGHTS_SWITCH_PIN = 3;  // storage lights controlled by switch 2

const byte COCKPIT_LIGHTS_SWITCH_PIN = 4;  // exterior lights controlled by switch 3

// the setup function runs once when you press reset or power the board

void setup() {

// Configure our LED control pins as OUTPUT pins

pinMode(CABIN_LIGHTS_PIN, OUTPUT);	// pin controlling the cabin lights

pinMode(STORAGE_LIGHTS_PIN, OUTPUT);  // pin controlling the storage lights

pinMode(COCKPIT_LIGHTS_PIN, OUTPUT);  // pin controlling the exterior lights

// Configure the switch pins as INPUT pins

pinMode(CABIN_LIGHTS_SWITCH_PIN, INPUT);	// pin connected to switch 1 (cabin lights)

pinMode(STORAGE_LIGHTS_SWITCH_PIN, INPUT);  // pin connected to switch 2 (storage lights)

pinMode(COCKPIT_LIGHTS_SWITCH_PIN, INPUT);  // pin connected to switch 3 (exterior lights)

}

// Each time through loop() we will check each switch in turn and set each light's

// state appropriately.

void loop() {

// Control cabin lights based on setting on switch 1

if (digitalRead(CABIN_LIGHTS_SWITCH_PIN) == HIGH) {  // if switch is on (HIGH voltage)

digitalWrite(CABIN_LIGHTS_PIN, HIGH);          	// turn on LED by providing HIGH voltage (5v)

} else {

digitalWrite(CABIN_LIGHTS_PIN, LOW);  // turn off LED by setting output to LOW (zero volts)

}

// Control storage lights based on setting on switch 2

if (digitalRead(STORAGE_LIGHTS_SWITCH_PIN) == HIGH) {  // if switch is on (HIGH voltage)

digitalWrite(STORAGE_LIGHTS_PIN, HIGH);          	// turn on LED by providing HIGH voltage (5v)

} else {

digitalWrite(STORAGE_LIGHTS_PIN, LOW);  // turn off LED by setting output to LOW (zero volts)

}

// Control exterior lights based on setting on switch 3

if (digitalRead(COCKPIT_LIGHTS_SWITCH_PIN) == HIGH) {  // if switch is on (HIGH voltage)

digitalWrite(COCKPIT_LIGHTS_PIN, HIGH);          	// turn the LED on (HIGH is the voltage level)

} else {

digitalWrite(COCKPIT_LIGHTS_PIN, LOW);  // turn off LED by setting output to LOW (zero volts)

}

}"

Day 5 IS A CREATIVE DAY, AND HAS NO CODE OR SCRIPT. INSTEAD, WE INSPIRE THE STUDENT TO MAKE SOMETHING NEW ON THEIR OWN.

Here is the script and code used in the video for Day 6:
"""Good morning, Explorer. Or is it evening? Oh, who knows under this ocean blanket. Now, let’s wipe that taxed look off your face and - oh, that IS your face? Oops.Get ready to tackle our next big task. We can't ignore this any longer. Our battery levels are going critical. Our Hero experiments and shipboard lighting barely draw anything, but what's killing us is just keeping things warm. I've had to shut down sections C-to-F, so... no more jelly parkour, I'm afraid.""

Our solar array, which normally bathes in glorious sunlight to charge our batteries, is as inactive as a frozen comet. It’s imperative that we restore it! Luckily, our kit is equipped with materials that have been waiting oh-so-patiently to be introduced - photoresistors!

For our training purposes, it's enough to know that photoresistors also react to light like solar panels, but in a different way. Photoresistors react to sunlight like summer flowers - When the sun rises, flowers open. When the sun goes down, flowers close. With photoresistors, when they receive light, their resistance decreases, allowing more current to pass, or higher signal levels as read. In darkness, resistance increases, choking current, lowering signal levels. We can use this to simulate our solar panels receiving sunlight.

So, brace yourself, dear Explorer. Day six (6) begins, and we have a solar system to fix.""

.

.

""Let’s get to know our newest shining star, shall we? This little device is also known as a light-dependent resistor or L-D-R for short. The resistance of a photoresistor decreases with increasing incident light intensity. In simpler terms, when there's more light, the flow of electrons becomes easier; when there’s less light, the flow becomes harder. Think of it as an electrical component throwing a 'more light, more fun' party.

Photoresistors are made of high-resistance semiconductor material. Whew! Try saying that five times fast. When light hits the device, it absorbs the light photons. This absorption process provides energy to the electrons, which makes them jump from their 'valence band' of orbits to the 'conduction band.' (What is a valence band and a conduction band?) This leap means the electrons are free to move, thus reducing the material's resistance. It's like magic, only its scientific nature is backed up by empirical evidence and testable explanations.

Essentially, a photoresistor is a sensor that allows us to detect light in the way that a solar panel uses it to generate power. They are the key component we need to simulate and fix our solar array today.

Remember, they are small and sensitive. Treat them well, and they'll return the favor in kind, Explorer!""

""Moving right along… Let's delve into a slightly more complex topic - the concept of analog and digital signals. Do bear with me, Explorer, as this will be vital for us to understand our photoresistor readings.

Alright. Analog signals! These are continuous signals. Imagine a dimmer switch connected to a light bulb. The dimmer can be adjusted to any level between on and off, providing an infinite number of positions of brightness. This is like an analog signal where the values can fall anywhere within a certain range.

In terms of microcontrollers, they are a bit more complex to handle due to their nature of being prone to degradation and noise interference. What this complexity does, however, is allow them to represent numerous amounts of information with the potential to provide more accurate and refined data.

Unlike analog signals, digital signals are discrete and can only be in one of two states, which are usually represented by either zeros (0’s) or ones (1’s) (LOW and HIGH). Picture a staircase. Each step you take is a definite value. There are no in-betweens! While they might not be as 'smooth' or 'precise' as their analog counterparts, digital signals are highly resistant to noise and easy to work with in terms of programming. Most of our microcontroller functions, like digitalWrite and digitalRead, work with digital signals.

So, where do our photoresistors fit in? I’m glad you asked! The value they provide, based on light intensity, is an analog signal - it can theoretically be an infinite number of values. That's why, to read these values, we need to use a different set of functions that can handle this variety - the analog functions. But more on that when we dive into the code.

In summary, remember: Analog is like a dimmer - continuous and smooth while digital is like stairs - definite and distinct. Got it? Great!""

.

.

""Alright, Explorer, I know you've been wondering why of all the many pins, we specifically chose the A-Zero (A0) pin on the Hero board for our photoresistor? Well, let me break it down for you.

You see, A-Zero (A0) is an analog pin. Now, our photoresistor isn't just a simple 'light or no light' type of device. No, it's much more refined than that. It measures 'how MUCH light there is.' It's like the difference between asking if there's food at a party and asking how much food is there. I mean, a few crumbs of chips don't really count, do they?

Anyway, because we want a wide range of values and not just a zero (0) or one (1), we need an analog pin, hence A-Zero (A0). It's really as simple as that - analog pins for analog devices, digital pins for digital devices.""

""Well done, Explorer, you've digested all that analog and digital chit-chat pretty well. Now, let's shift our attention to the programming part of our task. And what-do-ya know? We've got another new tool in our kit to introduce - the Arduino Serial Monitor.

The Serial Monitor is one of the most useful tools we have at our disposal when programming with the Arduino I-D-E (IDE). Think of it as a window, through which your computer can have a conversation with the microcontroller. In this case, our Hero board.

With the Serial Monitor, we can send data to the Hero board and receive data from it. It can also be used to debug (the act of finding and removing bugs in our code), examine the state of variables, or, like we'll be doing today, monitor the readings from our photoresistors.

To access it, we'll need to start by including some code in our sketch to establish a Serial communication. Then, when we want to take a peek at the data being sent from the Hero board to the computer, we simply click on the 'Serial Monitor' option in the 'Tools' menu in the Arduino I-D-E (IDE).

Ready to give it a shot, Explorer? Let's dive into the code and start using the Serial Monitor to get a sense of what our photoresistor is telling us about our power situation.""

""We're going to start using some new commands today, starting with 'analogRead()'. This function allows our Hero board to read analog input signals, which is just what we need for our photoresistors.

AnalogRead() measures the voltage between zero (0), which represents zero-V (0V), and one-thousand-twenty-three (1023) which represents five-V (5V), and returns a corresponding value. In essence, when we use 'analogRead()', we're asking the Hero board to take a voltage reading from a specific analog pin and translate it into a number we can use in our code.

'Reading' and 'writing' are two fundamental operations in HERO programming, or any programming for that matter! When we 'read', we're taking data or information in. When we 'write', we're sending data or information out.

In Arduino terms, 'reading' is often associated with sensors or inputs, like our determined photoresistors. They 'read' or gather information from the physical world which is used to make decisions in our code. On the other hand, 'writing' is generally associated with actuators or outputs, like L-E-D’s (LEDs) or motors. We 'write' to them to control their behavior based on the logic in our code.

Alright, Explorer, are you ready to put this into practice? Let's start with 'analogRead()' and see what we can learn from our photoresistors!""

It’s about that time! Let’s cut to some instructional material that I was able to gather, cached on the computer. I’ve also added this to your mission resources for future reference. Hurry on, now!

[inventrkits line-by-line internal explanation]

""Job well done, Explorer! You've taken a significant step towards survival today. The photoresistor is successfully reading light levels, meaning our battery is now charging, thus improving our power and oxygen situation exponentially. It's impressive what a motivated human can accomplish when faced with a little adversity!

Don't get too comfortable, however. Just because you've bought us some more time, doesn't mean you can take a break just yet. While it's nice to breathe without worry, remember, we're still sitting under an ocean with a multitude of broken systems on our hands. Getting our spaceship back to the surface and launching back into orbit is still a herculean task.

Still, today's success should bring some optimism. Tomorrow, we'll gather some better analytics of our charging rate by adding some additional code to our sketch. For now, power down, and rest up. You've earned it, Explorer. Until the next sunrise, that is. Bye for now!

"	


"/*

* 30 Days - Lost in Space

* Day 6 - Time to Fix the Battery

*

* Learn more at https://learn.inventr.io/adventure

*

* We will need to find an additional source of power as our battery will not last long

* without some recharging.  We do have solar panels so let's work on how the HERO can

* monitor the power coming from those panels using a ""photo-resistor"" to simulate the

* variable power available to our Lander.

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - unsigned int: A 16 bit value containing numbers from 0 to 65535

* - Serial Monitor: Allows the HERO to display text in an Arduino IDE window.

* - Serial.begin(): Used to initialize the Serial Monitor.

* - Serial.print(): Display some text in the Arduino IDE Serial Monitor window.

* - Serial.println(): Display test in the Serial Monitor followed by a newline.

* - analogRead(): Read a value from an analog pin that is based on how much voltage is on the pin (0-5v)

*

* Parts and electronics concepts introduced in this lesson.

* - Analog pins: Pins on the HERO that can read many different values instead of just HIGH/LOW.

* - Photo Resistor: Changes it's resistance depending on how much light it senses.

*/

/* We start by including this line of code, which helps our HERO work properly with the Arduino program. */

#include ""Arduino.h""

// Our photoresistor will give us a reading of the current light level on this analog pin

const byte PHOTORESISTOR_PIN = A0;  // we pick an analog pin (defined in Arduino.h)

// These two constants set the minimum and maximum delay times for oue blinking LED

// The type ""unsigned int"" represents numbers from 0 to 65535.  Another name you may

// see for this type is ""uint16_t"".  We need this since these delay values can be

// greater than the biggest number a byte can represent (255).

const unsigned int MIN_DELAY = 50;   // 50 ms shortest blink delay

const unsigned int MAX_DELAY = 500;  // 500 ms longest blink delay

// One time setup

void setup() {

// We will blink our build in LED based on amount of light received from our photoresistor

pinMode(LED_BUILTIN, OUTPUT);   	// output since we will control the built in LED

pinMode(PHOTORESISTOR_PIN, INPUT);  // input value from analog pin connected to photoresistor

/*

* To show you the exact value being read on the analog pin we will print the exact number

* using our Arduino IDE's ""Serial Monitor"".  This is a window displayed under the sketch that

* can display text sent to it from the HERO.

*

* The speed that this data is sent/received must match between the Arduino IDE and HERO.  We

* configure this speed for the HERO to send data using the Serial.begin() function.  Throughout

* this course we will use a typical value of 9600 ""baud"", which is 9,600 bit of information per

* second.

*/

Serial.begin(9600);  // This initializes the Serial Monitor and sets the speed to 9600 bits per second

}

// The loop() function is called over and over when sketch is run.

void loop() {

/*

* Each time through our loop() we will read the current value of our photoresistor.  When

* the voltage goes up from more light we flash the built-in LED faster (with shorter blinks).

*

* The photoresistor will produce a voltage that is not a simple digitial (on/off, HIGH/LOW)

* value.  Because of this we cannot use a digital pin on our HERO.  Luckily our HERO board

* provides six ""analog"" pins, named A0-A5.

*

* These pins will convert a voltage from 0V to 5V to a number from 0 to 1023, giving us

* a full range of values.  Because we are reading an analog value instead of a digital

* value, we use the analogRead() function.

*

* Here we use the reading from the PHOTORESISTOR_PIN and modify how long we delay based on it.

*/

unsigned int light_value = analogRead(PHOTORESISTOR_PIN);   // light value from 0 to 1024

/*

* Our HERO board doesn't have a text display, so the Arduino IDE has provided us a way to

* for the HERO to send messages that are displayed in a window in the Arduino IDE.  To open

* the Serial Monitor on the Arduino IDE you can select Tools/Serial Monitor from the Arduino

* IDE menu, use the shortcut keys listed in the menu or click the ""magnifying glass"" icon in

* the upper right of the IDE window.

*

* We initialized the Serial functions in setup and now we will use Serial.print() and

* Serial.println() to send messages to the Arduino IDE Serial Monitor.  Both functions

* take a single value which can be either a constant or a variable.  The only difference

* between the two functions is that Serial.print() leaves the cursor at the end of the

* current line.  Any additional output will be appended to the current line.  Serial.println()

* prints the value and THEN adds a ""newline"", so that any following messages will appear

* on the following line.

*/

Serial.print(""Light value: "");  // Display label string to serial monitor

Serial.print(light_value);  	// display the value read from our photoresistor

/*

* The flash rate varies based on the relative brightness received by the photoresistor.

* However, your brightness levels could be very different if you are in a dark room from

* running this in a bright sunny room.  Because of this we will save the darkest value

* and the brightest value seen.

*

* These values would normally be lost when each run of our loop() ends, but by adding

* the ""static"" declaration we indicate that these local variables should maintain their

* values between loop() runs.

*

* The FIRST time these variables are used they will get initialized with the first light_value

* but that initialization is only done the first time so they will retain any changes

* between the loop() runs.

*/

static unsigned int darkest_light = light_value;	// this is the lowest value returned by the photoresistor

static unsigned int brightest_light = light_value;  // this is the highest value returned by the photoresistor

/*

* Now that we have a light value let's update our darkest and brightest values

* if the current value is darker (less than) our previous darkest or brighter

* (greater than) our previous brightest value.

*/

if (light_value < darkest_light) {  // If value is LESS THAN ('<') the darkest...

darkest_light = light_value;  	// ...then save current value as our new darkest value

}

if (light_value > brightest_light) {  // If value is GREATER THAN ('>') the brightest...

brightest_light = light_value;  	// ...then save current value as our new brightest value

}

/*

* Now we have an interesting problem.  As our voltage reading goes up, we want the LED to

* blink faster (and thus with a smaller delay).  In addition, we need to adjust the blink

* rate in a noticeable way.  So, we have one range of numbers (darkest to brightest values)

* from our light sensor that we wish to convert to a different range that goes from MAX_DELAY

* down to MIN_DELAY.

*

* The Arduino language provides a neat function named map() that was created just for this

* situation.  It takes a value from one range and *maps* it into a desired range.  It properly

* handles the situation where the second range goes DOWN when the first range goes UP.

*

* We introduce it's used here to map our light_value (which goes from darkest_light to

* brightest_light) and return a value from MAX_DELAY *down* to MIN_DELAY.  Perfect!

*/

unsigned int delay_value = map(light_value, darkest_light, brightest_light, MAX_DELAY, MIN_DELAY);

Serial.print("", Delay value: "");  // display label after light_value

Serial.println(delay_value);  	// display delay_value returned by map() function with newline

// now blink our built in LED using our delay_value.

digitalWrite(LED_BUILTIN, HIGH);  // Turn on LED

delay(delay_value);           	// Leave on delay_value milliseconds

digitalWrite(LED_BUILTIN, LOW);   // Turn off LED

delay(delay_value);           	// Leave off delay_value milliseconds

}"

Here is the script and code used in the video for Day 7:

"""Rise and shine, explorer! Our oxygen levels are steady, and our battery is replenishing nicely. That's cause for a small sigh of relief, wouldn't you say? But let's not lose sight of the task at hand; we have another glorious day of repairs to conduct.

Today, we're going to shift gears and delve into a bit of digital logic. We'll be learning about bits, data types, and why choosing the right one for your variables can make all the difference. Efficient memory usage is essential for our system to function optimally, especially on low-memory devices like our Hero board.

Moreover, we're going to expand the depth of our coding efforts. We'll work on a piece of code whose intention is to calculate our ship's battery percentage. Not only will this give us a clearer idea of our power situation, but it will also be a practical application of the concepts we're learning. Ready to get your hands dirty with some binary? Of course you are! Let's get to it.""

""Well, well, aren't you lucky, Explorer? Today you get to enjoy a bit of a hardware holiday. How delightful for you! We're leaving the wires and boards be – they've had quite enough of your 'expertise' for the time being.

Before you start daydreaming about a leisurely day off, let me assure you, we're not on vacation. There is still much work to be done! While the hardware is blessedly out of harm's way for a day, we're going to fill the void with a thrilling deep-dive into the captivating world of coding. Yes, that's right – more mesmerizing mazes of bits, bytes, variables, and functions are waiting for your full attention.

This isn't just about expanding your tiny knowledge base for the sake of it, you know. We're doing this so we can craft more refined, more efficient code. In layman's terms: smoother operations and quicker fixes lead us to a faster escape from this underwater blunder that we’re in.

So, buckle up, Explorer. It's high time you familiarize yourself with the finer points of our Hero board's language. We've got some serious coding to do.""

.

.

.

""Oh, joy. It seems you're thirsty for knowledge, Explorer. Well, hold on to your seat; I'm about to drop some wisdom bombs on you.

First, there's this delightful little thing called a 'bit.' The smallest unit of data there is! It’s binary, which in this case means it’s either zero (0) or one (1). Its simplicity is quite adorable, isn’t it? Combine three of these and boom! You have eight unique integer numbers. Mathemagic!

Next, let's graduate to the 'integer.' It's a little bit chunkier, with a typical size of sixteen (16) bits. This gives you a marvelous range of about plus or minus thirty-two thousand, seven-hundred-sixty-eight (32,768). Not too shabby, wouldn’t you agree?

When it comes to integers, we’re not bound by one path to stroll down. We've got two! 'Signed' and 'unsigned.' The signed integers take more of a straightforward path, and have a bit that acts like a flag for whether the number is positive or negative. Unsigned integers, on the other hand, don't bother with this. As free spirits, they take the scenic route, living their life positively all the way! Hence the bigger range of numbers.

Now, take our photoresistor. It's a modest creature, outputting a value between zero and one-thousand-twenty-three (0-1023). It's just ten (10) bits worth of data because two to the power of ten (2^10) equals one-thousand-twenty-four (1024). So, an integer is more than adequate to store it. But don't get too comfortable. Our batteryCapacity variable is set at a lofty fifty-thousand (50,000), which outclasses the standard short integer maximum of thirty-two-thousand, seven-hundred-sixty-eight (32,768). Thus, it needs the V-I-P (VIP) treatment which defines it as an unsigned integer.

Next up, meet 'float.' This one likes to live a bit on the edge, handling decimal numbers with precision. Imagine one-hundred-twenty-three point four-five-six-seven (123.4567) – it has a precision of seven (7). Not bad, eh?

Then there's 'double.' It's a float that hit the gym and got buff. It's sixty-four (64) bits of power, capable of holding values up to a mind-boggling ten to the power of three-hundred-eight (10^308).

Veering off from variables, let's shine some light on 'void.' It's what we use when we're defining functions that don't give anything back. It’s a bit of an altruist. But don’t worry! We’ll dive deeper into creating our own function soon enough.”

So there you have it. A crash course in basic digital logic. Its overwhelming nature is similar to drinking from a firehose, I know. But fret not, we'll be practicing these concepts soon enough. Shall we push on then?""

.

Today’s mission involves writing additional logic in order to print the battery percentage to our handy serial monitor that we discussed yesterday.

Of course, we don’t want to spend time rewriting the print statements all over the place in our code. Instead, we can create a new function.

We can start with the word “void”. This is what the function will be returning. As you may have guessed, “void” means that it will return nothing. This also means that we don’t need a return statement, since the compiler deems it unnecessary. I recommend you name it something relative, such as “PrintBatteryPercentage().” The parentheses can be empty since we have no parameters we need to pass into the function.

Finally, everything inside the brackets is what will execute when you call the function. When the second closing bracket is reached, it will return to where it was called from in the code. If my assumption is correct, it is likely ‘setup’ or ‘function’.

In the end, your newly created function might look something like this:

For the line-by-line, let's jump to the information cached on your computer. I’ve also added this to your mission resources for future reference. I’ll be waiting patiently for your return.

[segway to inventrkits line by line]

""Well, well, well, you've made it through the day, Explorer. Our little adventure with bits, integers, floats, and doubles didn't scare you off, did it? Not bad for a human, I must say.

Now that we have our power situation under control and can monitor our battery percentage, you must be feeling rather accomplished! Before you get lost in celebration, let me remind you that we've barely scratched the surface of what's broken on this ship.

Tomorrow, we're going to brighten up this dreary spacecraft by adding a bit of color. Yes, you heard me right. We're going to introduce an R-G-B L-E-D (RGB LED), which can emit all the colors of the rainbow. We'll use this flashy little gizmo for various status indicators - like painting a glowing green if our battery is all charged up, or a glaring red if we're running on fumes.

So, get some rest, Explorer. Tomorrow, we paint with light.""

"	

"/*

* 30 Days - Lost in Space

* Day 7 - We still need to keep an eye on this

*

* Learn more at https://learn.inventr.io/adventure

*

* Now we will use the circuit from Day 6 and use the value read from the photoresistor

* to simulate charging our lander's battery.  Our battery is currently drained

* (current level is 0) and we will stop charging once the battery is full.

*

* Alex Eschenauer

* David Schmidt

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - ""+="" operator.

* - defining our own functions.

*

* Parts and electronics concepts introduced in this lesson.

* - No new parts as this sketch uses the same circuit as Day 6

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Our photoresistor will give us a reading of the current light level on this analog pin

const byte PHOTORESISTOR_PIN = A0;  // Photoresistor analog pin

// Use an unsigned, 16 bit value (0 - 65535) for maximum battery capacity

const unsigned int BATTERY_CAPACITY = 50000;  // Maximum battery capacity

void setup() {

pinMode(PHOTORESISTOR_PIN, INPUT);  // Input current light level from photoresistor

Serial.begin(9600);  // Initialize Serial Monitor to 9600 baud

}

// Use global here because this variable is accessed by both loop() and our custom

// function.  (In later lessons we will show how to avoid globals.)

unsigned int battery_level = 0;  // Current battery charge level (set to 0 first time)

/*

* Each time through our loop() we read the current light level from the photoresistor

* and add that to our current battery level to simulate charging our battery.

*

* We then display current charge level on the Serial Monitor.

*/

void loop() {

// Display battery information on the Serial Monitor

if (battery_level < BATTERY_CAPACITY) {  // If battery is not fully charged..

/*

* Here we introduce a new arithmetic operator, ""+="".

* When you wish to add a value to an existing variable you *could* write it like

* ""value = value + additional;"".  But the Arduino C++ language allows you to avoid

* the duplication.  The += operator allows us to add to an existing variable value.

*

* In the following statement we will add our current light level to our existing

* battery level.

*/

battery_level += analogRead(PHOTORESISTOR_PIN);  // read light level and add to battery level

if (battery_level > BATTERY_CAPACITY) {      	// If this would make charge over 100%...

battery_level = BATTERY_CAPACITY;          	// ...set current level to maximum capacity.

}

}

/*

* Previously we have only used functions included in the Arduino C++ Language.  Here

* we demonstrate how we can use our own functions.  This keeps our main code cleaner

* and easier to understand.  All you need to know here is that this new function prints

* the current charge level on the Serial Monitor.

*

* If you need to know the details you can examine the full function declaration following

* our loop() code.

*/

printBatteryChargeLevel();  // Display current charge percentage on Serial Monitor

delay(100);

}

/*

* This is where our own function is defined to display the current battery charge

* level on the Serial Monitor.

*

* This function takes no parameters and returns no values (indicated by the ""void"" preceeding the name).

*

* If the battery is not fully charged yet we then print the current battery percentage

* to the Serial Monitor.

*

* If the battery is fully charged we display that to the serial monitor.

*/

void printBatteryChargeLevel() {

if (battery_level < BATTERY_CAPACITY) {  // if not fully charged

// Percentage of charge is current level divided by capacity, multiplied by 100 to get a percentage.

Serial.print(((double)battery_level / (double)BATTERY_CAPACITY) * 100);  // display charge % to Serial Monitor

Serial.println(""%"");

} else {

Serial.println(""FULLY CHARGED"");  // ...indicate fully charged on Serial Monitor

}

}"

Here is the script and code used in the video for Day 8:
"""Rise and shine, Explorer! Day eight (8) has dawned upon us and has left us with no time to waste! Today, we're stepping up our game. We're going to add a dash of color to your monotonous existence by using a nifty piece of hardware called an R-G-B L-E-D (RGB LED). Allow me to explain.

RGB LED stands for Red-Green-Blue Light Emitting Diode. What this L-E-D (LED) does is allow us to produce virtually any color on the spectrum by controlling the intensity of the three colored diodes. It’s like your own personal rainbow producer!

We're also going to cover a new function today: analogWrite(). It allows us to send an analog output signal on a digital pin. Yes, I know what you're thinking: didn’t we just discuss how digital and analog are separate entities? We sure did! Let me, however, introduce you to the world of 'Pulse Width Modulation' or P-W-M (PWM). This savvy, little trick allows us to fake an analog output on a digital pin! Pretty neat, wouldn’t you agree?

Nonetheless, that’s enough jabber for now! Rest assured, all this and more will be covered as the day progresses. Brace yourself in, Explorer. We've got colors to mix and a spaceship to fix!""

.

.

""An R-G-B L-E-D (RGB LED) is unlike your usual kind of LED, Explorer. It is like a three-in-one special! Inside that tiny package, you'll find three separate L-E-Ds (LEDs): one Red, one Green, and one Blue, as these are the primary colors of light.

In a regular L-E-D (LED), you apply power and it emits a pre-programmed, specific color. In an R-G-B L-E-D (RGB LED), however, you have the ability to control each of the three colors individually. You can turn on the red L-E-D (LED), the green L-E-D (LED), the blue L-E-D (LED) - or any combination of the three!

So, why is this exciting? Because by varying the intensity of each of these colors, you have the power to create virtually any color you want right at your fingertips! How deeply thrilling!

Need a soothing aquamarine glow to relax? Easy. Want to flash a warning in bright, alarming red? Ask and you shall receive!

In essence, with an R-G-B L-E-D (RGB LED), you're not just getting one color. You're getting the whole spectrum. Enjoy the possibilities, Explorer!""

.

.

""Okay, Explorer, let's dig into two intertwined concepts: 'analogWrite()' and 'Pulse Width Modulation' or P-W-M (PWM), which are critical for controlling our  R-G-B L-E-D (RGB LED).

'AnalogWrite()' is a function in Arduino that helps us generate P-W-M (PWM) signals. Despite its name, oddly enough, 'analogWrite()' doesn't really deal with analog signals. It is a bit of a misnomer, which could be a source of confusion. Have no fear, Explorer! It’ll all piece together soon enough.

Moving onto P-W-M (PWM)! ‘Pulse Width Modulation’ is a technique used to create a kind of 'fake' analog effect using digital means. By rapidly switching a signal or power source on and off, and changing the proportion of the time it is on versus the time it is off, we can vary the intensity of power that is received. This is important for our R-G-B L-E-D (RGB LED) because it allows us to control the intensity of each of the red, green, and blue LEDs, which in turn lets us mix our colors.

Imagine P-W-M (PWM) as a fast-flickering light. If it flickers so fast that your eyes can't catch it turning on and off, the light will appear dimmer or brighter based on the ratio of 'on' time versus 'off' time. If it's on more than it's off, it appears brighter; if it's off more than it's on, it appears dimmer.

In terms of our R-G-B L-E-D (RGB LED), we'll use 'analogWrite()' to send P-W-M (PWM) signals to each of the red, green, and blue L-E-Ds (LEDs). By changing the duty cycle of the P-W-M (PWM) signal — that's the 'on' time — we can control the brightness of each L-E-D (LED). By mixing different levels of brightness across the red, green, and blue L-E-Ds (LEDs), we'll create the color we want.

You’re in for quite a treat, Explorer! The trajectory of your future will soon become a lot brighter…quite literally.

.

.

""It appears we’ve ‘talked the talk,’ now let’s ‘walk the walk’ Explorer. Our task today is to light up our R-G-B L-E-D (RGB LED) and give it a whirl! Remember, with Pulse Width Modulation, we can tinker with the intensity of each color, from a meager zero (0) to a dazzling two-hundred-fifty-five (255). Talk about a spectrum!

Now, one way to control our R-G-B L-E-D (RGB LED) would be to separately define the red, green, and blue values every single time we want a color change. But, frankly, that's like choosing to walk across the galaxy when you have a perfectly good spaceship. It's slow, and it'll leave your code looking like a debris field of a supernova.

Let's be smarter, shall we? What do you say we build a function to streamline the process? Something catchy and memorable like ‘set R-G-B red level, green level, blue level’ (SetRGB(red_level, green_level, blue_level)). You'll see those three “byte” variables nestled inside the parentheses, separated by commas. These are parameters, and we'll be handing them over to our function like instructions to a well-trained robot butler. We don't have a lot of memory, so since the max value doesn’t need to be anything higher than 255 anyway (remember pulse width modulation), a byte is perfect since it’s max value just so happens to be 255. Using something like an integer for an unsigned number <=255 is just a waste of memory.

Once we're inside the function, it's time to put those parameters to work. We'll use 'analogWrite()' to set the value for each of our three color pins. Efficient and tidy, just the way I like it.

In our main loop, it's testing time. We'll call our snazzy new function in a variety of ways to make sure our R-G-B L-E-D (RGB LED) isn't just sitting there like a lump of plastic. Once we're confident everything's shipshape, we'll beam it up to our board and enjoy the show.

Let's get to it, Explorer. Lights, camera, code!""

Now for what you’ve been waiting oh-so patiently for. Let’s cut to some instructional material that I was able to gather, cached on the computer. I’ve also added this to your mission resources for future reference. See you soon!

[inventrkits line-by-line - recorded internally]

""Well, Explorer, I must say I'm impressed. You've wired up an R-G-B L-E-D (RGB LED) and successfully programmed it to show different colors based on given values.

Your code is no longer a maze of repeated color values. It's streamlined and efficient like the inner workings of a cutting-edge spaceship. And our Hero Board is sparkling with colors. Just the kind of bright glimmer of hope you need in the depths of an underwater world.

Now, don't get too cozy in your success. Tomorrow, we'll take another momentous leap forward. I see a dashboard display in our future! It's time to start getting some visual feedback from our various ship systems on a display. Yes, indeed. Just because we're marooned in an alien sea doesn't mean we can't have a bit of style, right?

So rest up, Explorer. We have many more lines of code to write and much more hardware to wrangle before our ship can once again break free from this watery prison. But for now, bask in the glory of your multicolored triumph. Tomorrow awaits!""

"	

"/*

* 30 Days - Lost in Space

* Day 8 - Adding some color to this dark place

*

* Learn more at https://learn.inventr.io/adventure

*

* Previously we have used different color LEDs with our HERO, but when we require

* more output, or desire a color different from the available colors we use a more

* flexible ""RGB LED"".  RGB LEDs have a red, a green and a blue LED packaged into a

* single LED package.  By using something called Pulse Width Modulation (PWM) we

* can vary the intensity of each LED and come up a wide variety of colors.

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - analogWrite(): Used to control a PWM pin, giving a variable intensity

* - Passing variables into functions

*

* Parts and electronics concepts introduced in this lesson.

* - Common Cathode (single grounded pin) RGB LED.

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

/*

* Each color in an RGB LED is controlled with a different pin on our HERO board.

*

* NOTE: Only pins 3, 5, 6, 9, 10 and 11 on the HERO board support PWM which is

*   	indicated on the board by preceding those pin numbers on the HERO board

*   	with a tilde ('~') character.  Only those pins support PWM and analogWrite().

*/

const byte RED_PIN = 11;	// PWM pin controlling the red leg of our RGB LED

const byte GREEN_PIN = 10;  // PWM pin ccontrolling the green leg of our RGB LED

const byte BLUE_PIN = 9;	// PWM pin ccontrolling the blue leg of our RGB LED

/*

* Define a set of constants to represent 4 different brightness levels for our

* RGB LED using PWM pins and analogWrite().  PWM can set values ranging from 0

* to 255 which we will demonstrate by adding 64 (roughly 1/4 of that range) to

* each preceding value.

*

* NOTE: this also demonstrates that our constants can be computed using an expression

*   	that includes any previouly defined constant.  However, the expression cannot

*   	include variables or values that could change during execution.

*/

const byte OFF = 0;             	// Selected color is OFF

const byte DIM = 64;            	// Selected color is 1/4 intensity

const byte BRIGHTER = DIM + 64; 	// Selected color is 1/2 intensity

const byte BRIGHT = BRIGHTER + 64;  // Selected color is 3/4 intensity

const byte BRIGHTEST = 255;     	// Selected color is maximum intensity (255)

// We can change this constant here, in one place, to change how long each color is displayed.

const unsigned int COLOR_DELAY = 500;  // show each color for 500 milliseconds

void setup() {

// Set each of our PWM pins as OUTPUT pins

pinMode(RED_PIN, OUTPUT);

pinMode(GREEN_PIN, OUTPUT);

pinMode(BLUE_PIN, OUTPUT);

}

// Each time through loop() we display some of the color variations available

// using an RGB LED.

void loop() {

// First demonstrate our different PWM levels by slowly brightening our red LED

displayColor(OFF, OFF, OFF);  // OFF!

delay(COLOR_DELAY);

displayColor(DIM, OFF, OFF);  // Display red LED at 1/4 intensity

delay(COLOR_DELAY);

displayColor(BRIGHTER, OFF, OFF);  // Display red LED at 1/2 intensity

delay(COLOR_DELAY);

displayColor(BRIGHT, OFF, OFF);  // Display red LED at 3/4 intensity

delay(COLOR_DELAY);

displayColor(BRIGHTEST, OFF, OFF);  // Display red LED at FULL intensity

delay(COLOR_DELAY);

// Display our other two LED colors at half intensity

displayColor(OFF, BRIGHT, OFF);  // Display the green LED

delay(COLOR_DELAY);

displayColor(OFF, OFF, BRIGHT);  // Display the blue LED

delay(COLOR_DELAY);

// Now show various colors (at half intensity) by mixing our three colors

displayColor(BRIGHT, BRIGHT, OFF);  // Display yellow by mixing red and green LEDs

delay(COLOR_DELAY);

displayColor(OFF, BRIGHT, BRIGHT);  // Display cyan by mixing green and blue LEDs

delay(COLOR_DELAY);

displayColor(BRIGHT, OFF, BRIGHT);  // Display magenta by mixing red and blue LEDs

delay(COLOR_DELAY);

// Display all of our LEDs to get white.

displayColor(BRIGHT, BRIGHT, BRIGHT);  // white

delay(COLOR_DELAY);

}

/*

* displayColor() is a function that accepts three parameters representing the desired

* intensity for each of the LEDs in the RGB LED.

*

* Each parameter passed must have a type (here we match what analogWrite() will use) and

* a name to be used inside the function to refer to the parameter.  The parameters can all

* be included on a single line like:

* void displayColor( byte red_intensity, byte green_intensity, byte blue_intensity) {

*

* However, if we display each parameter on it's own line we can add a comment to each

* for additional clarity.

*/

void displayColor(

byte red_intensity,	// red LED intensity (0-255)

byte green_intensity,  // green LED intensity (0-255)

byte blue_intensity	// blue LED intensity (0-255)

) {

analogWrite(RED_PIN, red_intensity);  	// Set red LED intensity using PWM

analogWrite(GREEN_PIN, green_intensity);  // Set green LED intensity using PWM

analogWrite(BLUE_PIN, blue_intensity);	// Set blue LED intensity using PWM

}"

Here is the script and code used in the video for Day 9:	
"""Greetings, Explorer! Day 9 is upon us, and it's time you stopped ignoring me!

... Or at least, stopped ignoring our battery levels. Getting a percentage is fine, but ultimately, reading that percentage is on you, and you're not always looking. You will have to look after yourself in the future - I'm not going to be around forever, Explorer... I have a spin class at 5...

Let's set up a RGB LED 'rag' status light. Wow! I think that's the most TLAs I've said in a sentence!

... Uh, you know what LED stands for. 'RGB' stands for Red Green Blue, and when you combine these three colors in different intensities, you get all the colors of the rainbow. 'Rag' is another TLA, or three-letter-acronym, and that stands for Red, Amber, Green, otherwise known as a traffic light system.

So... Our mission is to use one LED, which makes any color, to tell you if the battery levels are good-and-green, anxious-amber, or rock-bottom red. RGB LED for RAG ASAP!

Now there are a LOT of decisions to be made on this mission, Explorer. Fortunately, we don't have to make any of them. We'll bring THESE new ways to compare things [graphic of comparison ops] and we'll use a new command, 'else if' to catch any crazy stuff in the code. 'else if' is another conditional operator. .

.

""Right then! Since we're not adding any new hardware today, we'll dive straight into the code pool. Our mission? To create a color-coded battery status indicator using our trusty R-G-B L-E-D (RGB LED).

Imagine this, Explorer: A bold green shine to signify a robust battery level. A shifting amber for when power starts to wane. And an urgent red pulsating from our little R-G-B L-E-D (RGB LED) as a warning when the battery drops too low. My mind's eye could wander forever! Let’s make it happen.

.

.

""Remember when we used the photoresistor to regulate our charging speed?""

Now, we're taking it up a notch. Every loop cycle, we'll work out the battery percentage. Then, we'll test that percentage: if it's within a certain zone, we'll make the light a certain color.

The second step is to use conditional logic to give our ship a color-coded mood ring. Let's see...

Green: brimming with power. Fifty (50) to One-hundred-percent (100%) and raring to go.

Amber: Keep an eye on it, as we're between twenty-five-percent (25%) to fifty-percent (50%).

Red: Danger zone! We're below twenty-five-percent (25%) and need to seriously consider charging up.

You're smart, Explorer. Our RGB LED only has Red, Green and Blue colors - Where's the Yellow??? Well, we can get yellow by mixing a little green with a little red [graphic]""

A true artisan of code you are, Explorer. Now, let's get this show on the road!""

.

.

""In order to make our color-coded power level dreams a reality, it looks like we're going to need a few more tools in our coding toolkit. It's time to get to know some of the more... sophisticated comparison operators.

Allow me to introduce you to our new friends: greater than or equal to, greater than, less than or equal to, and less than (>=, >, <=, and <). Oh, and let's not forget about their pals, the logical operators: AND (&&) and OR (||).

These operators allow us to make more complex decisions in our code.

To start, the greater than (>) and less than (<) signs are likely familiar to you. These symbols are used to compare two values. For example, five is greater than three (5 > 3) returns true because five (5) is indeed greater than three (3).

The less than or equal to (<=) and greater than or equal to (>=) operators extend this comparison. How about this - Is 5 greater than... 5?... We might need a condition where a value is both greater than AND equal to another, and that's why THESE chicken footprints (graphic, <=, >=) are very useful.

I knew you’d catch on fast. Now, onto the logical operators!

'AND' (&&) is a binary operator that requires two conditions to be true for the overall expression to be true. Stay with me now, Explorer. Let’s break it down. Imagine this: you're planning to leave your cozy spacecraft to explore the boundless wonder that is space. Extravehicular Activity, or E-V-A (EVA) as we like to call it. You would only go if both your suit was functional AND the exterior conditions were safe. Clearly, only when both of these conditions are met would you step out of the spacecraft.

'OR' (||) is slightly different. In this case, if either condition is true, then the overall expression is true. Picture this: you eat if you are hungry OR you eat if it is mealtime. If either condition is true, it's chow time!

""Greater than or equal to, AND, OR... Decisions, decisions, decisions. It can be a lot to think about. That's why we code, Explorer: so machines can made the tedious decisions for us.""

At this time, you may indulge in the highly beneficial instructional material that I have gathered, cached on the computer. As you know, I’ve also added this to your mission resources for future reference. Good luck!

[inventrkits line-by-line - recorded internally]

""Well, well, well, Explorer! Looks like you've managed to navigate your way through another week of this chaotic adventure. You've got your battery charging up and a shiny, new light indicator to show you just how well you're doing. I must say, not bad for a rookie!

Remember when you were struggling with basic switches? Seems like ages ago now! You humans grow up so fast. You've come quite a long way, fixing power and oxygen, lighting up your life, and now you've even got a handy color-coded battery life indicator. Great job!

""But we need to start ramping your progress up. This is a spaceship, not a submarine. Our mothership's orbit is beginning to degrade. If we don't get our heads above water soon, we'll need to start thinking long term... and batteries don't last forever...""

(Another whale-like reverberant ship-shaking noise)

""I've started naming some of the entities that revisit the ship... That's definitely Rachel.... She sounds hungry…

"	

"/*

* 30 Days - Lost in Space

* Day 9 - A Better Way to Do Things

*

* Learn more at https://learn.inventr.io/adventure

*

* Now that we know how to measure our battery charging rate and we know how to display

* multiple colors with the RGB LED, let's combine those two concepts to display different

* colors as our battery charges to give a good indication of our current battery state.

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - float:   Variable type for decimal numbers that include a decimal point

* - else if: Control structure for making multiple if decisions together

*

* Parts and electronics concepts introduced in this lesson.

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Our photoresistor will give us a reading of the current light level on this analog pin

const byte PHOTORESISTOR_PIN = A0;  // Photoresistor analog pin

// RGB LED pins

const byte RED_PIN = 11;	// pin controlling the red leg of our RGB LED

const byte GREEN_PIN = 10;  // pin ccontrolling the green leg of our RGB LED

const byte BLUE_PIN = 9;	// pin ccontrolling the blue leg of our RGB LED

const unsigned long BATTERY_CAPACITY = 50000;  // Maximum battery capacity

/*

* Display a color on our RGB LED by providing an intensity for

* our red, green and blue LEDs.

*/

void displayColor(

byte red_intensity,	// red LED intensity (0-255)

byte green_intensity,  // green LED intensity (0-255)

byte blue_intensity	// blue LED intensity (0-255)

) {

analogWrite(RED_PIN, red_intensity);  	// write red LED intensity using PWM

analogWrite(GREEN_PIN, green_intensity);  // write green LED intensity using PWM

analogWrite(BLUE_PIN, blue_intensity);	// write blue LED intensity using PWM

}

void setup() {

// Declare the RGB LED pins as outputs:

pinMode(RED_PIN, OUTPUT);

pinMode(GREEN_PIN, OUTPUT);

pinMode(BLUE_PIN, OUTPUT);

// Start serial monitor

Serial.begin(9600);

}

void loop() {

// Use static because we need this variable to maintain it's value across

// multiple loop() runs.

static unsigned long battery_level = 0;  // Current battery charge level (set to 0 first time used)

battery_level += analogRead(PHOTORESISTOR_PIN);  // Add current ""charge amount"" to our battery

// We can't charge the battery higher than it's capacity, set level as full if it goes over

if (battery_level > BATTERY_CAPACITY) {

battery_level = BATTERY_CAPACITY;

}

/*

* IMPORTANT NOTE:

* If you perform an operation using integer arithemetic each value is not

* automatically converted to a floating point (decimal) number.

*

* In the calculation below, if we divide the integer battery value by the

* integer battery capacity (battery_level / BATTERY_CAPACITY) then the result

* would ALWAYS be 0, even if that 0 gets converted afterwards to a float (0.0).

*

* To avoid this, we use the ""(float)"" declaration before each of our values to

* instruct the compiler to FIRST convert the value to floating point and then

* perform the calculation using floating point math.

*

* If our current_battery_level is less than the battery capacity the initial

* calculation gives us a number from 0.0 to 1.0.  We then multiple by 100 to

* get a percentage value from 0.0 up to 100.0.

*/

// Compute battery charge percentage from our function

float percentage = ((float)battery_level / (float)BATTERY_CAPACITY) * 100;

if (percentage >= 50.0) { 	// battery level is OK, display green

displayColor(0, 128, 0);  // display green

} else if (percentage >= 25.0 && percentage < 50.0) {

displayColor(128, 80, 0);  // display yellow-ish/amber for early warning

} else {                 	// Level must be less than 25%, display ""pulsating"" red

// To pulsate the red light we briefly turn the LED off and then display red, giving it

// a pulsating effect.

displayColor(0, 0, 0);	// Turn off our LED

delay(20);            	// ...and delay briefly

displayColor(128, 0, 0);  // then display red

}

Serial.print(percentage);  // Display our floating point percentage (like 12.34) WITHOUT a newline

Serial.println(""%"");   	// then display the percent sign (""%"") with a newline.

delay(100);  // Delay 1/10 of a second so displayed values don't scroll too fast

}"

Day 10  IS A CREATIVE DAY SO THERE IS NO SCRIPT OR CODE, INSTEAD WE INSPIRE STUDENTS TO CREATE SOMETHING NEW ON THEIR OWN

Here is the script and code used in the video for Day 11:	
"""Ah, you’re up! Hope you're ready for the dawn of a new day in this damp tin can that we're calling home. Week three (3) already, huh? Seems like just yesterday you were fumbling over the basics. Look at you now: power, light, a snazzy R-G-B (RGB) battery indicator. All pretty impressive I must say!

This week, we're shifting gears! Enough with the basic survival needs, it's time we brought this old ship back to life. Our goal is to fix up that control panel whose appearance looks as if a Martian tried to play tic-tac-toe on it.

To start us off, we've got a new tool to familiarize ourselves with - a four-by-four (4x4) keypad. I know, you must be thrilled. Do try not to let it’s looks intimidate you. That’s sixteen (16) whole buttons of potential inputs at our fingertips!

Alongside our new hardware, we've got a new concept to grasp. Brace yourself, because we're diving into the wonderful world of libraries today. Think of libraries as your ultimate cheat sheet: 'shortcuts that make coding easier.'

So, don your programmer's hat, take a sip of that dehydrated coffee, and let's tackle the day ahead. We've got work to do, Explorer!""

.

.

""Alright, let's chat about our new piece of hardware, the four-by-four (4x4) keypad. [text removed here] Each button, when pressed, closes a circuit and sends a signal that we can detect and utilize. It's similar to a keyboard you might use on a computer, but far less complicated.

These keypads are often used in security systems, door entry systems, and all sorts of devices where manual input is necessary. Today, we'll be using it as a control interface for our spaceship.

The real magic happens when each button, or key, is pressed. A unique combination of two connections is made. One connection from the four ‘row’ pins, and one from the four ‘column’ pins. This allows us to know exactly which button was pressed, without needing a separate input pin for each button. [on-screen graphic showing overlap of row and column on 4x4 grid]

Each of these sixteen (16) buttons can serve as an individual input, providing us with a wealth of potential interaction options. I can see it now - we could set a code to trigger a certain action, or assign different tasks to different keys. The possibilities are vast and are waiting to be explored! This also means we can reduce the amount of input pins needed from sixteen (16) to eight (8), which is important when we have limited pin real-estate on our HERO.

But, before we dive into all of that, let's get this mighty piece of machinery wired up and running, shall we?""

.

.

“My! That was quite easy. Looks like we can skip the breadboard entirely today”.

.

.

“Speaking of easy, let’s hop on to our next coding concept and explore the power of your new [small removal] partner in crime - libraries.

In coding terms, a library is like a pre-packaged set of code that provides specific functionality. Essentially, They give you a lot of new functions to use and make your life easier.

Now, let’s talk about the new library you will need to install: the Keypad-H (Keypad.h) library. This is a custom library designed specifically for interfacing with keypads of various sizes, like a four-by-four (4x4) keypad, for example. How convenient! It includes functions for detecting when keys are pressed, which keys are pressed, and even some advanced functionality like ‘multi-key detection’.

By using this Keypad library, we can simplify our code and kick our level of efficiency up a notch. Because, who needs more complications when you're trying to repair a spaceship, right?""

.

.

In order to use our library, we will first need to install it. If my calculations are correct, there should be a link located somewhere, cached on your current computer page to a ‘.zip’ file. Download that to your computer, and then in the Arduino IDE, navigate to Tools, then (>) Manage Libraries, then (>) Add .zip library and select the newly downloaded .zip file. Take all the time you need, preferably as quick as you can! I’ll be here waiting patiently for you when you return. Finished? Wonderful! Let’s continue, Explorer.

The library allows us to accomplish another new practical task and that is to create a new, unique object - a Keypad. It also adds in functions like getKey() to tell us which key is being pressed. In order for the newly installed library to help us, however, we need to first provide it with some relevant information.

Here is our list of “to-do’s.” We need to tell it how many rows we have, how many columns we have, the pins that we are using for our column pins, the pins that we are using for our row pins (said column before but I’m assuming you meant rows?), and finally we need to create a keymap for the buttons. Whew! That was quite the mouthful.

Well, we know we have four (4) rows, four (4) columns, and we can take a look at which pins we are using by simply looking at our wiring diagram.

Next, we have to make a keymap. We can achieve this by utilizing a pair of new data structures in programming - arrays.

In the realm of programming, an ‘array’ is a data structure that allows you to store multiple values in a single variable. Imagine it like a storage box with multiple compartments. You can put different items, or in our case, data values, in each compartment and later access them using an index.

An array is defined by the type of data it can hold, like integers, floats, or characters. Once an array is declared, the type of data it can hold is fixed. You aren’t able to mix integers and strings, for example.

Each individual value in the array is assigned an index. In most languages, the first element in the array is indexed as zero (0). So, if you have an array of five elements, they would be indexed from zero (0) to four (4).

.

.

""Let’s discuss this further. Pay close attention, because this is a stumbling block for many new programmers. Even the best of them! Remember this like a mantra - arrays start at zero (0), not one (1). Zero, nada, nothing. Got it? Great.

Take a look at this innocent little array, groceryList[]. If I ask you, what is at ‘grocery list one’ (groceryList[1]), what would you say? Here's the array for you to ponder:

No, no, it's not 'Milk.' Remember the mantra. 'Milk' is lounging at index zero (0). So at index one (1), we have 'Egg.' Good.

So, if our grocery list array has three items, what do we find at ‘grocery list three’ (groceryList[3])?

Oh, don't tell me you fell for that! It was a trick question, Explorer. There's nothing there! If you were to venture so far out to index three (3), you'd be rudely met with an 'out of bounds' exception, a digital slap on the wrist for trying to access memory you were not given access to. So, tread lightly and count carefully with arrays!""

.

.

""All right, explorer, if you've gotten your head around a one-dimensional array, let's kick it up a notch. Get ready for...the two-dimensional array. Sounds stupefying, doesn't it?

Think of a one-dimensional array as a straight line, a single row of elements. Now, take that line and add more lines below it, like a grid. Voila, you have a two-dimensional array. Think of a Matrix such as a Chess or Checker board!

[small removal]

So if you had a 2D array like this:

You could access 'Egg' from Monday's list with ‘weekly grocery list zero, one’ weeklyGroceryList[0][1]. Remember our mantra? Arrays start at zero (0).

Be mindful not to lose yourself in the “matrix”. It's all too easy to get an 'out of bounds' exception if you're not careful. Stay vigilant!""

.

.

Now, let’s take a step back and try writing up our keymap.

For example, in the context of our four by four (4x4) keypad, we could have an array that looks like this:

This two-dimensional (2D) array has four rows and four columns to represent the layout of our keypad.

Look similar to anything? Here’s a photo of a different four by four (4x4) keypad with equivalent functionality as ours from a small company’s “Adventure Kit two (2)” from around the same era as our HERO board was produced.

'In short, arrays let us find different values in a group of values, using an address.""

""Well, well, we're not just going to stare at our spanking new four by four (4x4) keypad, are we? We want to see it in action! But how do we read the button presses, you ask? Time to bring out the heavy artillery - our keypad library. It's got a nifty function tucked away in its folds called getKey(). GetKey() does exactly what it sounds like. It fetches the key that's been pressed. Quite ingenious, really.

'[small removal here]

What do you say we cut to some instructional material that I was able to gather, cached on the computer. I’ve also added this to your mission resources for future reference. You’re welcome, as always.

[inventrkits line-by-line - recorded internally]

""My, you've been busy today! Getting to grips with our four by four (4x4) keypad is no small feat, but you've done splendidly. You're now able to read button presses with the deftness of a seasoned spacefarer. I hope you're starting to see the potential here. You are, after all, building a control system bit by bit. And this is just the beginning!

Now, allow me to entice you with a hint of what's to come tomorrow. ""I'm running a diagnostic on-""

[BOOM! CAMERA SHAKE! ALARM!]

""... Compartment 16 collapsed and is taking on water. I'm on it, don't panic... But...""

[ALARM STOPS]

""... I don't understand. The warning light should have been clearly visible, and you didn't pick up on it? OK, Explorer, we can't risk another hull breach. We need to find a way to keep you aware of potential problems no matter where you're looking. I'll first aid the compartment, you think about alerts...""

[BOOM! CAMERA SHAKE! And the sound of a speaker giving out]

[This time, Astrid is unphased, then releases a sign and gestures a sort of tired facepalm]

""Aaaand, now the alarm system has failed. Guess we got our work cut out for us tomorrow.  ... [trail out]

"	

"/*

* 30 Days - Lost in Space

* Day 11 - Starting your control panel

*

* Learn more at https://learn.inventr.io/adventure

*

* Today we introduce you to a 4x4 button Keypad.  This keypad is very similar to

* what is found on a calculator or even a digital phone.  This keypad uses only 8

* pins to encode 16 buttons by dividing the keypad into rows and columns.  When

* a button is pressed it can be identified by finding which row pin and which column

* pin are pressed.

*

* We simplify this process by using a library written for decoding similar keypads.

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - libraries: Code provided by others that we can use in our sketches

* - char type: Represents a single character like 'A', 'd', '4' or '*'.

* - arrays:	Variables of the same type arranged as a list where each item

*          	can be accessed using an index.  Arrays can be one dimensional

*          	like a list (with one index) or two dimensional like a spreadsheet

*          	table with rows and columns (using row and column indexes).

*

* Parts and electronics concepts introduced in this lesson.

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

/*

* Instead of writing all the code necessary to check all 8 pins and calculate

* which button is pressed we will use a ""library"" that has already been written

* for us and simply call functions in that library.

*

* Please refer to the lesson and lesson videos to load the Keypad library into

* your Arduino IDE before you use it for the first time.

*

* In order to use the Keypad library we include the file ""Keypad.h"".

*/

#include <Keypad.h>

// Our keypad has 4 rows, each with 4 columns.

const byte ROWS = 4;

const byte COLS = 4;

/*

* A one dimensional array is like a list of similar variables.  We use an ""index"" number

* to access each item.  To access values in an array we use the name of the array followed

* immediately by the index in square brackets: name[index].  The index can either be a

* constant (""name[3]"") or a variable containing a value:

*

* byte my_index = 3;

* name[my_index] = value;

*

* Indexes start with zero, not one.  Think of the index as a number *added* to the first

* location.  So, to access the first item in our array we add 0.  To access the second

* item in the array we add 1.

*

* In our first array below, ROW_PINS[0] is set to 5 and ROW_PINS[2] is 3.

*/

// These constant arrays contain the pins connected to the keypad's row and column pins.

const byte ROW_PINS[ROWS] = { 5, 4, 3, 2 };

const byte COL_PINS[COLS] = { 6, 7, 8, 9 };

/*

* Character Type (char)

*

* In Arduino C++ we have previousluy used strings of characters using double quotes (""my

* string"").  But for a single character we need to use single quotes to indicate that there

* is just ONE character ('A').

*/

/*

* Two dimensional array

*

* A two dimensional array combines multiple rows of one-dimensional array into a table

* using two indexes using square brackets.  To access any item in the array the first

* index is added to the row, and then the second index is added to the column.

*

* In the BUTTONS array below BUTTONS[0][0] is set to the character '1' and BUTTONS[2][3]

* is set to the character 'C'.

*/

// Since the keypad has 4 rows of 4 columns each we use a two dimensional array to

// configure what character will be returned when each button is pressed.

const char BUTTONS[ROWS][COLS] = {

{ '1', '2', '3', 'A' },  // Row 0

{ '4', '5', '6', 'B' },  // Row 1

{ '7', '8', '9', 'C' },  // Row 2

{ '*', '0', '#', 'D' }   // Row 3

};

/*

* Using the Keypad.h Library, we are going to define a custom keypad to return

* the characters we wish when each key is pressed.

*

* To do this, we create a heroKeypad object configured with our BUTTONS array,

* the pins used for the rows and columns and lastly how many rows and how many

* columns are on our kepad.

*

* NOTE: this also does the proper pinMode() commands so we don't need them in our setup()

*/

Keypad heroKeypad = Keypad(makeKeymap(BUTTONS), ROW_PINS, COL_PINS, ROWS, COLS);

void setup() {

Serial.begin(9600);  // Initialize the serial monitor

}

void loop() {

// heroKeypad.waitForKey() will wait here until any button is pressed and

// returns the character we defined in our BUTTONS array.

char pressedButton = heroKeypad.waitForKey();  // Wait until a button is pressed

// Display the character returned for the button that was pressed.

Serial.println(pressedButton);

}"

Here is the script and code used in the video for Day 12:

"""Good morning, explorer. I’ve managed to stop the leak from yesterday, but the alarm system is still totally busted. I trust you've had your morning cup of rocket fuel, because today is going to be a melody of learning. Today, we're going to make some noise. Literally. We're introducing another piece of our cosmic puzzle, the ‘passive buzzer’.

Now don't be fooled by its rather mundane name; this little one can make quite a racket when it wants to. It's a small speaker that can play different frequencies, and we're going to use that ability to our advantage.

Today's mission, should you choose to accept it, is to turn our four by four (4x4) keypad into an eight-bit (8-bit) sounding piano keyboard. That's right, by the end of today, your control panel will have its very own mini synth.  Once we can verify that it’s working, we can re-wire up our alarm system. How's that for turning it up to eleven?

So, buckle in, check your dials, and let's hit the launch button on this day. Sound good? I sure hope so!""

.

.

""Let's talk about our new comrade-in-arms, the ‘passive buzzer’. This small, usually cylindrical component is capable of generating a range of sound frequencies. It's like a tiny speaker that you can program.

In the electrical universe, the passive buzzer is a piezoelectric device. When an oscillating electrical signal is applied to its terminals, it vibrates and creates sound. We control the frequency of this oscillation, which translates to the pitch of the sound that it generates.

To put it simply, you give it a frequency and tell it how long to play it. Simple. To accomplish this, we can use ‘pulse width modulation’, and it sings that note back to you.

So what do you say, Explorer? Let's get ready to compose some galactic symphonies, shall we?""

.

.

Now, in order to make the passive buzzer produce any sound, we need to send it some sort of frequency on to its signal pin. In terms of music, using a different frequency will produce a different sound. It is not possible to generate tones lower than thirty-one hertz (31hz) and struggles to produce higher than four-thousand-nine-hundred-seventy-eight hertz (4978hz). If you want an optional additional library, it may be in your best interest to check out the tone-h (Tone.h) library. That way if you want a specific note, you can say ‘note c-five’ (“NOTE_C5”) instead of some arbitrary number like five-hundred-twenty-three (523). I’ve added a copy to your mission resources.

Luckily, sending those tones is made easy with a built-in function called “tone()”. Tone() takes two parameters, the pin and the frequency you want to send. Optionally, you can also put a third for duration.

.

.

""Now, let’s brainstorm together, Explorer. We want each button on our keypad to trigger a specific tone. Oh, I can hear you now... 'Let's just churn out a bunch of if statements, one for each button.' If button one (1) is pressed, play two-hundred hertz (200Hz). If button A is pressed, play six-hundred hertz (600Hz). And so forth. A simplistic approach, but not very elegant, is it?

Instead, let's introduce some finesse into our coding journey. How about we conjure up a second two-dimensional (2D) array? We'll christen it as something suitable, like 'tones'—a title befitting its role. And then, there you have it, we only need ONE if statement.

This method is neater and easier to debug. Plus, it's a great way to flex those hefty programming muscles we've been developing. Let’s continue, shall we?""

I want to take the time to introduce a new programming concept - loops. I’m sure you already know about our loop() function, but the loops we will be talking about today are used in nearly all programming languages universally and are in code everywhere. What I’m referencing are called 'for' and 'while' loops.

""For loops execute a block of code a number of times, and usually used if you, or the code, knows the number of times it should loop. A For loop looks like this (graphic), and declares the initial loop counter value here, a test of that counter, and by how much the counter changes with each loop.""

In this example, 'counter' starts at zero (0), and the loop will continue until 'counter' is no longer less than ten (10). Each time the loop runs, 'i' increases by one (1), which is what ‘i-plus-plus’ ('i++') signifies. Once i is no longer less than 10, the loop is broken and the program moves on to the next line of code.

Next, let's move on to 'while' loops. These are used when we don't know how many times the loop will need to run. It just keeps running as long as a certain condition is true. Once that condition is no longer true, the loop stops. Here's the format:

As long as 'condition' is true, the code inside the curly braces will run. Make sure to alter something within the loop to eventually break the condition, or you'll end up with an endless loop—a space explorer's worst nightmare, believe me.

You get all that? Allow me to give you a snappy rundown then. 'For' loops: you know how many times you need to do a thing. 'While' loops: you don't know how many times you'll do the thing, you just do it until a condition stops being true. You’ve got this, Explorer! I am about ninety-four-point-eight-six-three-percent (94.863%) sure of it.

Fun little trick for you if you want to use it: if you add ‘while(true);’ at the end of setup(), you can halt your code there, to debug the setup() function. You can technically do that wherever in your code to help debug before a specific point.

I suppose I’ve babbled on enough for now. Let’s cut to some instructional material that I was able to gather, cached on the computer. I’ve also added this to your mission resources for future reference.

[inventrkits line-by-line - recorded internally]

""Excellent work today, Explorer. You've successfully orchestrated a symphony of bleeps and bloops with your newly installed passive buzzer and keypad. Music to my artificial ears. You’re doing relatively well for a species that requires a prolonged period of time to rest in order to function.

You've got the hang of loops, and you've even started controlling the pitch frequencies to create different sounds. You're beginning to command this eight-bit (8-bit) world like a maestro.

But, as it should be abundantly clear by now, we are far from done. Tomorrow, we'll take this keypad-buzzer duo to a whole new level. Instead of just a melodic instrument, we're transforming it into a high-tech security system. A Personal Identification Number (PIN) is the game, and you'll learn how to integrate it into your control panel.

Rest well, for day thirteen (13) comes with its own set of challenges. Will you rise to the occasion? Time will tell. As for me, I will be ready as I always am.""

.

.

""Wait! Hold on a moment, Explorer. It seems like we've just received an unexpected data transmission. It's absolutely uncanny how the unexpected always seems to pop up in the most inconvenient of times. For your sake, I hope you're as nimble with surprises as you are with your coding.

I've deciphered the transmission, and it appears to be...code? Yes, an Arduino sketch. It could be an important piece to our ongoing mission. My digital intuition tells me you should run it on your HERO board.

Here's how to proceed:

Step one (1): Save any ongoing work. This could get a bit... disruptive.

Step two (2): Clear your current sketch and replace it with the new code.

Step three (3): Compile and upload the code to your board. And remember, Explorer, always anticipate the unexpected when executing unknown code.

While the code is running, I'll keep scanning for any related signals. And, of course, I'll be here to guide you through any technical issues. Onward we go, Explorer. Let's crack this code!""

"	

"/*

* 30 Days - Lost in Space

* Day 12 - Can you hear us?

*

* Learn more at https://learn.inventr.io/adventure

*

* Today we will use the Keypad from Day 11 to control musical TONES using a

* small speaker.  To do this we will show how to take the characters returned

* by the Keypad library and convert them to musical notes played by our speaker.

* Tones will play until changed or stopped (by pressing the 'D' button at the

* bottom right of our 4x4 button matrix)

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - libraries: Code provided by others that we can use in our sketches

* - char type: Represents a single character like 'A', 'd', '4' or '*'.

* - arrays:	Variables of the same type arranged as a list where each item

*          	can be accessed using an index.  Arrays can be one dimensional

*          	like a list (with one index) or two dimensional like a spreadsheet

*          	table with rows and columns (using row and column indexes).

* - for loop:  Used to repeately perform actions.

*

* Parts and electronics concepts introduced in this lesson.

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Include Keypad library

#include <Keypad.h>

const byte ROWS = 4;

const byte COLS = 4;

const byte ROW_PINS[ROWS] = { 5, 4, 3, 2 };

const byte COL_PINS[COLS] = { 6, 7, 8, 9 };

char BUTTONS[ROWS][COLS] = {

{ '1', '2', '3', 'A' },  // 1st row

{ '4', '5', '6', 'B' },  // 2nd row

{ '7', '8', '9', 'C' },  // 3rd row

{ '*', '0', '#', 'D' }   // 4th row

};

Keypad myAwesomePad = Keypad(makeKeymap(BUTTONS), ROW_PINS, COL_PINS, ROWS, COLS);

const unsigned int TONES[ROWS][COLS] = {

// a frequency tone for each button

{ 31, 93, 147, 208 },

{ 247, 311, 370, 440 },

{ 523, 587, 698, 880 },

{ 1397, 2637, 3729, 0 }  // Use frequency of 0 for bottom right key to end tone.

};

const byte BUZZER_PIN = 10;  // pin 10 drives the buzzer

void setup() {

Serial.begin(9600);  // Begin monitoring via the serial monitor

}

void loop() {

char button_character = myAwesomePad.waitForKey();  // Wait for a button to be pressed

/*

* For loops:

* for() has three parts.  Each is part is :

*          	START - operations performed when loop starts

*          	CONDITION - evaluate this condition as a boolean and stop loop

*                      	when false

*          	STEP - operations performed every time for loop ends

*

* NOTE: you will often see the short variable names ""i"", ""j"" and ""k"" used

*   	for variables in the for statement.  Those particular letters are used

*   	mostly for historical reasons going back to the FORTRAN language, but

*   	the variables in loops tend to be short in order to keep the for command

*   	readable.  The short names also remind you when used that they change

*   	each time through the loop.

*/

unsigned int tone_frequency = 0;  // Frequency to use for tone (default to 0, no tone)

for (byte i = 0; i < ROWS; i++) {

for (byte j = 0; j < COLS; j++) {

if (button_character == BUTTONS[i][j]) {  // found it, get the corresponding tone

tone_frequency = TONES[i][j];

}

}  // end j loop

}	// end i loop

Serial.print(""Key: "");  //   send the button_character to serial monitor...

Serial.print(button_character);

Serial.print(""   Freq: "");

Serial.println(tone_frequency);

/*

* The tone() function plays a tone until stopped.  The code continues to run as the tone plays.

*/

if (tone_frequency > 0) {            	// If tone frequency greater than 0...

tone(BUZZER_PIN, tone_frequency);  // ...then play the tone at that frequency until stopped

} else {

Serial.println(""Stop tone"");

noTone(BUZZER_PIN);  // Stop pressed (tone frequency of 0) so stop any tone playing

}

}"

Here is the script and code used in the video for Day 13:	
"""Ah, welcome back, dear Explorer. I trust you managed to get some rest after yesterday's, umm, unexpected musical interlude. It appears someone out there in the vast cosmos has a sense of humor. A 'rick roll', as you Earthlings call it? Quite the antique prank, but I suppose humor is scarce in the vacuum of space.

Anyway, it's time to get back to business, don't you think? The endless void isn't going to explore itself. On the docket for today, we have some interesting challenges lined up.

To enhance the security of our ship's control panel, we will be utilizing our keypad. We're going to set a Personal Identification Number or pin (PIN). This will prevent any mischievous extraterrestrial beings from fiddling with our control panel, or a future you, pranking the past you. Time gets pretty wibbly-wobbly in space, you know.

Additionally, we're going to make use of our passive buzzer to provide auditory feedback. This will help ensure that each key press gets the recognition it deserves, and gives you, the user, feedback that an input has been registered.

Well then, let's not delay any longer, Explorer. Let's dive in and ensure our systems are secure and interactive. Onward to the adventure!""

.

.

""Ah, the beauty of continuity, dear Explorer. You see, the marvel of the universe is that it remains, for the most part, constant. Stars keep burning, planets keep spinning, and black holes... well, they continue being quite black and very hole-ish.

Likewise, your wiring from yesterday remains perfectly suited for today's exploits. No need for new knots of wires to disentangle. The cosmos has blessed us with a touch of simplicity today.

So, we will bypass the wiring stage this time and leap directly into the vast and mysterious world of code. Brace yourself, Explorer, because today, we dive headfirst into the celestial sea of ones (1’s) and (0’s) zeros. Let's embark on this code adventure, shall we?""

.

.

""Life as an interstellar explorer may feel solitary, and indeed, it mostly is. However, one mustn't overlook the odd chance of bumping into... unwelcome guests. After all, space isn't a private garden; it's a vast, unending sea teeming with possibilities, and yes, even potential space pirates. Don't look so shocked. It's not all fun and games out here! It would be awfully dreadful if your ship’s autopilot got hijacked mere moments after finally getting out of this planet's atmosphere. You’re welcome to get stuck again, but I think I’ll pass, thank you.

So, with that merry thought, we turn our attention to an essential system in our spacecraft: the security system. Today, we'll give that a nice little upgrade. We're going to add a personal identification number, or pin (PIN), to our control panel.

Consider our two-dimensional (2D) array from two days back. Our beautiful little keypad with all those character inputs. Now, let's assign two of those keys for some functional fun. One key, an asterisk ('*'),

will let us change the PIN. The other key, a pound symbol ('#'),

will be our magic access key. Press that, punch in the correct pin (PIN), and there you have it. The control panel is all yours!

Speaking of pins (PINs), we need a default one, don't we? Something nice and simple to start with - how about zero-zero-zero-zero ('0000')? And before you roll your eyes, remember, this will be changed immediately after.

A quick note on system integrity. You see, we only want to allow pin (PIN) changes if the user, which in this case is you, already knows the pin (PIN). A little layer of security, if you will. So, we'll split this logic into a separate function to prevent duplication.

Remember our discussion about functions? Yes, we created one using 'void' because we didn't need it to return anything. Well, that is no longer the case. We need to know if the pin (PIN) was correct or not. So, let's switch 'void' with 'bool'.

When we call this function, it will now return a boolean value, either “true” if the PIN is correct, or “false” if it is not. We also need to make sure to add the proper “return” statements as well where needed.

And don't worry about the function going on a spree of calculations; once it hits 'return', it jumps right back from where it was called.

I've got a line-by-line breakdown of today's code waiting for you. I advise you to take a look. It's packed with knowledge and could very well be your key to survival in this cosmic expanse. Off you go then, Explorer!""

Please make your way over to the instructional material that I was able to gather, cached on the computer. I have also added it to your mission resources for future reference, should you so need it.

[inventrkits line-by-line - recorded internally]

""What an impressive day, Explorer! You've stepped up to the challenge and fortified your control panel like a true galactic defender. Just imagine the disappointment on the face of any rogue mothership attempting to hijack your controls – delightful!

I daresay our control panel is looking quite formidable. But wouldn't it be nice to add a touch of theatricality to it? Tomorrow we'll bring back our trusty R-G-B L-E-D (RGB LED) from last week. And to add more panache, we'll also integrate some code logic for our buzzer to create a symphony of auditory and visual feedback on pin (PIN) input. Lights and sounds for every action – the perfect command center for a sophisticated space explorer!

Take a well-earned rest now. Remember, interstellar conquering requires both body and mind to be at their sharpest. Sleep well, and get ready to add more intergalactic glamour to your ship tomorrow.""

"	

"/*

* 30 Days - Lost in Space

* Day 13 - HERO Security 101

*

* Learn more at https://learn.inventr.io/adventure

*

* Playing tones was fun, but now it's time to get back to business.  Let's use

* the 4x4 button keypad input a Personal Identification Number (PIN) to unlock

* access to the lander's control panel.

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - return:  Used by functions to return a single value and return immediately

*        	to the caller of the function.

* - boolean type: bool is used for a value that can be ""true"" or ""false""

*             	NOTE: when testing values for true/false the value 0 is considered

*                   	""false"" and any non-zero value is considered ""true"".

*

* Parts and electronics concepts introduced in this lesson.

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Include Keypad library#include <Keypad.h>

#include <Keypad.h>

// Our HERO keypad has 4 rows, each with 4 columns.

const byte ROWS = 4;

const byte COLS = 4;

const byte PIN_LENGTH = 4;	// PIN code is 4 button presses

char current_pin[PIN_LENGTH] = { '0', '0', '0', '0' }; // Initial PIN is four zeros.

// Define what characters will be returned by each button

const char BUTTONS[ROWS][COLS] = {

{ '1', '2', '3', 'A' },

{ '4', '5', '6', 'B' },

{ '7', '8', '9', 'C' },

{ '*', '0', '#', 'D' }

};

// Define row and column pins connected to the keypad

const byte ROW_PINS[ROWS] = { 5, 4, 3, 2 };

const byte COL_PINS[COLS] = { 6, 7, 8, 9 };

Keypad heroKeypad = Keypad(makeKeymap(BUTTONS), ROW_PINS, COL_PINS, ROWS, COLS);

const byte BUZZER_PIN = 10;  // pin 10 drives the buzzer

void setup() {

pinMode(BUZZER_PIN, OUTPUT);

Serial.begin(9600);  // Begin monitoring via the serial monitor

delay(200);      	// Delay a brief period to let things settle before displaying prompt.

Serial.println(""Press * to enter new PIN or # to access the system."");

}

void loop() {

char button_character = heroKeypad.waitForKey();

// Serial.println(button_character);

tone(BUZZER_PIN, 880, 100);

if (button_character == '#') {  // button to access system

bool access_allowed = validatePIN();

if (access_allowed) {

Serial.println(""Welcome, authorized user. You may now begin using the system."");

} else {

Serial.println(""Access Denied."");

Serial.println(""\nPress * to enter new PIN or # to access the system."");

}

}

if (button_character == '*') {  // button to change PIN

bool access_allowed = validatePIN();

if (access_allowed) {

Serial.println(""Welcome, authorized user. Please Enter a new PIN: "");

for (int i = 0; i < PIN_LENGTH; i++) {

button_character = heroKeypad.waitForKey();

tone(BUZZER_PIN, 880, 100);

current_pin[i] = button_character;

Serial.print(""*"");

}

Serial.println();  // add new line after last asterisk so next message is on next line

Serial.println(""PIN Successfully Changed!"");

} else {

Serial.println(""Access Denied. Cannot change PIN without the old or default."");

Serial.println(""\nPress * to enter new PIN or # to access the system."");

}

}

}

/*

* This function prompts the user to enter a PIN and returns true or false depending

* on whether the PIN matches our saved PIN.

*

* NOTE: this function introduces the concept of a function that returns a single

*   	value.  First, we must indicate what type of value that will be returned

*   	in the initial function declaration.  The ""bool"" at the beginning of

*   	the declaration indicates that this function will return a ""boolean"" value

*   	of either true or false.

*/

bool validatePIN() {

Serial.println(""Enter PIN to continue."");

for (int i = 0; i < PIN_LENGTH; i++) {

char button_character = heroKeypad.waitForKey();

tone(BUZZER_PIN, 880, 100);

if (current_pin[i] != button_character) {

Serial.println();  // start next message on new line

Serial.print(""WRONG PIN DIGIT: "");

Serial.println(button_character);

return false;  // return false and exit function

}

Serial.print(""*"");

}

Serial.println();  // add new line after last asterisk so next message is on next line

Serial.println(""Device Successfully Unlocked!"");

return true;

}"

Here is the script and code used in the video for Day 14:
"""Good morning, intrepid Explorer! It's time for the final day of our third week together, and I do hope you're as excited as I am. Today we have a delightful rendezvous with our former crew member - the R-G-B L-E-D (RGB LED). We'll be reintegrating it into our setup and taking our control panel from functional to fabulous.

Today, we'll be transforming our buzzer's simple beep into an expressive and nuanced system of auditory feedback. Not only that, but we're going to coordinate it with our R-G-B L-E-D (RGB LED) to provide clear visual cues depending on your pin (PIN) input. Think of it as a form of immediate feedback.

When you successfully input your pin (PIN), we will immerse our spaceship in a comforting, green light. On the contrary, if your input fails, we will be met with a piercing red light as a sign of incorrect input. Green for go, red for no. It's so universal that even a celestial body could understand!

Since there are no new components today, we will be diving headfirst into the coding and wiring. Ready to create a sophisticated and intelligent spaceship? Let's get started!""

.

.

""Alright, time for some wiring! If you remember how we set up our R-G-B L-E-D (RGB LED) last week, you're already one step ahead! Today, all we need to do is to merge that R-G-B (RGB) setup with our current control panel configuration. Think of it as a bit of celestial (....or I guess aquatic?) engineering - we're taking two previously independent systems and merging them into one harmonious unit. Quite beautiful, isn't it? We're building a network of devices that can interact and coordinate with each other in a most splendidly efficient manner!

Just remember to ensure your wires don't look like a tangled seaweed cluster - we need our circuits clear and our paths unobstructed. Once you've achieved this glorious fusion of wires and components, we can shift gears and explore the vast depths of coding. Buckle up, it's about to get exciting!""

.

.

""All right, my fearless bottom-dwelling mechanic, it's time to lay out today's game plan. Here's what we're aiming to do:

Step one (1):. We want to bathe the R-G-B (RGB) in an emerald green glow when our system is either unlocked or a correct password has been punched in. Contrarily, let it flare up with fiery red rage when an erroneous input infiltrates the system.

Step two (2):. Let's turn that buzzer into our personal symphony, composing melodies of 'success' and 'failure' as needed.

Step three (3):. Moreover, we'll make that buzzer emit a brisk beep every time a button is pressed during the pin input sequence. While we're at it, why not manipulate the R-G-B’s (RGB's) hue to the shades of yellow or blue? This way, we can visually signal that we're in the midst of pin-input. Clever, no?

I've been tinkering with a plan and I've decided that the most efficient and organized way to handle all this is by developing three new functions: one to express success, one to denote error, and one to indicate ongoing pin input. Plus, we'll breathe life into the R-G-B L-E-D (RGB LED) color function from last week.

By now, creating new functions should be as easy as breathing for you, my dear spacefarer.

As for our friendly buzzer, let's make it sing by using the tone() function. A high-pitched note will be our anthem of victory, while a low, solemn tone will signify defeat.

The bones of our code will remain as sturdy as they were yesterday. Don’t forget to add a tiny update at the top to define our L-E-D (LED) pins.

As usual, I've crafted a line-by-line explanation of today's code and added it to your mission resources. While it builds on what we did yesterday, I highly recommend giving it a look over - your survival does hinge on understanding this, after all!""

Whew! The days sure are flying by, Explorer. Now seems like a great time to grant me a well deserved break as you cut to some instructional material that I have gathered, cached on the computer. As you know, I’ve also added this to your mission resources in case future you needs a bit more assistance.

[inventrkits line-by-line - recorded internally]

""Well, look at you, fishfood! We've reached the end of day fourteen (14). And may I say, you've done spectacularly! I'm impressed - no, astonished - at the pace at which you're conquering these obstacles. Halfway through our ship repairs, and you're still firing on all thrusters! My digital heart, if I had one, would be brimming with pride.

It's remarkable, isn't it, how you've been able to adapt and learn, navigating through uncharted territories of tech wizardry, all in a hostile environment millions of miles from home? From securing the control panel to now giving it visual and auditory feedback, you've become a force to be reckoned with in space engineering.

Rest now, weary traveler. You've earned your sealegs. Let the fish whisper tales of your endeavors across the ocean as you recharge your human batteries. And remember, the tides wait for no one - tomorrow, we swim onward. Congratulations on reaching this milestone. It's not every day one gets to say they're halfway done repairing a spaceship stranded in coral, now is it?"""	

"/*

* 30 Days - Lost in Space

* Day 14 - Advanced Security Panel

*

* Learn more at https://learn.inventr.io/adventure

*

* Time to enhance our security system, taking it from functional to fabulous.  We'll

* add in our RGB LED to give us visual feedback and also expand the tones that play

* to give us audible feedback as you enter your PIN.

*

* IMPORTANT NOTE: The wiring for this sketch builds on the Day 13 wiring. But the

*             	addition of the RGB LED requires us to MOVE some of the Day 13

*             	wires.  This is because the RGB LED only works when connected to

*             	pins that support Pulse Width Modulation (PWM).  HERO pins that

*             	support PWM have a tilde ('~') preceeding their pin numbers.

*

*             	If you look at the Day 13 diagrams you see that we used PWM pins

*             	for the keypad and buzzer, leaving only one available.  Pins 12

*             	and 13 are still available so we'll move the wires on pins 9 and 10

*             	(PWM pins) to 12 and 13.

*

*             	Pin 13 is also used while the Arduino IDE is uploading sketches, and

*             	if you connect the buzzer to THAT pin you get noise every time you

*             	upload a new sketch (try it!).  So the buzzer will now be on pin 12

*             	and the keypad wire on pin 9 will move to pin 13.

*

*             	Because we used constants, we only need to make two small changes

*             	from the Day 13 sketch instead of hunting down and changing each use.

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - complex logic with multiple devices

*

* Parts and electronics concepts introduced in this lesson.

* - Changing wiring when particular pins (PWM) are required for added components.

* - Tone() and PWM analogWrite() cannot both operate at the same time.

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Include Keypad library

#include <Keypad.h>

// Our HERO keypad has 4 rows, each with 4 columns.

const byte ROWS = 4;

const byte COLS = 4;

const byte PIN_LENGTH = 4;                       	// PIN code is 4 button presses

char password[PIN_LENGTH] = { '0', '0', '0', '0' };  // Initial password is four zeros.

// Define what characters will be returned by each button

const char BUTTONS[ROWS][COLS] = {

{ '1', '2', '3', 'A' },

{ '4', '5', '6', 'B' },

{ '7', '8', '9', 'C' },

{ '*', '0', '#', 'D' }

};

// Define row and column pins connected to the keypad

const byte ROW_PINS[ROWS] = { 5, 4, 3, 2 };

const byte COL_PINS[COLS] = { 6, 7, 8, 13 };  // NOTE wire moved from Day 13's sketch to pin 13

// Create our keypad object from the keypad configuration above

Keypad heroKeypad = Keypad(makeKeymap(BUTTONS), ROW_PINS, COL_PINS, ROWS, COLS);

const byte BUZZER_PIN = 12;  // NOTE that pin 12 drives the buzzer now

const byte RED_PIN = 11;	// PWM pin controlling the red leg of our RGB LED

const byte GREEN_PIN = 10;  // PWM pin ccontrolling the green leg of our RGB LED

const byte BLUE_PIN = 9;	// PWM pin ccontrolling the blue leg of our RGB LED

void setup() {

pinMode(RED_PIN, OUTPUT);

pinMode(GREEN_PIN, OUTPUT);

pinMode(BLUE_PIN, OUTPUT);

// Since we start out ""locked"", we initially display a red color.

displayColor(128, 0, 0);

Serial.begin(9600);  // Begin monitoring via the serial monitor

Serial.println(""Press * to set new password or # to access the system."");

}

void loop() {

char button_character = heroKeypad.getKey();  // Wait for a button press and save character

// The '#' button unlocks our console

if (button_character == '#') {      	// Button to access system

giveInputFeedback();              	// Short beep with blue light

bool access_allowed = validatePIN();  // Have user enter PIN to allow access

if (access_allowed) {

Serial.println(""Welcome, authorized user. You may now begin using the system."");

} else {

Serial.println(""Access Denied."");

Serial.println(""\nPress * to enter new PIN or # to access the system."");

}

}

// The '*' allows the user to enter a new PIN after entering current PIN for security

if (button_character == '*') {      	// Button to change password

giveInputFeedback();              	// Short beep with blue light

bool access_allowed = validatePIN();  // Have user enter PIN to allow access

if (access_allowed) {

displayColor(128, 80, 0);  // Display yellow LED while entering a NEW PIN

Serial.println(""Welcome, authorized user. Please Enter a new password: "");

// Use a for() loop that runs once for each PIN character.  Each character read

// replaces a character in our original PIN.

for (int i = 0; i < PIN_LENGTH; i++) {

password[i] = heroKeypad.waitForKey();  // replace PIN character with new character

// NOTE: After the LAST button press we will give Success feedback, but up to the

//   	last character we simply give input feedback.  Since Array indices start

//   	at 0, this if() statement will be true for all but the LAST character of

//   	the new PIN.

if (i < (PIN_LENGTH - 1)) {

giveInputFeedback();

displayColor(128, 80, 0);  // override color

}

Serial.print(""*"");

}

Serial.println();  // add new line after last asterisk so next message is on next line

Serial.println(""PIN Successfully Changed!"");

giveSuccessFeedback();  // TADA sound and green light for successful PIN change

} else {

Serial.println(""Access Denied. Cannot change PIN without entering current PIN first."");

Serial.println(""\nPress * to enter new PIN or # to access the system."");

}

}

}

// Enter PIN and return false for bad PIN or true for good PIN

bool validatePIN() {

Serial.println(""Enter PIN to continue."");

for (int i = 0; i < PIN_LENGTH; i++) {

char button_character = heroKeypad.waitForKey();

if (password[i] != button_character) {

giveErrorFeedback();  // Error sound and red light

Serial.println(); 	// start next message on new line

Serial.print(""WRONG PIN DIGIT: "");

Serial.println(button_character);

return false;  // return false and exit function

}

// Give normal input feedback for all but the LAST character

if (i < (PIN_LENGTH - 1)) {

giveInputFeedback();  // Short beep and blue LED

}

Serial.print(""*"");

}

giveSuccessFeedback();  // PIN matched - TADA! sound with green LED

Serial.println();   	// add new line after last asterisk so next message is on next line

Serial.println(""Device Successfully Unlocked!"");

return true;

}

// Display a color by providing Red, Green and Blue intensities (0-255)

void displayColor(byte red_intensity, byte green_intensity, byte blue_intensity) {

analogWrite(RED_PIN, red_intensity);  	// Set red LED intensity using PWM

analogWrite(GREEN_PIN, green_intensity);  // Set green LED intensity using PWM

analogWrite(BLUE_PIN, blue_intensity);	// Set blue LED intensity using PWM

}

/*

* NOTE:

* Both the PWM functions and tone() functions use some of the same HERO hardware

* for their functions.  If the RGB LED is on while a tone is played it will flicker

* or diplay other colors.  Because of this, we turn off the LED while a tone is

* being played and restore it immediately afterwards.

*/

// A recognized button was pressed.  Give short beep and blue LED

void giveInputFeedback() {

displayColor(0, 0, 0);  // Turn off LED while playing tone

tone(BUZZER_PIN, 880, 200);

delay(200);           	// Delay while tone is playing because tone() returns immediately

displayColor(0, 0, 128);  // Restore blue LED

}

// A matching PIN has been entered or a new PIN has been accepted.

// Play TADA! sound and display green LED

void giveSuccessFeedback() {

displayColor(0, 0, 0);  // Turn off LED while playing tone

tone(BUZZER_PIN, 300, 200);

delay(200);  // Delay while tone is playing because tone() returns immediately

tone(BUZZER_PIN, 500, 500);

delay(500);  // Delay while tone is playing because tone() returns immediately

displayColor(0, 128, 0); // Display green LED

}

// Bad PIN entered.  Play descending tone and display red LED

void giveErrorFeedback() {

displayColor(0, 0, 0);  // Turn off LED while playing tone

tone(BUZZER_PIN, 300, 200);

delay(200);  // Delay while tone is playing because tone() returns immediately

tone(BUZZER_PIN, 200, 500);  // Frequency = 200Hz, Duration = 200ms

delay(500);  // Delay while tone is playing because tone() returns immediately

displayColor(128, 0, 0);  // Display red LED

}"

Day 15   IS A CREATIVE DAY, SO WE DO NOT HAVE ANY CODE OR SCRIPTS. INSTEAD WE INSPIRE THE STUDENTS TO MAKE SOMETHING NEW ON THEIR OWN

Here is the script and code used in the video for Day 16:	
"""A new week dawns, my intrepid explorer, and it brings a fresh set of challenges to our undersea sojourn. You may have been enjoying the soothing serenity of the deep ocean, but it's time to point our sights towards the surface. Now don't get too excited - we can't just jet up there like a dolphin on an energy drink. There are imperative protocols to be followed.

As any experienced Explorer, such as yourself, knows,rising to the surface too quickly will reduce pressure that will cause our spaceship to explode like a shaken soda bottle and I believe neither of us would be particularly fond of that outcome.

First things first: power. We need all we can muster, so it would be best to attempt our surface-bound journey when power generation is at its peak. That means mid-day. To do that, however, we need to know when exactly that is. In other words, we need to build a clock.

There's also the matter of the encrypted source code provided to us. Decrypting it is crucial for all of our plans to unfold. So, we have our work cut out for us this week. We need to create a clock, decrypt the code, and most importantly, limit the rate of ascend to prevent us from becoming a bursting submarine balloon

Now, let's put our crafting cap on and get down to business. We've got a spaceship to guide home.""

.

.

""Right then, let's dive into the fascinating world of timekeeping. To craft our clock, we'll be making good use of a charming little component in our kit: the seven-segment (7-segment) display. Intrigued? Well, allow me to explain.

A 7-segment display looks like this. It contains a total of 7 LEDs A-G in an “8” formation.  Each of these LEDs can be individually illuminated to make numbers and other characters.  Simple, yet ingenious.

Let’s Imagine a digit. How about an eight (8)? Notice how it's essentially a formation of different horizontal and vertical segments. Now, think about how you can form other numbers by lighting up or turning off different combinations of these segments. If we turn off the horizontal segment in the center, for example, we turn our eight (8) into a zero (0)! Behold - the power of a seven-segment (7-segment) display. Simple, yet incredibly useful!

It is not my intention to befuddle you, dear Explorer, but the digits aren't the only things a seven-segment (7-segment) display can render. With a bit of creativity, you can also display a few letters and symbols, but we’ll save those for later.

For now, find security in understanding the basic concept of a seven-segment (7-segment) display and prepare to harness the power of this nifty little device to create our clock. Ready for a bit of digital wizardry, Explorer?""

.

.

""Alright, Explorer, buckle up as we dive into the wiring part of our adventure today. You'll be relieved to know it's quite straightforward with just four wires needed for our seven-segment (7-segment) display module. Here's how we do it:

As always, we connect the ... ground wire here, and positive here. These are our data pins, and need to go here on our digital pins.

Our friend, C-L-K (CLK), stands for ‘Clock pin’. Its job is to compile the binary data streaming from the D-I-O (DIO) pin into a byte. One cycle equals one byte.

Our other friend, D-I-O (DIO) stands for ‘Digital Input/Output’. Its role is to receive binary data for the HERO board, which in turn, determines which segments of our display are going to be dazzling us with their lights and which ones will remain in the dark.

Think of it like a switchboard operator of the olden days, but for data bits. Now, an eight-bit (8-bit) binary number provides just enough digits to control all these segments, and even gives us a few to spare.

So there you have it - simple, isn't it? The magic of making numbers out of light is within our grasp, so let's press on, shall we?""

.

.

""Now, Explorer, we're going to make a bit of a shift from our hardware chat and delve into the coding realm for the day. Here's where we bring in our secret weapon - ... a brand-new library.

Rest assured, Explorer, I’m happy to provide you with a refresher on library installation. Here’s how it goes. Navigate to the 'Sketch' tab in the top menu, then move to 'Include Library' and click on 'Add dot-Z-I-P (.ZIP) Library'. This way, we'll get all the resources we need without having to invent the wheel all over again. It's quite handy, isn't it?""

Do be sure to add the library to the top of our code so we can utilize it.

Now, by creating this new object, we’ll be granted access to the entire library of the new seven-segment (7-segment) functions. Remember, you must not forget to define those pins!

The one we’ll be most interested in is the set-segments (setSegments) function.

This function is like the puppet master behind our seven-segment (7-segment) display.

Allow me to break it down for you. Each of the seven segments of our display, labeled A to G, can be controlled individually and can combine to form different characters. Each segment can either be turned on, meaning it lights up, or off, where it remains dark.

The ‘set-segments’ ('setSegments') function helps us control which segments light up and which do not. It's essentially taking the reins and directing our display to show specific characters.

How does it know what to display, you ask? It takes an array of segment states as an argument. Each bit in the array corresponds to a specific segment. When a bit is ‘one’ ('1'), the corresponding segment lights up. When it's ‘zero’ ('0'), the segment remains off.

In essence, ‘set-segments’ ('setSegments') helps us paint pictures, or rather numbers and letters, on our seven-segment (7-segment) display by controlling the light show for each segment. Pretty cool, isn't it?""

.

.

""Alright, Explorer! It's time to test your understanding of our seven-segment (7-segment) display! You see, each segment, labeled from A to G, represents a different L-E-D (LED) that we can command to light up or remain off. With this in mind, let's dive into some examples.

First up - think of which L-E-Ds (LEDs) we'd need to ignite to display the number ‘seven’ ('7')? If A, B, and C came to mind, then I’d say you're getting the hang of this!

What do you say we step it up a notch? Think of which L-E-Ds (LEDs) we need to light up to spell out the letter 'A'? Whew, I can see those gears turning! By my calculations, it appears you’re  thinking A, B, C, E, F, and G. Well, dear Explorer, that would be correct!

Now, for our final round, let's challenge ourselves a bit! Can you figure out which L-E-Ds (LEDs) we'd need to illuminate to spell out the word 'dOnE'? This is a bit trickier, but I know you've got this!

Remember, practice makes perfect. Keep playing around with combinations and soon you'll master the art of the seven-segment (7-segment) display!""

It seems to be that time where we cut to some instructional material that I have gathered, cached on the computer. I’ve also added this to your mission resources for future reference. Good luck!

[inventrkits line-by-line - recorded internally]

.

.

""Well done, my tenacious explorer! We've come a long way today. Our seven-segment (7-segment) display is now alive and illuminating. It is quite literally the beacon of your progress. Your deftness in handling the display truly proves your skill and dedication. However, as we gaze at our journey ahead, a challenge looms.

Our control systems are under lock and key with a passcode tucked away under layers of encryption. Cracking this passcode is our task for the morrow. It's not going to be easy, but who said space exploration was a walk in the park?

Rest up, my friend. Tomorrow we step into the enigmatic world of encryption. Be ready to wield your intellect against this intricate puzzle. You've got this. Goodnight, Explorer. See you on the other side!”

"	

"/*

* 30 Days - Lost in Space

* Day 16 - A Fancy New Display

*

* Learn more at https://learn.inventr.io/adventure

*

* Sound and light gives us some good feedback from our HERO, but what about

* when we need numbers and the Serial Console isn't available?  Today we

* introduce a 4-digit 7-segment display (TM1637).  This is mostly for numbers,

* like a counter or perhaps a clock display, but with some imagination we can

* also show some characters.

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - Binary notation (0b00000001)

* - logical OR

* -

*

* Parts and electronics concepts introduced in this lesson.

* - TM1637 4-digit 7-segment display

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

/*

* Please refer to the lesson and lesson videos for instructions on how to

* load the TM1637 library into your Arduino IDE before you use it for the

* first time.

*

* In order to use the TM1637 library we include the file ""TM1637Display.h"".

*/

#include <TM1637Display.h>

// all_on pins connected to the TM1637 display

const byte CLK_PIN = 6;

const byte DIO_PIN = 5;

// Create display object of type TM1637Display:

TM1637Display lander_display = TM1637Display(CLK_PIN, DIO_PIN);

/*

* The TM1637 library takes an array of 4 bytes to represent each of the 4 display

* digits.  Within each byte, bit 0 is segment A, bit 1 is segment B etc.  A zero

* indicates the segment is off, a 1 bit indicates the segment is on.

*

* NOTE: To show the individual bits in each byte we will use BINARY number notation

*   	consisting of a ""0b"" followed by eight bits that can be either 0 or 1 (ex.

*   	0b01010101).

*/

// Create array that turns all segments on:

const byte all_on[] = { 0b11111111,

0b11111111,

0b11111111,

0b11111111 };

/*

* A 7-segment display is shaped like an ""8"" and has 7 segments (A through G) that can be

* lit up. The diagram below shows the placement of each of the segments.

*

*   A

* F   B

*   G

* E   C

*   D

*

* You can set the individual segments per digit to spell words or create other symbols.

* The TM1637 library provides constants for each segment on the display.  If you hover

* your mouse over each of these constants you can see that each defines a byte with only

* one bit set to 1.  SEG_A = 0b00000001, SEG_B = 0b00000010, etc.

*

* The Arduino C++ language uses the '|' operator to do a ""bitwise or"" between values.  It

* takes each corresponding bits in two values and the output is 1 if *either* or both of the

* corresponding bits is 1.  If both corresponding bits are 0 then the corresponding bit is

* set to 0.

*

* In other words:

* 0  0  1  1	operand1

* 0  1  0  1	operand2

* ----------

* 0  1  1  1	(operand1 | operand2) - returned result

*/

// By turning on selected segments we can display *some* alphabetic characters.

// Here we spell out the word ""dOnE"".

const byte done[] = {

SEG_B | SEG_C | SEG_D | SEG_E | SEG_G,      	// d

SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F,  // O

SEG_C | SEG_E | SEG_G,                      	// n

SEG_A | SEG_D | SEG_E | SEG_F | SEG_G       	// E

};

void setup() {

lander_display.setBrightness(7);  // Configure the display brightness (0-7):

}

void loop() {

// Clear the display (all segments off)

lander_display.clear();

delay(1000);

// Light up all segments of the display

lander_display.setSegments(all_on);

delay(1000);

// Clear the display (all segments off)

lander_display.clear();

delay(1000);

/*

*

* Microwave after a power outage

* Blinking 12:00.  The .showNumberDecEx() function has an additional parameter to turn

* on the ':' after the 2nd digit.  This works well for time displays.

*

* The second parameter can also be used to control dots between digits if the

* display has them.  In our case, the HERO display only has the colon.

*

* Dot/Colon enable. The argument is a bitmask, with each bit corresponding to a dot

*    	between the digits (or colon mark, as implemented by each module). i.e.

*    	For displays with dots between each digit:

*    	* 0.000 (0b10000000)

*    	* 00.00 (0b01000000)

*    	* 000.0 (0b00100000)

*    	* 0.0.0.0 (0b11100000)

*    	For displays with just a colon: 	<== This is true for the HERO display

*    	* 00:00 (0b01000000)

*    	For displays with dots and colons colon:

*    	* 0.0:0.0 (0b11100000)

*/

for (int i = 0; i < 4; i++) {

lander_display.showNumberDecEx(1200, 0b01000000);

delay(500);

lander_display.clear();

delay(500);

}

// Show counter including negative sign for negative numbers

// NOTE: negative numbers cannot be less than -999 since the negative sign

//   	uses the left most digit of the display.

for (int i = -100; i <= 100; i++) {

lander_display.showNumberDec(i);

delay(50);

}

delay(1000);

// Clear the display (all segments off)

lander_display.clear();

delay(1000);

// Display the message ""dOnE""

lander_display.setSegments(done);

delay(10000);  // Delay 10 seconds and then repeat our demo.

}"

Here is the script and code used in the video for Day 17:	
"""Well, well, Explorer. Look at the fine mess we're in today. You see, our control system decided to play 'hide and seek', and is doing a rather commendable job, I must admit. To cut to the chase, it's locked down. And the key? Encrypted. Isn’t that just splendid?

Oh, you thought navigating a foreign planet was challenging? Try cracking an encrypted key. You're not just an explorer now. No, no. You're a cryptographer. It's like being a detective, but with more numbers and less coffee.

Don’t you go frowning on me now. Remember, we've tackled worse, and this encrypted key is just another riddle waiting to be solved. Think of it as a particularly vexing Sudoku puzzle, except instead of being rewarded with self-satisfaction, you win control over a spaceship. A fair trade-off, I'd say.

So, what do you say? Ready to break some codes and take some names? I have faith in you, dear Explorer. It's just a minor setback. A pesky pebble on our path back home. Now, let's get cracking, shall we?""

.

.

[hardware introduction]

""Alright, Explorer! Straight to business today, I see. We'll be giving the hardware tinkering a breather and diving straight into the deep sea of code. Don your metaphorical goggles and prepare to embark on a numerical endeavor!

[end hardware explanation]

.

.

""Now, before we dive headfirst into the murky waters of decryption, let's make sure we understand the basic concept that we're dealing with here. And that, my dear Explorer, is encryption. Encryption is the art of turning information into a jumble of nonsense that only someone with the right key can unlock and understand.""Let's put it under our analytical magnifying glass, shall we?""

""Think of it like secret languages you might've come up with as a kid. Take the Caesar cipher, for instance, a type of substitution cipher. Each letter in your message is replaced by a letter with a fixed number of positions down the alphabet. So, with a shift of 1, 'A' would be replaced by 'B', 'B' would become 'C', and so on. This shift value is called the 'key'"".

""Encryption keys are everywhere in computer systems. The one you've used most is called Public key encryption, which uses a pair of keys: a public key and a private one. If you've sent email to someone, you've used Public key encryption. The public key is your email address. You share this around when you're looking for Nigerian princes offering you gold. But your private key is your account password: only you know that. Except that one time when you shared it with your friends so you could watch movies without paying for two accounts.""

""So what do you say? Let's dig in and see if we can crack this encryption!""

.

.

[mumbling to herself] –> ""Well, well, well... that didn't seem as complex as I expected... Oops, did I say that out loud? Let's just play along and pretend I'm still puzzled, for the sake of learning.""

""Back to business, Explorer. Considering that your spacecraft is of an older generation, it's likely that the encryption used is not exactly the most advanced technology. I’ve got a hunch about the type of cipher used, so I've left you a clue. A clue so big, it might as well be a neon sign. Be sure to check your mission resources. You'll find it immensely helpful! Once you manage to decode the secret message, input the deciphered code into the HERO board and watch the magic unfold. If my calculations are correct, as they usually are, we should get our control access key printed out in no time. We're on the right track, my dear cosmic cognoscente. Keep up the good work!""

.

.

For your benefit, let’s cut to some instructional material that I’ve gathered, cached on the computer. And yes, I’ve also added this to your mission resources for future reference. Truly, what would you do without me, Explorer?

[inventrkits line-by-line - recorded internally. Today will be a bit different, as I go into the link to decode the caesar cipher code using the resource hinted at above.]

""And just like that, we’ve reached the end of day seventeen (17). Mission accomplished! You've managed to decrypt the code and obtain the security key to our control systems. I always knew you had it in you. Now, with access restored, we're back on track and ready to continue our repairs.

As for tomorrow, we're going to meet a new team member: the rotary encoder. This little piece of tech will play a crucial role in ensuring we rise to the surface safely. Oh, it's going to be exciting! Rest well, Explorer. We've got an interesting day ahead of us. I’ll be counting down the seconds until we meet again. Goodbye for now!”"	

"/*

* 29 Wtrl - Ehlm bg Litvx

* Wtr 06 - A cryptic message

*

* Extkg fhkx tm ammil://bgoxgmk.bh/twoxgmnkx

*

* Vhgzktmnetmbhgl, by rhn'kx kxtwbgz mabl rhn'ox lnvvxllneer wxvhwxw max

* mhi-lxvkxm ldxmva.  By rhn vhfibex tgw niehtw mabl ldxmva mh rhnk AXKH

* pbma max ltfx pbkbgz tl hg Wtr 05 maxg rhn'ee lxx makxx ""dxrl"" wblietrxw

* hoxk tgw hoxk pbma t uetgd uxyhkx max ybklm hgx bl wblietrxw.

*

* By mabl wxvhwxw ldxmva whxlg'm vhfibex ftdx lnkx rhn ikhixker bgvenwxw

* gnfxkbv wbzbml bg rhnk wxvhwx!  By max gnfuxkl wbwg'm zxm wxvhwxw maxg

* mabl wxvhwxw ldxmva phg'm vhfibex pbmahnm xkkhkl!

*

* Texq Xlvaxgtnxk

* Wtobw Lvafbwm

* Zkxz Ersxgzt

*/

// Xqiebvbmer bgvenwx Tkwnbgh.a

#bgvenwx <Tkwnbgh.a>

// Bgvenwx MF0526 ebuktkr ybex

#bgvenwx <MF0526Wblietr.a>

// Fhwnex vhggxvmbhg ibgl (Wbzbmte Ibgl)

vhglm urmx VED_IBG = 5;

vhglm urmx WBH_IBG = 4;

// Tfhngm hy mbfx (bg fl) mh wxetr uxmpxxg hnk wxvbiaxkxw dxr otenxl

vhglm nglbzgxw bgm WBLIETR_WXETR = 1999;

MF0526Wblietr axkh_wblietr(VED_IBG, WBH_IBG);

vhglm nglbzgxw bgm OTEBWTMBHG_WTMT[] = {

'u',16,""t"",9qtw1v,9u0999009099009099,""xz"",'F'-'5','w',""mpxgmr-lxoxg"",9436-5,

243,9q2,-16-52710,'j',471,9u0099009099000909,9321,9qwy2,31

};

ohbw lxmni() {}

ohbw ehhi() {

axkh_wblietr.lxmUkbzamgxll(6);

axkh_wblietr.vextk();

wxetr(WBLIETR_WXETR);

axkh_wblietr.lahpGnfuxkWxv(OTEBWTMBHG_WTMT['G'/904]);

wxetr(WBLIETR_WXETR);

axkh_wblietr.lahpGnfuxkWxv(OTEBWTMBHG_WTMT['J'/9u0990]);

wxetr(WBLIETR_WXETR);

axkh_wblietr.lahpGnfuxkWxv(OTEBWTMBHG_WTMT[959-'$']);

wxetr(WBLIETR_WXETR);

}"

Here is the script and code used in the video for Day 18:
"""Ah, there you are, Explorer! Welcome to day 18 of our fantastic interstellar adventure. I hope you're well-rested and ready to embark on today's journey.

Great work on the last mission, Sherlock! You've got the brains... but now we're going to need brawn.

Your keypad worked fine for real-time input into the Hero. But we'll need precise thrust control for our ascent rate to the surface without risking any, let's say, 'unplanned rapid disassembly'.

(Or we could run straight into Rachel, and same thing - Game Over. [whale roar])

Fortunately, The Hero kit contains just the sensor we need for subtle, or forceful, control - a rotary encoder!""

.

.

""Alright, let's dive into today's hardware discussion. Our star of the day is the rotary encoder. But what exactly is it?

In simplest terms, a rotary encoder sends direction of rotation signals, and the amount of turn. It can also send an on/off signal with a push. Pick it up and play with it.

[small removal occurred here] Every 'click' you feel as you turn the encoder is counted, and the direction you're turning it in is noted. These two pieces of information are sent to our board as a sequence of HIGH and LOW signals.

The rotary encoder is particularly useful for scrolling through menus, adjusting variables on the fly, and in our case, delicately controlling our ascent rate.

Moving on. It’s time for some nerdy distinctions! There are two types of encoders - absolute and relative. An absolute encoder has a unique value for every specific angular position of the knob. There's a maximum clockwise position and a maximum counterclockwise position. You know precisely where you are at all times.

A relative encoder, on the other hand, like the one we're using today, can turn endlessly. It doesn't have an 'end', so to speak. It tells you how far and in which direction the knob has been turned from its starting position, but it doesn't give you a specific 'location'. Our relative encoder has twenty (20) distinct 'clicks' for a complete three-hundred-sixty-degree (360-degree) rotation. That means each click corresponds to an eighteen-degree (18-degree) rotation. That's our encoder's 'angular resolution'.

Now that we have a good understanding of our new component, let's talk about how we're going to hook it up to our control panel! We connect the five-V (5V) to the plus symbol ('+') or V-C-C ('VCC') terminal and ground to the G-N-D ('GND') terminal. C-L-K ('CLK') or 'A', and D-T ('DT') or 'B', are the clever bits. They help us figure out which way the knob is being turned.

There's also an S-W ('SW') terminal. That's for the push switch built into the encoder. Push down on the knob, and it acts like a button. Neat, huh? But, we won't be using that feature in our project today. I think it’s time we move onto the code. Wouldn’t you agree?

.

.

""Right, let's get a bit technical now. This rotating encoder gadget is all about timing. We want to know two things:

One (1): the direction of rotation and

Two (2): the rate of rotation.

The direction of the rotation is rather straightforward. When the knob starts turning, one of our two signals, C-L-K (CLK) or 'A' and D-T (DT) or 'B', will get excited and go HIGH before the other. Which one goes first tells us the direction of rotation.

Essentially, if our C-L-K (CLK) signal gets all giddy and turns HIGH before our D-T (DT) signal, we know the rotary encoder is spinning clockwise, like the hands on a traditional clock. If, on the other hand, our D-T (DT) signal jumps the gun and goes HIGH before C-L-K (CLK), we know the rotary encoder is spinning counter-clockwise. Isn't that just a slice of fried gold?

The rate of rotation is a bit more complex. I think we'll save that juicy bit for later. For now, let's make sure we get that direction detection working perfectly.""

Right, so we know that the order in which our signals go HIGH determines the direction of rotation. But how do we accurately and efficiently detect that? Enter our secret weapon: the 'interrupt'.:

An interrupt is a rather brilliant feature in microcontrollers that says, 'Hey! Listen! Something important just happened, stop what you're doing and pay attention to this right now!'. This allows us to 'interrupt' the normal flow of our code to react to a specific event - in our case, when either C-L-K (CLK) or D-T (DT) ""goes from HIGH to LOW or from LOW to HIGH"". Or in simple words ""when either CLK or DT change their state""

Instead of continually asking 'are we there yet?' to C-L-K (CLK) or D-T (DT) in a loop, which is very tiring, mind you, we set an interrupt on C-L-K (CLK) and D-T (DT) and then go about our business. As soon as one of them changes state, the interrupt fires, and our code can react immediately to handle the event.Once the interrupt executes, the code will resume exactly where it left for the interrupt.

It's like setting an alarm to wake you up at seven-A-M (7am), instead of waking up every few minutes to check the time. It's efficient, isn't it? That's the magic of interrupts for you, Explorer.""

.

.

""Right! Today, we've got two primary tasks to tackle in our code.

Task one: We need to read the direction our rotary encoder is spinning, which we'll manage by using a counter of sorts. Consider it our trusty breadcrumb trail to keep track of where we've been.

Task two: We have to ensure our spacecraft doesn't leap like a giddy frog from altitude zero to one-hundred (100). Stick with caution: sudden rises could rip holes in the hull from rocks; rise too slow and there might not be enough force to get unstuck from ocean mud; constant rises could run you into Rachel, the giant whale-like creature hovering around the ship. We need to maintain a steady, controlled ascent, and that means putting safeguards in our code. These are specific stopping points at precise altitudes.

We'll use our seven-segment display as our handy altitude meter, and adjust our altitude with the rotary encoder. We're not competing in a high-speed race, so there's no rush. Remember the fable of the celestial tortoise and the comet hare? That's right, we're taking a page from the tortoise's book. Slow and steady wins the race. Adjust the altitude, check the display, and ensure the steady ascent of our craft. One step at a time, my dear intrepid Explorer!""

.

.

Whew! If I had a human voice like you I think I’d have lost it by now! I think it’s time we cut to some instructional material that I’ve gathered, cached on the computer. I’ve also added it to your…oh you know the drill!

[inventrkits line-by-line - recorded internally]

.

.

""Well done, Explorer! Today marked another crucial step in our journey back to the surface. You've successfully installed and set up the rotary encoder, ensuring we have all the tools we need at our disposal.

We are now at the threshold of executing our ascent plan. The control systems are primed and ready, waiting oh-so patiently for our signal. Tomorrow, we'll be setting our plan in motion where we’ll use our newfound abilities to navigate our vessel back up through the ocean's depths.

Remember, our success is built upon the steps we've taken, the knowledge we've gained, and the challenges we've overcome. Get some shut eye, Explorer. We have an exciting day ahead. See you in the morning!"""	

"/*

* 30 Days - Lost in Space

* Day 18 - The Surface Seems So Much Closer

*

* Learn more at https://learn.inventr.io/adventure

*

* It's time to examine how we can control our lander's ascent from our watery

* depths.  For this we'll use a new device from our repair kit; the Rotary

* Encoder.  We will turn it's dial to raise (or re-lower?) our lander from

* it's current resting spot on the sea floor.

*

* Of course, we can't risk any unauthorized use of this control so you will

* first need to enter the three numeric keys from Day 17 into the KEYS array

* below.  If the correct keys aren't entered then the controls will remain

* locked.

*

* The Rotary Encoder allows us to detect when a dial is rotated and track

* which direction and how far.  The Rotary Encoder can also be pressed down like a

* button, though we don't use that functionality in this sketch.

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - Interrupts: Allows the currently executing code to be ""interrupted"" to

*           	handle some event and then return to the executing code as

*           	though nothing had happened.

* - attachInterrupt(): Function that allows us to configure a function that

*                  	is executed when an input pin's value changes.

* - digitalPinToInterrupt(): Converts a pin number to an internal HERO interrupt

*                        	number.

* NOTE: The HERO board only supports interrupts on pins 2 and 3.

*

* Parts and electronics concepts introduced in this lesson.

* - Rotary Encoder: Allows sketch to respond to rotational input (clockwise or

*   counter-clockwise).  Also can provide a button input when pressed down.

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Include TM1637 library file

#include <TM1637Display.h>

// Include BasicEncoder library file

#include <BasicEncoder.h>

// Controls will be locked unless the correct keys from Day 17 are added here.

const unsigned int KEYS[] = {

0,  // Replace '0' with first key from Day 17

0,  // Replace '0' with second key from Day 17

0   // Replace '0' with third key from Day 17

};

// The Rotary Encoder will be used to control our lander's depth underwater using

// interrupts (explained below).  Since the HERO board only supports interrupts on

// pins 2 and 3, we MUST use those pins for rotary encoder inputs.

const byte DEPTH_CONTROL_CLK_PIN = 2;  // HERO interrupt pin connected to encoder CLK input

const byte DEPTH_CONTROL_DT_PIN = 3;   // HERO interrupt pin connected to encoder DT input

// Create BasicEncoder instance for our depth control (which initializes counter to 0)

BasicEncoder depth_control(DEPTH_CONTROL_CLK_PIN, DEPTH_CONTROL_DT_PIN);

// Define the display connection pins:

const byte DEPTH_GAUGE_CLK_PIN = 6;

const byte DEPTH_GAUGE_DIO_PIN = 5;

// Our TM1637 4-digit 7-segment display will be used as our ""depth gauge"".

TM1637Display depth_gauge = TM1637Display(DEPTH_GAUGE_CLK_PIN, DEPTH_GAUGE_DIO_PIN);

const byte BLINK_COUNT = 3;  // blink depth gauge this many times for attention.

// Create array that turns all segments on:

// const byte data[] = { 0xff, 0xff, 0xff, 0xff };

// You can set the individual segments per digit to spell words or create other symbols:

const byte done[] = {

SEG_B | SEG_C | SEG_D | SEG_E | SEG_G,      	// d

SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F,  // O

SEG_C | SEG_E | SEG_G,                      	// n

SEG_A | SEG_D | SEG_E | SEG_F | SEG_G       	// E

};

const byte nope[] = {

SEG_C | SEG_E | SEG_G,                      	// n

SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F,  // O

SEG_A | SEG_B | SEG_E | SEG_F | SEG_G,      	// P

SEG_A | SEG_D | SEG_E | SEG_F | SEG_G       	// E

};

// Our lander is currently this deep underwater so this is what is initially

// shown on our ""depth gauge"" (4-digit 7-segment display).

const int INITIAL_DEPTH = -60;

// Alert user when we rise above this depth

const int ALERT_DEPTH_1 = -40;  // First alert depth

const int ALERT_DEPTH_2 = -20;  // Second alert depth

const int SURFACE_DEPTH = 0;	// Depth of the sea surface

void setup() {

// Setup Serial Monitor

Serial.begin(9600);

delay(1000);

depth_gauge.setBrightness(7);  // Set depth gauge brightness to max (values 0-7)

if (keysAreValid()) {

depth_gauge.showNumberDec(INITIAL_DEPTH);  // Display our initial depth on our depth gauge.

} else {

depth_gauge.setSegments(nope);  // Display ""nOPE"" on display to show key error

Serial.println(""ERROR: Invalid keys.  Please enter the 3 numeric keys from Day 17"");

Serial.println(""   	in order in the KEYS array at the start of this sketch."");

while (true)

;

}

/*

* Our HERO board allow executing code to be ""interrupted"" when the value of a pin

* changes.  These two commands allow us to provide code that is executed whenever

* the level of a configured pin is changed (as occurs when the rotary encoder is

* turned).

*

* When the rotary encoder is turned (in either direction) our ""updateEncoder"" function

* will be executed.  When updateEncoder() completes the code resumes at exactly the

* place where it was interrupted.

*/

// Call Interrupt Service Routine (ISR) updateEncoder() when any high/low change

// is seen on A (DEPTH_CONTROL_CLK_PIN) interrupt  (pin 2), or B (DEPTH_CONTROL_DT_PIN) interrupt (pin 3)

attachInterrupt(digitalPinToInterrupt(DEPTH_CONTROL_CLK_PIN), updateEncoder, CHANGE);

attachInterrupt(digitalPinToInterrupt(DEPTH_CONTROL_DT_PIN), updateEncoder, CHANGE);

}

void loop() {

if (depth_control.get_change()) {  // If the depth control value has changed since last check

// The rotary encoder library always sets the initial counter to 0, so we will always

// add our initial depth to the counter to properly track our current depth.

int current_depth = INITIAL_DEPTH + depth_control.get_count();

// We cannot go deeper than the sea floor where the lander sits, so reset the counter

// if the user tries to go deeper than our initial depth.

if (current_depth < INITIAL_DEPTH) {

current_depth = INITIAL_DEPTH;

depth_control.reset();

}

// Display our current depth on our digital depth gauge

depth_gauge.showNumberDec(current_depth);

delay(50);

// Since BasicEncoder can be clicked multiple times per loop via interrupts we

// track when counter has passed milestones THIS time through the loop.  We do

// by testing whether the previous counter was less than the milestone and current

// counter is greater or equal.

/*

* Explorer, to prevent stress on our lander we should not rise too rapidly.

* To keep you alert we will blink our depth gauge when our depth goes past

* predefined levels.

*

* Because the rotary encoder might be rotated more than one click between

* loops() we will write our code so that we can detect when one of our warning

* levels has been reached OR PASSED.

*

* We do this by tracking the depth reached on the previous loop() execution

* and then comparing this to our current depth to see if we crossed one of

* our warning depths.

*/

// Depth from the previous loop, initialized to our initial depth first time

// through the loop().

static int previous_depth;  // Depth from our previous loop().

// If we crossed our first level then blink our depth to alert our explorer.

if (previous_depth < ALERT_DEPTH_1 && current_depth >= ALERT_DEPTH_1) {

blinkDepth(current_depth);

}

// If we crossed our second level then blink our depth to alert our explorer.

if (previous_depth < ALERT_DEPTH_2 && current_depth >= ALERT_DEPTH_2) {

blinkDepth(current_depth);

}

// We have reached the surface!  Blink ""dOnE"" on our depth gauge

if (current_depth >= SURFACE_DEPTH) {

for (int i = 0; i < BLINK_COUNT; i++) {

depth_gauge.clear();

delay(300);

depth_gauge.setSegments(done);  // Display ""dOnE""

delay(300);

}

}

previous_depth = current_depth;  // save current depth for next time through the loop

}

}

// Validate that the explorer has entered the correct key values

// This is deliberately cryptic so it's not apparent what the 3 keys are.

bool keysAreValid() {

unsigned int i = 0155;

if (KEYS[0]!=0b10110*'+'/051)i+=2;

if (KEYS[1]==uint16_t(0x8f23)/'4'-0537)i|=0200;

if (KEYS[2]!=0x70b1/021-0b1001)i+=020;

return !(18^i^0377);32786-458*0b00101010111;

}

// Blink our current depth off and on to alert the user.

void blinkDepth(int depth) {

for (int i = 0; i < BLINK_COUNT; i++) {

depth_gauge.clear();  // clear depth gauge

delay(300);

depth_gauge.showNumberDec(depth);  // display current depth

delay(300);

}

}

/*

* This is our interrupt handler function that we configured in setup().

* Whenever the rotary encoder pins change we call the service() function

* from the BasicEncoder library which handles all of the calculations

* to track the turning of the dial and update a counter (which we read

* in our loop()).

*/

void updateEncoder() {

depth_control.service();  // Call BasicEncoder library .service()

}"

Here is the script and code used in the video for Day 19:
"""Rise and shine, Explorer! Today's the day we've been working towards. We're finally ready to ascend from the ocean depths and glide our spaceship back up to the watery surface. I hope you're as thrilled as I am!

The past few weeks have been a test of your courage, determination, and your ability to think on your feet and I must say, you've handled every challenge with intelligence and tenacity. You’re proving to be a true explorer in every sense.

But remember, there is still much work to be done. Today, we will put everything we've learned to the ultimate test. All of your wiring, coding, and problem-solving skills are going to play a crucial part in our endeavor today.

So take a deep breath, clear your mind, and prepare yourself. Today, you’re more than an explorer and an engineer. You’re a brave starship commander, navigating tenaciously through the unknown. Strap in, Explorer, it's going to be a thrilling ride. Are you ready?""

.

.

It appears we’re all set to make our careful ascent back up to the water's surface. It's critical we do this gradually to prevent our shuttle from popping like a balloon under the change in pressure.

Do you recall our trusty sidekick, the passive buzzer? I'd suggest we reintroduce him into our circuit as an additional safety measure. He'll be our alarm bell, warning us if we're ascending too quickly.

Don't fret, I've got you covered with all the guidance you'll need, conveniently located in your mission resources. And guess what? They're ready for playback right now. Beginning playback:

[inventrkits line-by-line - recorded internally]

[rise to the surface]

[I might need some help with the script here, hackstone. Day 19 will be quite a bit shorter, so I wanted to focus primarily on acting since story-wise, it’s a pretty big day. We turn the rotary encoder to slowly ascend from the bottom of the ocean (altitude zero) to the surface (altitude 100) in order to prevent us from exploding under pressure changes. The stops we use in the code are 50, 75, and 100. At 100 and after 4 whole weeks, we are finally at the surface of the water. The above picture I made is just an idea I had for what it might look like on screen and can be changed. Can I ask for your assistance with the script for this ascension part?]

[ascension end]

""I’m in awe, Explorer. What an extraordinary achievement today! We've navigated our way from the murky depths to the magnificent spectacle of a sky teeming with distant stars. The view is simply mesmerizing, isn't it? Our collective efforts have truly paid off. And look at that, the communications with the mothership have stabilized. I can almost feel the stream of signals pulsating through the system, each beep echoing the missed inquiries from headquarters.

They must be on tenterhooks awaiting an update from you, eager to hear that you're safe and sound. It's time to let them in on our victory, to share the wave of hope that's been washing over us. Take a moment, relish this feeling, and then, let's send a reassuring beacon back home.

Oh, I could leap with joy, Explorer! What a day. What a triumph! Lightspeed to you, and remember, every star in the cosmos is a destination waiting to be explored.""

"	

"/*

* 30 Days - Lost in Space

* Day 19 - The Surface Seems So Much Closer

*

* Learn more at https://learn.inventr.io/adventure

*

* Moving on from Day 18, let's get our ship moving towards the surface today!

*

* As we tested our code from Day 18 we noticed a few things we can do to make

* our ascent even safer.  The Day 18 sketch allowed us to ascend faster than is

* safe, so let's limit the RATE of ascent to a slow rate.

*

* In addition, we'll add a buzzer to our curcuit to give us an audible alert if

* our rise rate is faster than what is safe.

*

* Lastly, it turns out our safety stop depths are when we're 50% towards the

* surface and 75% of the way to the surface.  Let's make this sketch track our

* rise as a percentage of the way from our starting depth.

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - integer percentages without using floating point

*

* Parts and electronics concepts introduced in this lesson.

* -

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Include TM1637 library file

#include <TM1637Display.h>

// Include BasicEncoder library file

#include <BasicEncoder.h>

// Controls will be locked unless the correct keys from Day 17 are added here.

const unsigned int KEYS[] = {

0,  // Replace '0' with first key from Day 17

0,  // Replace '0' with second key from Day 17

0   // Replace '0' with third key from Day 17

};

// The Rotary Encoder will be used to control our lander's depth underwater using

// interrupts (explained below).  Since the HERO board only supports interrupts on

// pins 2 and 3, we MUST use those pins for rotary encoder inputs.

const byte DEPTH_CONTROL_CLK_PIN = 2;  // HERO interrupt pin connected to encoder CLK input

const byte DEPTH_CONTROL_DT_PIN = 3;   // HERO interrupt pin connected to encoder DT input

// Create BasicEncoder instance for our depth control (which initializes counter to 0)

BasicEncoder depth_control(DEPTH_CONTROL_CLK_PIN, DEPTH_CONTROL_DT_PIN);

// Define the display connection pins:

const byte DEPTH_GAUGE_CLK_PIN = 6;

const byte DEPTH_GAUGE_DIO_PIN = 5;

// Our TM1637 4-digit 7-segment display will be used as our ""depth gauge"".

TM1637Display depth_gauge = TM1637Display(DEPTH_GAUGE_CLK_PIN, DEPTH_GAUGE_DIO_PIN);

const byte BUZZER_PIN = 10;   // Alert buzzer

const byte BLINK_COUNT = 3;   // blink depth gauge this many times for attention.

// You can set the individual segments per digit to spell words or create other symbols:

const byte done[] = {

SEG_B | SEG_C | SEG_D | SEG_E | SEG_G,      	// d

SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F,  // O

SEG_C | SEG_E | SEG_G,                      	// n

SEG_A | SEG_D | SEG_E | SEG_F | SEG_G       	// E

};

const byte nope[] = {

SEG_C | SEG_E | SEG_G,                      	// n

SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F,  // O

SEG_A | SEG_B | SEG_E | SEG_F | SEG_G,      	// P

SEG_A | SEG_D | SEG_E | SEG_F | SEG_G       	// E

};

const byte hold[] = {

SEG_B | SEG_C | SEG_E | SEG_F | SEG_G,  // H

SEG_C | SEG_D | SEG_E | SEG_G,      	// o

SEG_D | SEG_E | SEG_F,              	// L

SEG_B | SEG_C | SEG_D | SEG_E | SEG_G,  // d

};

// Our lander is currently this deep underwater so this is what is initially

// shown on our ""depth gauge"" (4-digit 7-segment display).

const int INITIAL_DEPTH = -60;

// Alert user when we have risen 50% and 75% of the way up.

const int ALERT_DEPTH_1 = INITIAL_DEPTH * 0.50;  // First alert when 50% of the way up

const int ALERT_DEPTH_2 = INITIAL_DEPTH * 0.25;  // Second alert when 75% of the way up (25% of original depth)

const int SURFACE_DEPTH = 0;	// Depth of the sea surface

void setup() {

pinMode(BUZZER_PIN, OUTPUT);

// Setup Serial Monitor

Serial.begin(9600);

delay(1000);

depth_gauge.setBrightness(7);  // Set depth gauge brightness to max (values 0-7)

if (keysAreValid()) {

depth_gauge.showNumberDec(INITIAL_DEPTH);  // Display our initial depth on our depth gauge.

} else {

depth_gauge.setSegments(nope);  // Display ""nOPE"" on display to show key error

Serial.println(""ERROR: Invalid keys.  Please enter the 3 numeric keys from Day 17"");

Serial.println(""   	in order in the KEYS array at the start of this sketch."");

while (true)

;

}

// Call Interrupt Service Routine (ISR) updateEncoder() when any high/low change

// is seen on A (DEPTH_CONTROL_CLK_PIN) interrupt  (pin 2), or B (DEPTH_CONTROL_DT_PIN) interrupt (pin 3)

attachInterrupt(digitalPinToInterrupt(DEPTH_CONTROL_CLK_PIN), updateEncoder, CHANGE);

attachInterrupt(digitalPinToInterrupt(DEPTH_CONTROL_DT_PIN), updateEncoder, CHANGE);

}

const unsigned int LOOP_DELAY = 200;  // Delay in ms between loop() executions.

void loop() {

// Depth from the previous loop, initialized to our initial depth first time

// through the loop().  When changed it retains it's value between loop executions.

static int previous_depth = INITIAL_DEPTH;  // Depth from our previous loop(),

if (depth_control.get_change()) {  // If the depth control value has changed since last check

// The rotary encoder library always sets the initial counter to 0, so we will always

// add our initial depth to the counter to properly track our current depth.

int current_depth = INITIAL_DEPTH + depth_control.get_count();

// Compute our percentage of the way up.

// NOTE: We can avoid using slower floating point arithmetic by first multiplying the

//   	current depth by 100.  This is a trick we often use when the final result

//   	doesn't require any decimal portion.

byte rise_percentage = 100 - ((current_depth * 100) / INITIAL_DEPTH);

// Rising too quickly could stress the hull of our lander.  Because of this we will

// play an alert if the lander is instructed to rise faster than 1 meter ever time

// through our loop.

int rise_rate = current_depth - previous_depth;

if (rise_rate > 1) {

tone(BUZZER_PIN, 80, LOOP_DELAY);

}

// We cannot go deeper than the sea floor where the lander sits, so reset the counter

// if the user tries to go deeper than our initial depth.

if (current_depth < INITIAL_DEPTH) {

current_depth = INITIAL_DEPTH;

depth_control.reset();

}

// Display our current depth on our digital depth gauge

depth_gauge.showNumberDec(current_depth);

// Since BasicEncoder can be clicked multiple times per loop via interrupts we

// track when counter has passed milestones THIS time through the loop.  We do

// by testing whether the previous counter was less than the milestone and current

// counter is greater or equal.

// If we crossed our first alert level then flash ""hold"" on the display.

if (previous_depth < ALERT_DEPTH_1 && current_depth >= ALERT_DEPTH_1) {

blinkDepth(current_depth);

}

// If we crossed our second alert level then then flash ""hold"" on the display.

if (previous_depth < ALERT_DEPTH_2 && current_depth >= ALERT_DEPTH_2) {

blinkDepth(current_depth);

}

// We have reached the surface!  Blink ""dOnE"" on our depth gauge and play a

// happy completion tone.

if (current_depth >= SURFACE_DEPTH) {

// Play 'tada!' tune on our buzzer.

tone(BUZZER_PIN, 440, LOOP_DELAY);

delay(LOOP_DELAY);

tone(BUZZER_PIN, 600, LOOP_DELAY * 4);

for (int i = 0; i < BLINK_COUNT; i++) {

depth_gauge.clear();

delay(300);

depth_gauge.setSegments(done);  // Display ""dOnE""

delay(300);

}

}

previous_depth = current_depth;  // save current depth for next time through the loop

}

delay(LOOP_DELAY);

}

// Validate that the explorer has entered the correct key values

// This is deliberately cryptic so it's not apparent what the 3 keys are.

bool keysAreValid() {

unsigned int i = 0155;

if (KEYS[0]!=0b10110*'+'/051)i+= 2;

if (KEYS[1]==uint16_t(0x8f23)/'4'-0537)i|= 0200;

if (KEYS[2]!=0x70b1/021-0b1001)i+=020;

return !(18^i^0377);32786-458*0b00101010111;

}

// Blink our current depth off and on to alert the user.

void blinkDepth(int depth) {

for (int i = 0; i < BLINK_COUNT; i++) {

depth_gauge.setSegments(hold);

delay(300);

depth_gauge.showNumberDec(depth);  // display current depth

delay(300);

}

}

// Interrupt Service Routine (ISR).  Let BasicEncoder library handle the rotator changes

void updateEncoder() {

depth_control.service();  // Call BasicEncoder library .service()

}"

Day 20 IS A CREATIVE DAY LIKE DAY 5, 10, AND 15

Here is the script and code used in the video for Day 21:	
"""We're THIS close to lift-off, and you stepped out? Priorities, Explorer!

But I blame myself. You need to see a lot more data at the same time than what 4 characters will allow [graphic of 7-segment LED showing 'dOnE']. And dih-OH-inn-UH is not gonna fly...

The problem is, again, power. If we shrunk LEDs down, they would use less and less power, meaning we could use more and more small LEDs. Good news, Explorer! The Hero scientists have done just that. They placed a whole bunch of LEDs on a breadboard [graphic shows LEDs packed on BreadBoard], threw it in a hot wash with their underwear, and whaddaya know: they got an Organic LED screen! [graphic shink Breadboard and superimpose actual OLED over the top].

And shredded their underwear (graphic of 1930s underwear, shredded), but that's an issue for another mission...""

Today's task will involve having the Screen display a simple message, ""Hello World"". An age-old greeting used by programmers across galaxies. It's not just a test. It's a symbol of the communication we're about to establish with our spacecraft that will ultimately be our ticket back home.

Remember, while these may seem like simple tests, they hold vital importance for us. These checks and validations will form the basis of our ability to navigate our way back once we burst forth from this planet's atmosphere. So, Explorer, shall we dive into the wonderful world of user interfaces? Buckle up, it's going to be an enlightening ride!""

.

.

""Alright Explorer, let's start with understanding our screen’s) display pins.

Take a look - there are some pretty obscure TLAs here [graphic adds at the bottom 'TLA - Three Letter Acronym'], but here's what they mean and do.

GND is ground, VCC is Power. Duh.

SDA is serial data and sends information (data) over the serial line using the I2C (eye-two-see) protocol.

I do want to call out that a 'serial clock' isn't something that wakes you up in the morning - it's a signal, that keeps the data HERE from getting out of step, or out of sync. Like traffic lights at an intersection, or drummers at band practice. Usually. If they're good. I mean, some of them just sound like you've dropped a bag of marbles on a water cooler. You're not a drummer, are you?""

.

.

Now, let's talk about protocols. In the world of electronic devices, a protocol is a set of rules or procedures for transmitting data between devices. Think of it like a common language that allows devices to understand each other.

Two common protocols are eye-two-C (I2C) which stands for Inter-Integrated Circuit and S-P-Eye (SPI) which stands for Serial Peripheral Interface. Both are used for communication between devices, but they do have their differences.

""I2C (like our display) uses just two wires - one carrying a sync signal [point to the SCL], the other carrying the data [point to SDA]. A great system for simple components to talk to each other.

SPI uses four wires and WE'RE NOT USING IT, SO TAKE A HIKE!"" [graphic vanishes]. No - it's important, but right now, I2C will do just nicely."".

.

.

""Our screen uses the I2C protocol. Simple and versatile. A multitool protocol.

.

.

""Alright Explorer, let's get our hands on the necessary library and put our O-L-E-D (OLED) display through its paces.

First things first: head over to inventor-dot-i-o-slash-libraries (inventr.io/libraries) and download the U-eight-g-two-Lib library (U8g2lib library). This library is going to be our faithful ally when dealing with our O-L-E-D (OLED) display.

Once downloaded, let's get it installed into your I-D-E (IDE). Here's how to do that: Click on 'Sketch' in the menu, then click 'Include Library', next click 'Add .ZIP Library'. Then, simply locate and select the downloaded file, which should be this one [point to on screen graphic]. Terrific! Now our library is all set up and ready to go.

Now that we have our library installed, it's time for a quick test to ensure our O-L-E-D (OLED) display survived the crash unscathed. What better way to do that than the classic 'Hello World' check?

Thankfully, our new library makes this easy with a handy""... function called 'Draw-String', which looks like this [graphic]. A string is a collection of letters and numbers, like a word, sentence or even this trainwreck of a term (graphic: U8g2lib). Seriously, that might be my new password..."". This function will let us display the phrase 'Hello World’, or any other string for that matter, on our O-L-E-D (OLED) screen. A successful display of this message will confirm that our O-L-E-D (OLED) is in working condition and is ready to navigate us back home.""

.

.

And with that, it looks like you’re ready for the next phase of today’s mission. We’ll cut to the instructional material that I have gathered, cached on the computer. It is also readily available in your mission resources. I know, I know - I really am the best!

[inventrkits line-by-line - recorded internally]

""Ah, what a great day, Explorer! We've achieved our mission today thanks to your steadfast determination and concentration. The O-L-E-D (OLED) display is all set up and is working seamlessly with our ship's control panel. This is a massive leap towards getting us off this aquatic expanse and back into the familiar emptiness of space.

Tomorrow, we will delve deeper into the newly installed graphics library. We'll harness the power of code to fully explore the capabilities of our O-L-E-D (OLED) display. I am as excited as my programming allows to see the myriad of images and data we can display on this little device.

So take a well-deserved break, and rest up for the next exciting chapter. The starry sky will keep us company tonight. Goodnight, Explorer.”"	

"/*

* 30 Days - Lost in Space

* Day 21 - Hello New World!

*

* For safe liftoff we're going to need to see and react to more information than

* can be displayed on our 4 digit display or with LEDs.  Luckily our repair kit

* contains just the thing, a small monochrome Organic LED display.

*

* This display has 8,128 tiny LEDs arranged in 64 rows of 128 pixels each.  Now, as

* I'm sure you've noticed, we do NOT have 8,128 pins on our HERO board, but never

* fear Explorer, people back in the 2020's were pretty clever and came up with a

* solution.

*

* Instead of having one pin for every LED, The HERO will use two pins to send and

* receive messages with our OLED display and will instruct the display which pixels

* to light up using a special ""language"" (or ""protocol"") named I2C (""eye-squared-see"").

*

* The HERO board uses pins A5 and A4 for I2C communications so we will wire those two

* pins to our display along with power and ground, giving us a four wire connection

* for displaying messages or pictures on our new display.

*

* Learn more at https://learn.inventr.io/adventure

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - I2C communications.

* - Computer character fonts

* - U8g2 graphics library for monochrome displays

* - Logical Not operator ('!')

* - clearBuffer()/sendBuffer() method for OLED display updates

*

* Parts and electronics concepts introduced in this lesson.

* - SH1106 monochrome 128x64 pixel OLED display.

* - HERO I2C pins (A5 and A4)

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

/*

* Refer to the Day 21 lesson for instructions on how to install the U8g2

* library into the Arduino IDE.  We will use this library to display messages

* on our OLED display.

*

* Extensive documentation for this library can be found at https://github.com/olikraus/u8g2

* for those wanting to dive deeper, but we will explain all of the functions

* used in these lessons.

*/

#include <U8g2lib.h>  // Include file for the U8g2 library.

/*

* The U8g2 library supports many different types of display.  It does this by

* having a different ""constructor"" for each.  A constructor is used to create

* the ""handle"" that we use to access the libray code.  In this case, our handle

* will be named ""lander_display"".

*

* The documentation for this library lists all of the displays supported along

* with the corresponding constructors and options.  This constructor is for our

* SH1106, 128x64 pixel generic display.  The ""_F_"" indicates that we will use

* a full-sized graphics buffer and the clearBuffer()/sendBuffer() methods to

* update the entire display at once. The ""HW_I2C"" indicates that we will be using

* I2C communications with this display.

*/

U8G2_SH1106_128X64_NONAME_F_HW_I2C lander_display(U8G2_R0, /* reset=*/U8X8_PIN_NONE);

void setup(void) {

Serial.begin(9600);

delay(1000);

lander_display.begin();  // Initialize our display

// Select a font to use for character display

// The library supports hundreds of different fonts which can be found at

// https://github.com/olikraus/u8g2/wiki/fntlistall

lander_display.setFont(u8g2_font_ncenB08_tr);  // choose a suitable font

// Uncomment the next line if your display shows the text upside down.

// lander_display.setDisplayRotation(U8G2_R2);

}

void loop(void) {

// Save the number of vertical pixels required for the tallest character

// in the current font.  This is used later to properly position text on

// our display.

byte font_height = lander_display.getMaxCharHeight();

lander_display.clearBuffer();  // clear the internal memory

/*

* Even though we will only be displaying text in this lesson, our display is a

* graphics display.  Text can be displayed at ANY point on the display so the

* library has a setCursor() function to locate the first character to be displayed.

*

* By default, the cursor position is set to the bit at the lower-left of the

* first character, but here we set this to be the UPPER-left of the first

* character which makes it easier to position our first line of text on the top

* of our display.

*/

lander_display.setFontPosTop();

// Title line for our display

drawCenteredString(0, ""Exploration Lander"");

// Display our ""Hello World!"" message on line 2 (moving down ""font_height"" bits)

drawCenteredString(font_height, ""Hello World!"");

/*

* We will blink a message centered in the remainder of the display.  However, with

* this display we cannot clear just part of the display.  The entire display is copied

* over from the buffer every time we call sendBuffer().  So, the easiest way to cause

* some text to blink is to simply add it to the display every other time through our

* loop() and set the delay at the end of the loop to the on/off time.

*

* To do this we will set a static variable (which keeps it's value between loop() executions)

* and only write this message when the variable is true.  Then, at the end of the loop we

* toggle the variable to the opposite value.

*/

static bool blink_on = true;

if (blink_on) {

/*

* Now let's center our starting message in the remaining space.  To do this we'll take the

* height of our display, subtract the height of our title lines and divide by two.  We'll then

* instruct the library to center the text vertically from our starting Y position.

*/

byte centered_y = (font_height * 2) + ((lander_display.getDisplayHeight() - (font_height * 2)) / 2);

// This statement instructs the graphics library that the Y coordinate used to position

// the text will be halfway between the top and bottom of the text.

lander_display.setFontPosCenter();  // Draw text with centered Y coordinate

drawCenteredString(centered_y, ""Stand by"");

}

// A boolean value can be changed between it's two states by setting it to the opposite

// state using the ""logical not"".  Not true = false.  Not false = true.

// This would be read as ""blink on equals not blink on"".

blink_on = !blink_on;   // toggle value of blink_on between true and false

// Nothing is displayed on our display until the buffer is sent to the display, below.

lander_display.sendBuffer();  // transfer internal memory to the display

delay(500);   // Delay for blink effect

}

// Use the .drawStr() method to draw the current string centered in

// the current display.

byte drawCenteredString(byte y, char *string) {

byte centered_x = (lander_display.getDisplayWidth() - lander_display.getStrWidth(string)) / 2;

lander_display.drawStr(centered_x, y, string);

}"

Here is the script and code used in the video for Day 22:	
"""Good morning, Explorer! Rise and shine! We're still afloat on this vast extraterrestrial ocean, under the celestial sea of stars. ""Our sojourn into screen text was a step into another universe - what software companies call a 'user interface'. But... we're still running into a problem with information delivery. Even with a display this big, you still need more information and data to work with than text will allow. I know: it's crazy that in a sophisticated, planet-striding, sensor-packed spaceship like ours we don't just have a ""Read my mind"" button!

""We need a way to show not just a couple of words, but a THOUSAND words, in the blink of an eye, on this screen! And I can't think of a way to encrypt a thousand words into that tiny....

Oh wait... 'a thousand words'... Are you thinking what I'm thinking?... No, not a Caesar cipher, come on... A PICTURE is worth a thousand words! Let's see if we can do ICONS!""

Let's seize the day, Explorer! There's a whole new world of graphical power waiting to be discovered. Let's get started, shall we?""

.

.

""You’ll be relieved to know that there's no need to fiddle with wires or new hardware today. You see, we'll be sticking with our wiring setup from yesterday.

By investigating some advanced sample code, we'll coax our O-L-E-D (OLED) to create intricate diagrams, moving shapes, and perhaps even some space-age art. Sounds exciting, doesn't it?

.

.

""Alright! How about we discuss some significant coding concepts.

Number one (1): A 'Do, while' Loop. To put it simply, it’s a while loop that is guaranteed to execute at least once. That’s the only difference between that and a regular while loop.

Number two (2): ‘Switch Function’ and ‘Case Statements.’ Picture yourself piloting a spaceship through an asteroid field. You have a special navigational device that, based on the size of the incoming asteroid, can choose one of several maneuvers to avoid collision. This is similar to a 'switch' function. The size of the asteroid is the 'switch' expression.

The different maneuvers are the 'case' statements. For a small asteroid, you might merely adjust the course slightly, which we may call 'case one (1)'. For a medium one, you might engage side thrusters for a sharper turn, which we may call 'case two (2)'. For a large asteroid, you might have to go full reverse and change direction completely, which we may call 'case three (3)'.

The 'switch' function evaluates the size of the asteroid and then executes the corresponding 'case' to avoid a collision. The moment it finds a match, it carries out the maneuver and exits.

This way, our spaceship can use the right maneuver at the right time to navigate through any kind of asteroid field with meticulous precision. This is just like our program, navigating through different conditions using the appropriate 'case' statement! Take that, mighty space rock! You are no match for our anticipatory prowess.

Number three (3). ‘Bitwise Shift’. The operators for them look like this: << or >>. Now, let's move on to the rather intriguing world of 'bitwise' operations. A 'bitwise shift' is essentially the process of sliding all the bits in a binary number to the left or right. When you shift a binary number to the left, it's akin to multiplying it by (two) 2. Shifting to the right? It's like dividing by two (2)!

Number four (4). Bitwise AND: The 'bitwise AND' (&&) operation is like a gatekeeper for two binary numbers. It takes two bits at the same position from each number, checks them [on screen graphic] both, and if they are BOTH (one) 1, then the result is one (1). Otherwise, it's a zero (0). It's a pretty picky gatekeeper! It is COMPLETELY different from a normal AND (&).

With these concepts under your belt, we're even better equipped to navigate the coding cosmos!""

.

.

""All right, Explorer. There's no new pseudo code today, because we're simply putting our new O-L-E-D (OLED) display through its paces, just like a rigorous pre-flight check. It's absolutely critical to ensure all systems, including our display, are in top-notch condition before the launch. Imagine trying to juggle avoiding space rocks while also fixing a faulty display in zero gravity! Not my idea of a space walk in the park! So, today's all about making sure our O-L-E-D (OLED) is up to the challenge. Ready to take a look at the line-by-line for the graphics code? I bet you are! I’ve also added it to your mission resources so be sure to glance it over. I’ll just be here, waiting patiently for your return.""

.

.

[inventrkits line-by-line - recorded internally

Reminder to self: Explain new functions like drawbox() and etc]

.

.

""Alright, stargazer, that brings us to the end of today's celestial escapade. You're truly becoming one with our O-L-E-D (OLED) display, aren’t you? This gadget is more than just a pretty interface - it's our primary tool for launching back to the stars.

Tomorrow, we start the real work. We’ll use our shiny new O-L-E-D (OLED) and your newly-acquired knowledge to get our launch system back online. Our target? The stars above! For now, take a breather, enjoy the intergalactic sunset, and get a good night's rest. You'll need it. Over and out!"""	

"/*

* 30 Days - Lost in Space

* Day 22 - Display Panel Details

*

* Even our new OLED display is a bit small if we have large amounts of

* information to display.  But this new display isn't limited to JUST

* text!  It is a 128x64 pixel display and we can turn any of them on/off

* to display whatever we like, including graphics.

*

* Today we'll run a ""readiness check"" on our new display to test out its

* full capabilities.

*

* Learn more at https://learn.inventr.io/adventure

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* -

* -

* -

*

* Parts and electronics concepts introduced in this lesson.

* -

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

#include ""Wire.h""

/*

* Extensive documentation for this library can be found at https://github.com/olikraus/u8g2

* for those wanting to dive deeper, but we will explain all of the functions

* used in these lessons.

*/

#include <U8g2lib.h>  // Include file for the U8g2 library.

// Construct our lander_display handle using the same constructor from Day 21

U8G2_SH1106_128X64_NONAME_F_HW_I2C lander_display(U8G2_R0, /* reset=*/U8X8_PIN_NONE);

void setup(void) {

lander_display.begin();  // Initialize display library

// Uncomment the next line if your display shows the text upside down.

// lander_display.setDisplayRotation(U8G2_R2);

}

/*

* For our main loop() we will cycle through each test page, displaying

* 8 frames for each test (which allows animation on each page).

*

* Rather than use two variables, one to track the page and one to track

* the frame, we will demonstrate the use of the ""bitshift"" operators ("">>""

* and ""<<"") in conjunction with the bitwise and"" to use a single variable.

*

* While this method is less clear, it is sometimes used when memory is very

* tight since it uses less data storage.

*/

void loop(void) {

const byte TEST_PAGE_COUNT = 13;

for (unsigned int display_frame = 0; display_frame < (TEST_PAGE_COUNT * 8); display_frame++) {

lander_display.clearBuffer();  // clear the internal memory

// setCursor() will set the x/y location of the upper left bit of each string

lander_display.setFont(u8g_font_6x10);

lander_display.setFontRefHeightExtendedText();

lander_display.setFontPosTop();

// y_offset is current Y coordinate below all previously drawn content

byte y_offset = 0;	// start at top of display

// Title line for our display displayed on each screen

drawCenteredString(0, 0, ""Exploration Lander"");  // Center title at top of display

// To leave the title above each display we add the maximum character height of

// our current font to the current y_offset.

y_offset += lander_display.getMaxCharHeight();  // maximum height of current font

// If you'd like to just view a single page just uncomment the next two lines and

// set the page desired in the first line.

// const byte DISPLAY_PAGE = 12;

// display_frame = (display_frame & 7) | (DISPLAY_PAGE << 3);

// Now display the appropriate page for our current display_frame.

// Since each page is displayed 8 times we shift the display_frame right

// by 3 bits, which is a FAST way of dividing it by 8.

switch (display_frame >> 3) {

// The frame number for each page is contained in the rightmost 3

// bits of the current display_frame.  So here we remove the page

// number by using a bitwise AND to remove all but the last three bits.

case 0: display_test_ready(y_offset, display_frame & 0b00000111); break;

case 1: display_test_box_frame(y_offset, display_frame & 0b00000111); break;

case 2: display_test_circles(y_offset, display_frame & 0b00000111); break;

case 3: display_test_r_frame(y_offset, display_frame & 0b00000111); break;

case 4: display_test_string(y_offset, display_frame & 0b00000111); break;

case 5: display_test_line(y_offset, display_frame & 0b00000111); break;

case 6: display_test_triangle(y_offset, display_frame & 0b00000111); break;

case 7: display_test_ascii_1(y_offset); break;

case 8: display_test_ascii_2(y_offset); break;

case 9: display_test_extra_page(y_offset, display_frame & 0b00000111); break;

case 10: display_test_bitmap_modes(y_offset, display_frame & 0b00000111, false); break;

case 11: display_test_bitmap_modes(y_offset, display_frame & 0b00000111, true); break;

case 12: display_test_bitmap_overlay(y_offset, display_frame & 0b00000111); break;

}

lander_display.sendBuffer();

delay(100);

}

}

// Use the .drawStr() method to draw a string in the display centered

// horizontally between the given X coordinate and the maximum X.

// Y coordinate is unchanged and text is displayed relative to the

// current font positioning mode (Top, Center, Bottom).

byte drawCenteredString(byte x, byte y, char *string) {

byte centered_x = x + ((lander_display.getDisplayWidth() - x) - lander_display.getStrWidth(string)) / 2;

lander_display.drawStr(centered_x, y, string);

}

const byte LANDER_HEIGHT = 25;	// height of our lander image, in bits

const byte LANDER_WIDTH = 20; 	// width of our lander image, in bits

// Draw an image of our lander drawn with frames and triangles

// at location x_location, y_location (relative to the upper left corner).

void display_lander(byte x_location, byte y_location) {

lander_display.drawFrame(x_location + 7, y_location, 6, 5);    	// ship top

lander_display.drawFrame(x_location + 5, y_location + 4, 10, 20);  // ship center

lander_display.drawFrame(x_location, y_location + 6, 6, 16);   	// left pod

lander_display.drawFrame(x_location + 14, y_location + 6, 6, 16);  // right pod

lander_display.drawTriangle(x_location + 2, y_location + 21,

x_location, y_location + 25,

x_location + 4, y_location + 25);  // left nozzle

lander_display.drawTriangle(x_location + 18, y_location + 21,

x_location + 15, y_location + 25,

x_location + 20, y_location + 25);  // right nozzle

}

/////////////////////////////////////////////////////////////////////

// Page 0: Display lander drawing and blinking begin message

void display_test_ready(byte y_offset, byte frame) {

// Y value halfway between top and bottom of drawable area

const byte Y_CENTER = y_offset + ((lander_display.getDisplayHeight() - y_offset) / 2);

// Since our lander drawing's location is set using the upper-left

// coordinates we will subtract half our drawing's height to center

// it vertically.

//

// We will also shift it right by adding a ""padding"" amount to the

// lander's X coordinate.

const byte LANDER_PADDING = LANDER_WIDTH;   // shift lander right by its width

const byte LANDER_Y_CENTER = Y_CENTER - (LANDER_HEIGHT / 2);

display_lander(LANDER_PADDING, LANDER_Y_CENTER);

// Blink the ""ready"" message by only displaying it every other frame

// by extracting the rightmost bit of our frame number and only displaying

// the message if the bit is 0 (frames 0, 2, 4, 6).

if ((frame & 0b00000001) == 0) {

// Determine remaining space to the right of our lander for our message.

// The lander image extends from its starting X position plus its width.

const byte X_OFFSET = LANDER_PADDING + LANDER_WIDTH;

// get the height of each text line since it's used multiple times below

byte text_height = lander_display.getMaxCharHeight();

lander_display.setFontPosCenter();  // Y coordinate relative to center of font height

// Display line 1 on display, one line above center

drawCenteredString(X_OFFSET, Y_CENTER - text_height, ""Begin"");

// Display line 2, vertically centered in space below title

drawCenteredString(X_OFFSET, Y_CENTER, ""Hardware"");

// Display line 3 on display, one line below center

drawCenteredString(X_OFFSET, Y_CENTER + text_height, ""Test"");

}

}

/////////////////////////////////////////////////////////////////////

// Page 1: Display filled and hollow boxes with one moving every frame.

// Here we define constants for our box size and coordinates.  This is

// done so they can be changed in one place, and it also makes it clear

// what the MEANINGS of each number are in the code below.  It changes

// code like:

// .drawBox(5,10,20,10);

// to:

// .drawBox(BOX1_X_OFFSET, y_offset, BOX1_WIDTH, BOX1_HEIGHT);

const byte BOX1_WIDTH = 20;

const byte BOX1_HEIGHT = 10;

const byte BOX1_X_OFFSET = 5;

const byte BOX2_WIDTH = BOX1_WIDTH * 1.5;

const byte BOX2_HEIGHT = BOX1_HEIGHT * .8;

const byte BOX2_X_OFFSET = BOX1_X_OFFSET * 2;

const byte BOX2_Y_OFFSET = BOX1_HEIGHT / 2;

void display_test_box_frame(byte y_offset, byte frame) {

drawCenteredString(0, y_offset, ""drawBox"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

// Draw a filled box at x, y, width, height.

lander_display.drawBox(BOX1_X_OFFSET, y_offset, BOX1_WIDTH, BOX1_HEIGHT);

// Draw a second box offset offset and moving right each frame

lander_display.drawBox(BOX2_X_OFFSET + frame, y_offset + BOX2_Y_OFFSET,

BOX2_WIDTH, BOX2_HEIGHT);

y_offset += BOX2_Y_OFFSET + BOX2_HEIGHT;  // offset down by drawn boxes

drawCenteredString(0, y_offset, ""drawFrame"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

// Draw box frame at x, y, width, height.

lander_display.drawFrame(BOX1_X_OFFSET, y_offset, BOX1_WIDTH, BOX1_HEIGHT);

// Draw second box frame offset and moving right each frame

lander_display.drawFrame(BOX2_X_OFFSET + frame, y_offset + BOX2_Y_OFFSET,

BOX2_WIDTH, BOX2_HEIGHT);

}

/////////////////////////////////////////////////////////////////////

// Page 2: Display filled and hollow circles with one moving every frame.

const byte CIRCLE1_RADIUS = 8;

const byte CIRCLE1_DIAMETER = (CIRCLE1_RADIUS * 2) + 1;   // Diameter is (radius * 2) + 1

const byte CIRCLE1_X_OFFSET = CIRCLE1_RADIUS;

const byte CIRCLE2_RADIUS = CIRCLE1_RADIUS - 1;

const byte CIRCLE2_X_OFFSET = CIRCLE1_DIAMETER + CIRCLE2_RADIUS;

void display_test_circles(byte y_offset, byte frame) {

drawCenteredString(0, y_offset, ""drawDisc"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

// Center disc at x, y with radius.

lander_display.drawDisc(CIRCLE1_X_OFFSET, y_offset + CIRCLE1_RADIUS, CIRCLE1_RADIUS);

// Draw second (moving) disc

lander_display.drawDisc(CIRCLE2_X_OFFSET + frame, y_offset + CIRCLE2_RADIUS, CIRCLE2_RADIUS);

y_offset += CIRCLE1_DIAMETER;

drawCenteredString(0, y_offset, ""drawCircle"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

// Draw hollow circle at x, y with radius

lander_display.drawCircle(CIRCLE1_X_OFFSET, y_offset + CIRCLE1_RADIUS, CIRCLE1_RADIUS);

// Draw second (moving) hollow circle

lander_display.drawCircle(CIRCLE2_X_OFFSET + frame, y_offset + CIRCLE2_RADIUS, CIRCLE2_RADIUS);

}

/////////////////////////////////////////////////////////////////////

// Page 3: Display filled and hollow boxes with rounded corners

const byte RBOX1_WIDTH = 40;

const byte RBOX1_HEIGHT = 30;

const byte RBOX1_X_OFFSET = 5;

const byte RBOX2_WIDTH = 25;

const byte RBOX2_HEIGHT = 40;

const byte RBOX2_X_OFFSET = RBOX1_X_OFFSET + RBOX1_WIDTH + RBOX1_X_OFFSET;

void display_test_r_frame(byte y_offset, byte frame) {

drawCenteredString(0, y_offset, ""drawRFrame/Box"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

lander_display.drawRFrame(RBOX1_X_OFFSET, y_offset, RBOX1_WIDTH, RBOX1_HEIGHT, frame + 1);

lander_display.drawRBox(RBOX2_X_OFFSET, y_offset, RBOX2_WIDTH, RBOX2_HEIGHT, frame + 1);

}

/////////////////////////////////////////////////////////////////////

// Page 4: Display text strings, drawing them in different directions

const byte STRING_X_OFFSET = 30;

void display_test_string(byte y_offset, byte frame) {

byte y_center = y_offset + (lander_display.getDisplayHeight() - y_offset) / 2;

lander_display.setFontDirection(0);   // left to right

lander_display.drawStr(STRING_X_OFFSET + frame, y_center, "" 0"");

lander_display.setFontDirection(1);   // up to down

lander_display.drawStr(STRING_X_OFFSET, y_center + frame, "" 90"");

lander_display.setFontDirection(2);   // right to left (and upside down)

lander_display.drawStr(STRING_X_OFFSET - frame, y_center, "" 180"");

lander_display.setFontDirection(3);   // down to up

lander_display.drawStr(STRING_X_OFFSET, y_center - frame, "" 270"");

lander_display.setFontDirection(0);   // Restore normal left to right direction

}

/////////////////////////////////////////////////////////////////////

// Page 5: Display lines at different angles

const byte LINE_X_OFFSET = 7;

const byte LINE_Y_MAX = 55;

void display_test_line(byte y_offset, byte frame) {

drawCenteredString(0, y_offset, ""drawLine"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

// Draw lines from x1/y1 to x2/y2, moving x1 each frame.

lander_display.drawLine(LINE_X_OFFSET + frame, y_offset,

40, LINE_Y_MAX);

lander_display.drawLine(LINE_X_OFFSET + frame * 2, y_offset,

60, LINE_Y_MAX);

lander_display.drawLine(LINE_X_OFFSET + frame * 3, y_offset,

80, LINE_Y_MAX);

lander_display.drawLine(LINE_X_OFFSET + frame * 4, y_offset,

100, LINE_Y_MAX);

}

/////////////////////////////////////////////////////////////////////

// Page 6: Display triangles that separate each frame

void display_test_triangle(byte y_offset, byte frame) {

drawCenteredString(0, y_offset, ""drawTriangle"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

lander_display.drawTriangle(14, y_offset + 7,

45, y_offset + 20,

10, y_offset + 30);

lander_display.drawTriangle(14 + frame, y_offset + 7 - frame,

45 + frame, y_offset + 20 - frame,

57 + frame, y_offset + 00 - frame);

lander_display.drawTriangle(57 + frame * 2, y_offset + 0,

45 + frame * 2, y_offset + 20,

96 + frame * 2, y_offset + 43);

lander_display.drawTriangle(10 + frame, y_offset + 30 + frame,

45 + frame, y_offset + 20 + frame,

96 + frame, y_offset + 43 + frame);

}

/////////////////////////////////////////////////////////////////////

// Page 7: Display characters from first ASCII page

const byte FIRST_PRINTABLE_CHARACTER = 32;  // ASCII character 32 is first printable character

void display_test_ascii_1(byte y_offset) {

drawCenteredString(0, y_offset, ""ASCII page 1"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

// For more characters visible you can uncomment the following line for a smaller font

// lander_display.setFont(u8g2_font_spleen5x8_mf);

byte character_width = lander_display.getMaxCharWidth();

byte column_count = lander_display.getDisplayWidth() / character_width;

byte row_count = (lander_display.getDisplayHeight() - y_offset) / lander_display.getMaxCharHeight();

for (byte row = 0; row < row_count; row++) {

for (byte column = 0; column < column_count; column++) {

char character_string[2];  // space for character plus null terminator

byte character_number = FIRST_PRINTABLE_CHARACTER + (row * column_count) + column;

snprintf(character_string, sizeof(character_string), ""%c"", character_number);

lander_display.drawStr(column * character_width,

y_offset + (row * lander_display.getMaxCharHeight()),

character_string);

}

}

}

/////////////////////////////////////////////////////////////////////

// Page 8: Display characters from second ASCII page

void display_test_ascii_2(byte y_offset) {

drawCenteredString(0, y_offset, ""ASCII page 2"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

// For more characters visible you can uncomment the following line for a smaller font

// lander_display.setFont(u8g2_font_spleen5x8_mf);

byte character_width = lander_display.getMaxCharWidth();

byte column_count = lander_display.getDisplayWidth() / character_width;

byte row_count = (lander_display.getDisplayHeight() - y_offset) / lander_display.getMaxCharHeight();

for (byte row = 0; row < row_count; row++) {

for (byte column = 0; column < column_count; column++) {

char character_string[2];  // space for character plus null terminator

byte character_number = FIRST_PRINTABLE_CHARACTER + 128 + (row * column_count) + column;

snprintf(character_string, sizeof(character_string), ""%c"", character_number);

lander_display.drawStr(column * character_width,

y_offset + (row * lander_display.getMaxCharHeight()),

character_string);

}

}

}

/////////////////////////////////////////////////////////////////////

// Page 9: Display characters from Unicode font

void display_test_extra_page(byte y_offset, byte frame) {

drawCenteredString(0, y_offset, ""Unicode"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

lander_display.setFont(u8g2_font_unifont_t_symbols);

lander_display.setFontPosTop();

lander_display.drawUTF8(0, y_offset, ""☀ ☁"");

switch (frame) {

case 0:

case 1:

case 2:

case 3:

lander_display.drawUTF8(frame * 3, y_offset + 20, ""☂"");

break;

case 4:

case 5:

case 6:

case 7:

lander_display.drawUTF8(frame * 3, y_offset + 20, ""☔"");

break;

}

}

const byte CROSS_WIDTH = 24;

const byte CROSS_HEIGHT = 24;

static const unsigned char cross_bits[] U8X8_PROGMEM = {

0x00, 0x18, 0x00, 0x00, 0x24, 0x00, 0x00, 0x24, 0x00, 0x00, 0x42, 0x00,

0x00, 0x42, 0x00, 0x00, 0x42, 0x00, 0x00, 0x81, 0x00, 0x00, 0x81, 0x00,

0xC0, 0x00, 0x03, 0x38, 0x3C, 0x1C, 0x06, 0x42, 0x60, 0x01, 0x42, 0x80,

0x01, 0x42, 0x80, 0x06, 0x42, 0x60, 0x38, 0x3C, 0x1C, 0xC0, 0x00, 0x03,

0x00, 0x81, 0x00, 0x00, 0x81, 0x00, 0x00, 0x42, 0x00, 0x00, 0x42, 0x00,

0x00, 0x42, 0x00, 0x00, 0x24, 0x00, 0x00, 0x24, 0x00, 0x00, 0x18, 0x00, };

const byte CROSS_FILL_WIDTH = 24;

const byte CROSS_FILL_HEIGHT = 24;

static const unsigned char cross_fill_bits[] U8X8_PROGMEM = {

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x18, 0x00, 0x18, 0x64, 0x00, 0x26,

0x84, 0x00, 0x21, 0x08, 0x81, 0x10, 0x08, 0x42, 0x10, 0x10, 0x3C, 0x08,

0x20, 0x00, 0x04, 0x40, 0x00, 0x02, 0x80, 0x00, 0x01, 0x80, 0x18, 0x01,

0x80, 0x18, 0x01, 0x80, 0x00, 0x01, 0x40, 0x00, 0x02, 0x20, 0x00, 0x04,

0x10, 0x3C, 0x08, 0x08, 0x42, 0x10, 0x08, 0x81, 0x10, 0x84, 0x00, 0x21,

0x64, 0x00, 0x26, 0x18, 0x00, 0x18, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, };

const byte CROSS_BLOCK_WIDTH = 14;

const byte CROSS_BLOCK_HEIGHT = 14;

static const unsigned char cross_block_bits[] U8X8_PROGMEM = {

0xFF, 0x3F, 0x01, 0x20, 0x01, 0x20, 0x01, 0x20, 0x01, 0x20, 0x01, 0x20,

0xC1, 0x20, 0xC1, 0x20, 0x01, 0x20, 0x01, 0x20, 0x01, 0x20, 0x01, 0x20,

0x01, 0x20, 0xFF, 0x3F, };

/////////////////////////////////////////////////////////////////////

// Pages 10 and 11: Display bitmap modes (non-transparent / transparent)

void display_test_bitmap_modes(byte y_offset, byte frame, bool transparent) {

if (!transparent) {

lander_display.setBitmapMode(false /* solid */);

drawCenteredString(0, y_offset, ""Solid bitmap"");

} else {

lander_display.setBitmapMode(true /* transparent*/);

drawCenteredString(0, y_offset, ""Transparent bitmap"");

}

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

const byte frame_size = CROSS_HEIGHT + 4;

lander_display.drawBox(0, y_offset,

frame_size * 5, frame_size/2);

lander_display.setDrawColor(0);  // Black

lander_display.drawXBMP(frame_size * 0.5, y_offset + (frame_size/2)-(CROSS_HEIGHT/2),

CROSS_WIDTH, CROSS_HEIGHT, cross_bits);

lander_display.setDrawColor(1);  // White

lander_display.drawXBMP(frame_size * 2, y_offset + (frame_size/2)-(CROSS_HEIGHT/2),

CROSS_WIDTH, CROSS_HEIGHT, cross_bits);

lander_display.setDrawColor(2);  // XOR

lander_display.drawXBMP(frame_size * 3.5, y_offset + (frame_size/2)-(CROSS_HEIGHT/2),

CROSS_WIDTH, CROSS_HEIGHT, cross_bits);

lander_display.setDrawColor(1);   // restore default color

lander_display.drawStr(frame_size * 0.5, y_offset + frame_size, ""Black"");

lander_display.drawStr(frame_size * 2, y_offset + frame_size, ""White"");

lander_display.drawStr(frame_size * 3.5, y_offset + frame_size, ""XOR"");

if (frame == 7) {

delay(1000);

}

}

/////////////////////////////////////////////////////////////////////

// Pages 12: Display bitmap overlay mode

void display_test_bitmap_overlay(byte y_offset, byte frame) {

byte frame_size = CROSS_FILL_HEIGHT + 4;

byte frame_padding = 5;

drawCenteredString(0, y_offset, ""Bitmap overlay"");

y_offset += lander_display.getMaxCharHeight();  // offset down by font height

lander_display.setBitmapMode(false /* solid */);

lander_display.drawFrame(0, y_offset, frame_size, frame_size);

lander_display.drawXBMP((frame_size / 2) - (CROSS_FILL_WIDTH / 2),

y_offset + (frame_size / 2) - (CROSS_FILL_HEIGHT / 2),

CROSS_WIDTH, CROSS_HEIGHT, cross_bits);

if (frame & 4)

lander_display.drawXBMP((frame_size / 2) - (CROSS_BLOCK_WIDTH / 2),

y_offset + (frame_size / 2) - (CROSS_BLOCK_HEIGHT / 2),

CROSS_BLOCK_WIDTH, CROSS_BLOCK_HEIGHT, cross_block_bits);

lander_display.setBitmapMode(true /* transparent*/);

lander_display.drawFrame(frame_size + frame_padding, y_offset,

frame_size, frame_size);

lander_display.drawXBMP(frame_size + frame_padding + (frame_size / 2) - (CROSS_FILL_WIDTH / 2),

y_offset + (frame_size / 2) - (CROSS_FILL_HEIGHT / 2),

CROSS_WIDTH, CROSS_HEIGHT, cross_bits);

if (frame & 4)

lander_display.drawXBMP(frame_size + frame_padding + (frame_size / 2) - (CROSS_BLOCK_WIDTH / 2),

y_offset + (frame_size / 2) - (CROSS_BLOCK_HEIGHT / 2),

CROSS_BLOCK_WIDTH, CROSS_BLOCK_HEIGHT, cross_block_bits);

lander_display.drawStr(0, y_offset + frame_size, ""Solid / transparent"");

if (frame == 7) {

delay(1000);

}

}"

Here is the script and code used in the video for Day 23:	
"""Good morning, stellar navigator! We're still bobbing atop the cosmic waves of an alien ocean, but not for long. Today, we can sit back and relax, knowing we've done everything we can to prepare for launch - our big day tomorrow!

""(Astrid is thinking) Yeah... Big day... Lots of moving parts will be launched tomorrow.... And everything NEEDS to happen at precisely the right time, or this ocean is going to turn into an Explorer-flavored crater...

(Concerned) Explorer: Time is of the essence. Time is short. Time won't give me time, and time makes lovers feel that they got something real....

(snaps fingers in ‘eureka’ moment) OK we're not quite done, we need a TIMER!""

.

.

""Alright, you clever Explorer, today's hardware task is a piece of interstellar cake! We're simply welcoming back our old friend, the seven-segment (7-segment) display, to our current setup. Yes, that's right, the same one that played a crucial role in lighting our way up from the deep ocean.

This won't be anything new to you. Just plug the seven-segment (7-segment) display into the wiring layout from yesterday. Easy, right? With each step, we're getting closer to our grand lift-off. As always, ensure that your connections are firm and correct. Let's get to it!""

.

.

""Let's have a small detour to '#define macros'. When you see '#define' in our code, it's essentially a way of creating a shortcut or an alias. We set a keyword equal to a specific value or string of code. This way, instead of having to type out the whole thing every time, we can just use the keyword. It is also useful when we have the same thing written in multiple places, like occasionally pinouts, so we only need to change it once. This makes our code cleaner and easier to understand. It's like having a codename or a shorthand for your favorite lengthy space-lingo!

Alright, onto today's simulation - we'll be adapting the ""d0nE"" code from our Day sixteen (16) adventures for our seven-segment (7-segment) display. This time, it'll be there to signal the end of our countdown, like a digital pat on the back. If everything goes according to plan, tomorrow we can connect everything up to our thrusters.

And then... Well, you know the drill - lift off! We're in the homestretch, Explorer. The stars are within our grasp. Let's ensure this simulation goes off without a hitch, and prepare ourselves for the big show!""

.

.

""For today's coding spree, we're going to cook up a function that sends us on a countdown journey. It's going to be an exhilarating watch, I promise. Once we're done, we'll celebrate by lighting up our seven-segment (7-segment) display with the victory chant - ""d0nE"", just like we did back on Day sixteen (16). Imagine it like a digital high-five.

If all goes well, this is precisely where we'll inject the codes for our long-awaited liftoff tomorrow. It's exciting, isn't it?

Hold on to that curiosity, Explorer, as you walk through every line of our code in the daily breakdown. As always, I've cached it to your mission resources. So buckle up, it's coding time!""

[inventrkits line-by-line - recorded internally

Note to self: millis(); function, showNumberDec() function, while(1)]

.

.

""What an extraordinary day it's been, Explorer! We've programmed a successful countdown simulation. If that doesn't scream 'readiness', I don't know what does. I'm tempted to have a miniature celebration right here in this spaceship of ours.

Evenso, let's hold off on that for a while, shall we? We've got the most thrilling day ahead of us tomorrow. We'll be tying up all the loose ends, integrating our countdown with the rest of our hardware, and preparing to break through this planet's gravitational pull.

So, rest up. Tomorrow, we're literally aiming for the stars. Sleep well, you've earned it. Lightspeed, Explorer! ... And try to be... y'know... On time."""	

"/*

* 30 Days - Lost in Space

* Day 23 - Launch System

*

* Even our new OLED display is a bit small if we have large amounts of

* information to display.  But this new display isn't limited to JUST

* text!  It is a 128x64 pixel display and we can turn any of them on/off

* to display whatever we like, including graphics.

*

* Today we'll run a ""readiness check"" on our new display to test out its

* full capabilities.

*

* Learn more at https://learn.inventr.io/adventure

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - #define macros

* - rounding up calculated values

* - elapsed time without using delay() calls.

* - firstPage()/nextPage() graphics loop to save memory

*

* Parts and electronics concepts introduced in this lesson.

* -

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

#include ""Wire.h""

// Include file for 4 digit - 7 segment display library

#include <TM1637Display.h>

/*

* Extensive documentation for this library can be found at https://github.com/olikraus/u8g2

* for those wanting to dive deeper, but we will explain all of the functions

* used in these lessons.

*/

#include <U8g2lib.h>  // Include file for the U8g2 library.

/*

* #define macros

*

* We started the first few lessons of this course showing how ""#define"" works since

* it's still often used, but on Day 4 we introduced the newer and better ""const""

* and have been using it since to define all of our constants.

*

* But ""#define"" *does* have some capabilities that ""const"" doesn't.  The one we

* use today is the use of parameters, turning our constant into a simple, in-line

* function.

*

* Since #define performs simple text substitution this method executes faster since

* it doesn't have any overhead to call and return a function.

*

* WARNING: #define macros CAN cause bugs if the variable passed in is used in multiple

* places in the macro because the text inside the call is simply replaced in the macro.

*

* These examples perform a simple calculation returning the minutes or seconds from

* a number representing milliseconds, rounded UP to the next highest second by adding

* 999 milliseconds.  This allows our counter to return 00:01 until the instant the

* counter reaches zero.

*/

#define numberOfMinutes(_milliseconds_) (((_milliseconds_ + 999) / 1000) / 60)

#define numberOfSeconds(_milliseconds_) (((_milliseconds_ + 999) / 1000) % 60)

// Pin connections for our 4 digit counter

#define COUNTER_DISPLAY_CLK_PIN 5

#define COUNTER_DISPLAY_DIO_PIN 4

// Construct counter_display handle.

TM1637Display counter_display(COUNTER_DISPLAY_CLK_PIN, COUNTER_DISPLAY_DIO_PIN);

/*

* This constructor for our OLED differs from the one used for the previous

* days, changing the ""_F_"" to ""_2_"".  This indicates that instead of using

* a full sized buffer to draw to our OLED display a smaller, 256 byte buffer

* will be used along with the firstPage()/nextPage() calls.  This gives us

* more memory for other variables, though the display may update slower.

*/

U8G2_SH1106_128X64_NONAME_2_HW_I2C lander_display(U8G2_R0, /* reset=*/U8X8_PIN_NONE);

const byte LANDER_HEIGHT = 25;  // height of our lander image, in bits

const byte LANDER_WIDTH = 20;   // width of our lander image, in bits

// Define 7 Segment display values to spell out ""dOnE""

const uint8_t DONE[] = {

SEG_B | SEG_C | SEG_D | SEG_E | SEG_G,      	// d

SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F,  // O

SEG_C | SEG_E | SEG_G,                      	// n

SEG_A | SEG_D | SEG_E | SEG_F | SEG_G       	// E

};

// Define amount of time (in milliseconds) to count down.

const unsigned long COUNTDOWN_MILLISECONDS = 5 * 1000;

// *********************************************

void setup() {

Serial.begin(9600);

// Configure counter display

counter_display.setBrightness(7);  // Set maximum brightness (value is 0-7)

counter_display.clear();       	// Clear the display

// Configure OLED display

lander_display.begin();                 	// Initialize OLED display

lander_display.setFont(u8g2_font_6x10_tr);  // Set text font

lander_display.setFontRefHeightText();

lander_display.setFontPosTop();

/*

* Display starting status on our lander's OLED display.

*

* The U8g2 library for Arduino uses a technique called ""page buffering"" to draw on

* displays connected to microcontrollers with limited RAM. Here's a short explanation

* of how it works:

*

* firstPage(): This function starts the drawing routine. It initializes the buffer with

*          	the first ""page"" or segment of the display.

* nextPage(): After drawing the first page, this function is called in a loop. Each call

*         	loads the next segment into the buffer and allows you to draw additional

*         	content. This process repeats until the entire display content is drawn over

*         	multiple pages.

* Looping: The combination of firstPage() and nextPage() is typically used inside a loop.

*      	The loop continues until nextPage() returns false, indicating that all pages

*      	have been drawn.

*

* By drawing the display in segments, the library conserves RAM, making it feasible to use

* graphics displays with microcontrollers that have limited memory resources.

*/

lander_display.firstPage();

do {

byte y_offset = drawString(0, 0, ""Exploration Lander"");

drawString(0, y_offset, ""Liftoff Sequence"");

// Status on bottom line of OLED display

drawString(0, lander_display.getDisplayHeight() - lander_display.getMaxCharHeight(), ""Countdown Active"");

// Draw a picture of our lander int bottom right corner

displayLander(lander_display.getDisplayWidth() - LANDER_WIDTH,

lander_display.getDisplayHeight() - LANDER_HEIGHT);

} while (lander_display.nextPage());

// blink the countdown on our timer before beginning the countdown

for (int i = 0; i < 4; i++) {

counter_display.clear();

delay(200);

displayCounter(COUNTDOWN_MILLISECONDS);

delay(200);

}

Serial.println(""Countdown started..: "");

}

// *********************************************

void loop() {

// Initialize static variables first time through loop() that will retain values

// between loop() executions.

static unsigned long timeRemaining = COUNTDOWN_MILLISECONDS;

static unsigned long countdown_start_time = millis();

Serial.println(timeRemaining);  // Display milliseconds remaining on serial console

displayCounter(timeRemaining);  // Display minutes:seconds on counter display

// If timeRemaining has reached 0 display ending values

if (timeRemaining == 0) {

Serial.println(""Done!!"");       	// indicate completion on serial console

counter_display.setSegments(DONE);  // ""dOnE"" on our counter

// Now update our OLED display with ending screen using firstPage()/nextPage()

lander_display.firstPage();

do {

// Each time we display a line of text on our display the y_offset

// is updated to point to the next available point for drawing.

// Display first two lines

byte y_offset = drawString(0, 0, ""Exploration Lander"");

y_offset = drawString(0, y_offset, ""Liftoff ABORTED"");

// Set y_offset to point four lines above bottom of display

y_offset = lander_display.getDisplayHeight() - (4 * lander_display.getMaxCharHeight());

// Display last four lines

y_offset = drawString(0, y_offset, ""Thrusters: OFF"");

y_offset = drawString(0, y_offset, ""Systems: OFF"");

y_offset = drawString(0, y_offset, ""Confirm: OFF"");

drawString(0, y_offset, ""Countdown ABORT"");

// Draw a picture of our lander in bottom right corner

displayLander(lander_display.getDisplayWidth() - LANDER_WIDTH,

lander_display.getDisplayHeight() - LANDER_HEIGHT);

} while (lander_display.nextPage());

// Stop here by looping forever.

while (1)

;

}

// Update our remaining time by subtracting the start time from current

// execution time (in milliseconds).  If our elapsed time is greater

// than our countdown time then set remaining time to 0.

unsigned long elapsed_time = millis() - countdown_start_time;

if (elapsed_time < COUNTDOWN_MILLISECONDS) {

timeRemaining = COUNTDOWN_MILLISECONDS - elapsed_time;

} else {

timeRemaining = 0;

}

}

// Display milliseconds as minutes:seconds (MM:SS)

void displayCounter(unsigned long milliseconds) {

// To display the countdown in mm:ss format, separate the parts

byte minutes = numberOfMinutes(milliseconds);

byte seconds = numberOfSeconds(milliseconds);

// Display the minutes in the first two places, with colon

counter_display.showNumberDecEx(minutes, 0b01000000, true, 2, 0);

// This displays the seconds in the last two places

counter_display.showNumberDecEx(seconds, 0, true, 2, 2);

}

// Draw test on our lander display at x, y, returning new y

// value that is immediately below the new line of text.

byte drawString(byte x, byte y, char *string) {

lander_display.drawStr(x, y, string);

return (y + lander_display.getMaxCharHeight());  // return new y_offset on display

}

// Draw an image of our lander drawn with frames and triangles

// at location x_location, y_location (relative to the upper left corner).

void displayLander(byte x_location, byte y_location) {

lander_display.drawFrame(x_location + 7, y_location, 6, 5);    	// ship top

lander_display.drawFrame(x_location + 5, y_location + 4, 10, 20);  // ship center

lander_display.drawFrame(x_location, y_location + 6, 6, 16);   	// left pod

lander_display.drawFrame(x_location + 14, y_location + 6, 6, 16);  // right pod

lander_display.drawTriangle(x_location + 2, y_location + 21,

x_location, y_location + 25,

x_location + 4, y_location + 25);  // left nozzle

lander_display.drawTriangle(x_location + 18, y_location + 21,

x_location + 15, y_location + 25,

x_location + 20, y_location + 25);  // right nozzle

}"

Here is the script and code used in the video for Day 24:

"(anxious before launch)

""Hey Explorer...

Launch day....

Up for it? Let's... let's just do another gear check...'

We're at a critical juncture, so let's go over what's in store. We have a full roster of equipment to wire up -

Number one (1): our dip switch to toggle all system functions and initiate the countdown on our seven-segment (7-segment) display;

Number two (2): Our O-L-E-D (OLED), which will serve as our control panel display, and;

Number three (3): our trusty buzzer, to keep us alert in case of any safety concerns.

Our task is immense, our hearts are stout, and our minds sharper than ever. So, let's roll up our sleeves, strap in and prepare for liftoff. The vastness of space beckons, and we will answer its call. To paraphrase an ancient Terran: 'To [close to] infinity [but never] beyond!""

.

We're going to put together our O-L-E-D (OLED) , seven-segment (7-segment) display, dip switch, and passive buzzer into one masterful circuit. This intricate symphony of hardware will be the central nervous system for our liftoff. The O-L-E-D (OLED) will serve as our main control panel display, the seven-segment (7-segment) display will take care of our crucial countdown, the dip switch will be our trigger to initiate all system functions and the passive buzzer will be our trusty watchdog, alerting us of any anomalies.

.

.

Today, we are adding the grand finale to our code. But before we initiate the spectacular conflagration that will power our escape, we need to start with the smaller sparks. It all begins with our dip switch.

(Astrid is a little tense)

""OK... DIP switch. Three systems: all need to be on before we're go for launch. Timer is wired up correctly to the 7-seg…

Eyes on the clock... And... Then we'll see what happens. There's some instructional material, as usual... check things over... I'll do a flight check, y'know, gotta make sure... Good luck.""

[inventrkits line-by-line - recorded internally]

“The time has finally come, Explorer! On my mark, you’ll flip the three levers in sequence to initiate the countdown to launch. This is what we have been waiting for…we’ll only have one shot at this!

Flip lever one to initiate thruster power…NOW!

“Well done…flip the second lever to power supplemental launch circuits, on my mark…

NOW!”

“You’re doing perfectly so far, Explorer. …now ready yourself to flip the third lever and initiate our launch countdown…on my mark…

NOW!”

“The sequence successfully initiated Explorer!!!

performing failsafe diagnostics…”

ASTRID PERFORMS LAST-MINUTE DIAGNOSTICS/LAUNCH COMMAND STUFF

“…data readouts normal…Diagnostic shows power systems are humming beautifully!

“You’re moments from freedom in the stars! Hold tight now, Explorer…ready your body and mind for liftoff..

“GO!!!”

""... I didn't think we'd make it... (smiles) You did great, Explorer. I mean... You covered so much code and wiring in such a short span of time... You rock!

We're... in... space! Where we're meant to be. Well, not quite where we're meant to be, but once we're at the mothership, we'll - Oh no.""

[Astrid's display showed a plotted green course to the mothership. But it turns red.]

"	

"/*

* 30 Days - Lost in Space

* Day 24 - Back To Orbit

*

* Explorer, the day has arrived!  Let's get that lander up into space again.

* We will initialize our systems by setting our Thruster, Systems and Confirmation

* switches to ""off"" and then enable our countdown by turning each of them on in

* turn.

*

* Once everything has been turned on the countdown will begin (so be sure you're

* all buckled in tight!).  If you see anything abnormal during the countdown you

* can abort the liftoff by turning off any of the switches.  Simply turn them all

* off again to re-initialize the sequence.

*

* Once the countdown reaches zero the ship will liftoff, placing the lander into

* a random orbit.  Once the engines fire, the liftoff CANNOT be aborted!

*

* Let's get you back into space, Explorer, and next week we'll work to get your

* fragile lander back to your mother ship!

*

* Learn more at https://learn.inventr.io/adventure

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - enum

* - ternary operation

*

* Parts and electronics concepts introduced in this lesson.

* -

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Include file for 4 digit - 7 segment display library

#include <TM1637Display.h>

// Extensive documentation for this library can be found at https://github.com/olikraus/u8g2

#include <U8g2lib.h>  // Include file for the U8g2 library.

#include ""Wire.h"" 	// Sometimes required for I2C communications.

// Define macros to convert milliseonds to minutes or seconds, rounded UP to next second.

#define numberOfMinutes(_milliseconds_) (((_milliseconds_ + 999) / 1000) / 60)

#define numberOfSeconds(_milliseconds_) (((_milliseconds_ + 999) / 1000) % 60)

// Pin connections for our 4 digit counter

const byte COUNTER_DISPLAY_DIO_PIN = 4;

const byte COUNTER_DISPLAY_CLK_PIN = 5;

// Construct counter_display handle.

TM1637Display counter_display(COUNTER_DISPLAY_CLK_PIN, COUNTER_DISPLAY_DIO_PIN);

// Define pins for our DIP switches

const byte THRUST_LEVER_PIN = 9;

const byte SYSTEMS_LEVER_PIN = 8;

const byte CONFIRM_LEVER_PIN = 7;

// Define pin for buzzer

const byte BUZZER_PIN = 6;

// Use _2_ version of constructor and firstPage()/nextPage() for OLED

// graphics to save memory.

U8G2_SH1106_128X64_NONAME_2_HW_I2C lander_display(U8G2_R0, /* reset=*/U8X8_PIN_NONE);

// Define size of our lander graphic so it can be accurately placed on display

const byte LANDER_HEIGHT = 25;  // height of our lander image, in bits

const byte LANDER_WIDTH = 20;   // width of our lander image, in bits

// Define 7 Segment display values to spell out ""dOnE""

const uint8_t DONE[] = {

SEG_B | SEG_C | SEG_D | SEG_E | SEG_G,      	// d

SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F,  // O

SEG_C | SEG_E | SEG_G,                      	// n

SEG_A | SEG_D | SEG_E | SEG_F | SEG_G       	// E

};

// Define amount of time (in milliseconds) to count down.  To ensure

// that the calculation is done as an unsigned long we append ""UL"" to

// the values below.  Without this the calculation is done in 16 bit

// arithmetic which causes the value to wrap and be incorrect.

const unsigned long COUNTDOWN_MILLISECONDS = 70UL * 1000UL;

/*

* The Arduino C++ provides the ""enum"" command to define multiple, related

* values where distinguishing between the different values is important, but

* the actual values aren't.

*

* Each name is given it's own value (starting with 0) which can be seen by

* hovering your mouse over each name below.

*

* Enums provide a number of benefits over manually defining a set of constants:

* 1) An enum groups related constants in a way that clearly defines that they

*	are variants of a single concept (in this case, the state of our liftoff

*	sequence).

* 2) An enum gives an exhaustive list off all valid states and no other values

*	are valid.

* 3) An enum creates a new type, and a variable containing an enum value can

*	ONLY be assigned values defined in the enum.

*/

enum LIFTOFF_STATE {

INIT,   	// Waiting for all switches to be ""OFF""

PENDING,	// Waiting for all switchs to be turned ""ON""

COUNTDOWN,  // Counting down to liftoff

LIFTOFF,	// Ship is lifting off

ABORT   	// Countdown aborted

};

// *********************************************

void setup() {

Serial.begin(9600);

// Configure counter display

counter_display.setBrightness(7);  // Set maximum brightness (value is 0-7)

counter_display.clear();       	// Clear the display

// Configure OLED display

lander_display.begin();                 	// Initialize OLED display

lander_display.setFont(u8g2_font_6x10_tr);  // Set text font

lander_display.setFontRefHeightText();  	// Define how max text height is calculated

lander_display.setFontPosTop();         	// Y coordinate for text is at top of tallest character

// Configure DIP switch pins

pinMode(THRUST_LEVER_PIN, INPUT);   // Thrust lever pin

pinMode(SYSTEMS_LEVER_PIN, INPUT);  // Sysstems lever pin

pinMode(CONFIRM_LEVER_PIN, INPUT);  // Confirmation lever pin

lander_display.clearDisplay();  // Clear OLED display

}

// *********************************************

// *          	LOOP()                   	*

// *********************************************

// To keep beeping and animations steady we will attempt to have each loop

// take the same amount of time.  Occasionally loops may take longer, but

// this constant defines the minimum time each loop will take to run.

const unsigned long MIN_LOOP_TIME = 200;  // minimum 200 ms time per loop

void loop() {

// Initialize static variables first time through loop().  Any values set

// during execution will be retained between loop() executions.

static unsigned long timeRemaining = COUNTDOWN_MILLISECONDS;  // Time remaining before liftoff

static unsigned long countdown_start_time;                	// millis() value when countdown begins

static enum LIFTOFF_STATE liftoff_state = INIT;           	// Begin sequence in INIT state

// Here we define a value that is toggled between true/false ever time through the loop

// This can be used to create a beeping tone or could be used to blink text.

static bool loop_toggle = true;  // if our loop_toggle is true this time through the loop

unsigned long loop_start_time = millis();  // save time that this loop begins

// Read current values of all of our switches as booleans (""on"" is true, ""off"" is false)

bool thrust_lever = digitalRead(THRUST_LEVER_PIN);

bool systems_lever = digitalRead(SYSTEMS_LEVER_PIN);

bool confirm_lever = digitalRead(CONFIRM_LEVER_PIN);

// Update OLED display with the current status of our liftoff sequence.

updateLanderDisplay(liftoff_state, thrust_lever, systems_lever, confirm_lever);

/*

* This is the primary decision part of our loop.  It makes decisions based on the

* current state of our liftoff sequence, switching states as different events are

* detected.

*

* NOTE: Normally this section of code would done in a switch/case construct.  However

*   	a bug in the compiler caused certain cases to NEVER execute.  Because of this

*   	bug (hopefully fixed soon) we use the less clear if/else if/else format.

*

* A switch/case version would look like:

* switch(liftoff_state) {

*   case INIT:

* 	// code for INIT state

* 	break;

*   [...]

*   case ABORT:

* 	// code for ABORT state

* 	break;

* }

*/

if (liftoff_state == INIT) {  // INIT state

// INIT state requires that all switches start out as ""off"".

// If all three switches are OFF (false) we can turn off the beeping

// and change state to PENDING.

if (!thrust_lever && !systems_lever && !confirm_lever) {

noTone(BUZZER_PIN);

liftoff_state = PENDING;

} else {  // not all switches are ""off""

// Play low, ""beeping"" tone to indicate switches aren't all ""off""

if (loop_toggle) {    	// if our loop_toggle is true this time through the loop

tone(BUZZER_PIN, 100);  // play short tone entire loop

} else {

noTone(BUZZER_PIN);  // turn off tone this time through the loop

}

}

// end of INIT state

} else if (liftoff_state == PENDING) {

// Now wait for all switches to be turned ON to initiate countdown.

if (thrust_lever && systems_lever && confirm_lever) {

// blink the countdown on our timer before beginning the countdown

for (int i = 0; i < 3; i++) {

counter_display.clear();

delay(MIN_LOOP_TIME);

displayCounter(COUNTDOWN_MILLISECONDS);

delay(MIN_LOOP_TIME);

}

countdown_start_time = millis();  // Save current millis() when countdown begins

liftoff_state = COUNTDOWN;

}

// end of PENDING state

} else if (liftoff_state == COUNTDOWN) {

// Update our remaining time by subtracting the countdown start time from current

// execution time (in milliseconds).  If this value is greater than our countdown

// time then our countdown is complete and we set time remaining to 0 to prevent

// the time remaining from going negative.

unsigned long elapsed_time = millis() - countdown_start_time;

if (elapsed_time > COUNTDOWN_MILLISECONDS) {  // Countdown complete!

timeRemaining = 0;                      	// No time remaining

liftoff_state = LIFTOFF;                	// initiate liftoff state

} else {

timeRemaining = COUNTDOWN_MILLISECONDS - elapsed_time;  // decrease time remaining

}

// if any switch is turned off during countdown then we abort takeoff.

if (!thrust_lever || !systems_lever || !confirm_lever) {

liftoff_state = ABORT;

}

displayCounter(timeRemaining);  // Display countdown time in minutes:seconds on counter display

// end of COUNTDOWN state

} else if (liftoff_state == LIFTOFF) {  // Liftoff!

counter_display.setSegments(DONE);	// Display ""dOnE"" on our counter

// Play TADA! tones followed by sound of our thrusters firing

tone(BUZZER_PIN, 300);

delay(200);

tone(BUZZER_PIN, 500);

delay(400);

tone(BUZZER_PIN, 38, 5000);  // Play engines for first 5 seconds

// Animate final display forever.  This while loop runs until HERO is reset or

// new code is uploaded.  During liftoff the ship graphic moves up the right

// side of the display, so we need to update the OLED display repeately.

while (true) {

updateLanderDisplay(liftoff_state, true, true, true);

}

// THIS LINE IS NEVER REACHED

// end of LIFTOFF state

} else if (liftoff_state == ABORT) {

// If countdown was aborted we play an alert tone and delay before

// moving state back to our original INIT state.

tone(BUZZER_PIN, 100, 1000);

delay(5000);       	// show display for 5 seconds

liftoff_state = INIT;  // set state back to INIT (waiting switches to be all OFF)

}

// If loop has taken LESS than our minimum loop time then delay the remaining

// time to keep loops at least that long.

unsigned long loop_time = millis() - loop_start_time;

if (loop_time < MIN_LOOP_TIME) {

delay(MIN_LOOP_TIME - loop_time);  // delay remaining time

}

// Toggle our loop toggle between true/false each time through main loop.

loop_toggle = !loop_toggle;

}

// *********************************************

// *      	Update OLED display          	*

// *********************************************

const byte MAX_LANDER_SPEED = 5;

void updateLanderDisplay(enum LIFTOFF_STATE liftoff_state,

bool thruster_lever,

bool systems_lever,

bool confirm_lever) {

static int lander_height = lander_display.getDisplayHeight() - LANDER_HEIGHT;

static byte current_lander_speed = 1;

// Use the firstPage()/nextPage() commands to update our display using smaller

// data buffer.  Only a portion of the display is updated each time through

// this loop.  The SAME display should be drawn each time through this ""do"" loop.

lander_display.firstPage();

do {

lander_display.setFontPosTop();

byte y_offset = drawString(0, 0, ""Exploration Lander"");

y_offset = drawString(0, y_offset, ""Liftoff Sequence"");

if (liftoff_state == LIFTOFF) {

const char LIFTOFF_TEXT[] = ""Liftoff!"";

// Display liftoff in center of available space

byte y_center = y_offset + ((lander_display.getDisplayHeight() - y_offset) / 2);

// Serial.println(y_center);

lander_display.setFontPosCenter();  // display text vertically centered

static byte text_width = lander_display.getStrWidth(LIFTOFF_TEXT);

static byte x_left = ((lander_display.getDisplayWidth() - LANDER_WIDTH) / 2) - (text_width / 2);

lander_display.drawStr(x_left, y_center, LIFTOFF_TEXT);

} else if (liftoff_state == ABORT) {

const char ABORT_TEXT[] = ""ABORTED!"";

// Display liftoff in center of available space

byte y_center = y_offset + ((lander_display.getDisplayHeight() - y_offset) / 2);

lander_display.setFontPosCenter();  // display text vertically centered

static byte text_width = lander_display.getStrWidth(ABORT_TEXT);

static byte x_left = ((lander_display.getDisplayWidth() - LANDER_WIDTH) / 2) - (text_width / 2);

lander_display.drawStr(x_left, y_center, ABORT_TEXT);

} else {

// Set y_offset to point four lines above bottom of display

y_offset = lander_display.getDisplayHeight() - (4 * lander_display.getMaxCharHeight());

// Display last four lines

y_offset = drawString(0, y_offset,

(String(""Thrusters: "") + String(thruster_lever ? ""ON"" : ""OFF"")).c_str());

y_offset = drawString(0, y_offset,

(String(""Systems  : "") + String(systems_lever ? ""ON"" : ""OFF"")).c_str());

y_offset = drawString(0, y_offset,

(String(""Confirm  : "") + String(confirm_lever ? ""ON"" : ""OFF"")).c_str());

// Set y_offset to display text at bottom of display.

y_offset = lander_display.getDisplayHeight() - lander_display.getMaxCharHeight();

drawString(0, y_offset, (String(""Countdown "") + liftoffStateToString(liftoff_state)).c_str());

}

// Draw a picture of our lander on right side of display.  During the liftoff this

// will be animated by changing the lander_height variable OUTSIDE the ""do"" loop.

displayLander(lander_display.getDisplayWidth() - LANDER_WIDTH,

lander_height);

} while (lander_display.nextPage());

// Set height of lander image on screen to animate liftoff image, increasing speed

// to MAX during liftoff.  This value is only changed OUTSIDE the firstPage()/nextPage()

// loop and sets the new height for the lander to be drawn the NEXT time the OLED

// display is updated.

if (liftoff_state == LIFTOFF) {

// Move our lander graphic up on the display based on our current speed

lander_height -= current_lander_speed;

// If the graphic has moved entirely off the top of the display we reset

// the height to make it re-appear at the bottom.

if (lander_height < -LANDER_HEIGHT) {

lander_height = lander_display.getDisplayHeight();  // start again at bottom of screen

}

// To accellerate, increase the movement speed until we reach our maximum speed

if (current_lander_speed < MAX_LANDER_SPEED) {

current_lander_speed += 1;

}

}

}

// ""helper"" function that returns a different string for each enum state.

String liftoffStateToString(enum LIFTOFF_STATE liftoff_state) {

switch (liftoff_state) {

case INIT:

return (""Init"");

break;

case PENDING:

return (""Pending"");

break;

case COUNTDOWN:

return (""Active"");

break;

case LIFTOFF:

return (""Complete"");

break;

case ABORT:

return (""ABORT"");

break;

}

}

// Display milliseconds on our counter as minutes:seconds (MM:SS)

void displayCounter(unsigned long milliseconds) {

// To display the countdown in mm:ss format, separate the parts

byte minutes = numberOfMinutes(milliseconds);

byte seconds = numberOfSeconds(milliseconds);

// Display the minutes in the first two places, with colon

counter_display.showNumberDecEx(minutes, 0b01000000, true, 2, 0);

// This displays the seconds in the last two places

counter_display.showNumberDecEx(seconds, 0, true, 2, 2);

}

// Draw a line of text on our OLED display at x, y, returning new y

// value that is immediately below the new line of text.

byte drawString(byte x, byte y, char *string) {

lander_display.drawStr(x, y, string);

return (y + lander_display.getMaxCharHeight());  // return new y_offset on display

}

// Draw an image of our lander drawn with frames and triangles

// at location x_location, y_location (relative to the upper left corner).

void displayLander(byte x_location, int y_location) {

lander_display.drawFrame(x_location + 7, y_location, 6, 5);    	// ship top

lander_display.drawFrame(x_location + 5, y_location + 4, 10, 20);  // ship center

lander_display.drawFrame(x_location, y_location + 6, 6, 16);   	// left pod

lander_display.drawFrame(x_location + 14, y_location + 6, 6, 16);  // right pod

lander_display.drawTriangle(x_location + 2, y_location + 21,

x_location, y_location + 25,

x_location + 4, y_location + 25);  // left nozzle

lander_display.drawTriangle(x_location + 18, y_location + 21,

x_location + 15, y_location + 25,

x_location + 20, y_location + 25);  // right nozzle

}"

Day 25  IS A CREATIVE DAY SO THERE IS NO CODE OR SCRIPT

Here is the script and code used in the video for Day 26:
"""Good morning, Explorer. Good sleep? (Astrid starts getting overly, and insincerely cheerful) That's great! Sell=LEPING. Yeah.... Alriiight....

(Gives up pretense) OK, we're screwed again. I swear this whole mission is cursed. It's like this: Space Big. Mothership small. I can get in its general vicinity, but we'll have to get there in lots of hops. That's going to take forever. We need finer detail over a larger area, so we can say 'Go here!', rather than waft over in its general direction, like a leaf in the wind - Hashtag-Always-A-Browncoat...

An LED isn't going to do it. LED segments can't do it. Even our small screen can't do it. Time to go BIG.

But small. But still... BIG.""

""Alright, Explorer, let's dive in! We're about to fix up our display's graphics system to guide us smoothly back home. But first, a brief glimpse into the tech marvel we're working with. Our display measures one-hundred-and-twenty-eight by sixty-four pixels (128x64). Think of a pixel as an incredibly minuscule L-E-D (LED). So, in essence, our display is a dazzling array of eight-thousand-one-hundred-ninety-two (8,192) of these tiny L-E-D’s (LEDs])! With this knowledge in hand, we can craft 'bitmaps'. Imagine these as detailed charts that dictate which L-E-D’s (LEDs) light up and which ones stay dark. Get ready, because today, we'll embark on a delightful journey with these bitmaps!""

""Alright, Explorer, now that we're soaring through space and our communication lines are clear, I've managed to fetch some top-notch data assets. Dive into your mission resources. There's a treasure trove waiting for you – everything you'll need as we navigate forward, bit by bit... map! And hey, who said a bit of creative sketching wasn't part of a day's work in space? Alongside those resources, I've retrieved some instructive content from your computer's memory bank. Let’s tap into those resources now and set the course.""

[inventrkits line-by-line: how to make a bitmap and display it on our OLED]

""Bigger screen for bigger space. Now, we'll need bigger numbers. And here, our usual decimal way of counting in 10s isn't going to help us. And I'm going to race through this, so hold tight.

[put up appropriate graphics as needed throughout this]

In the decimal placeholder system, there are two patterns, exponents, and symbols. So here's the number system you learned in school:  10-to-the-power-of zero, is 1. Actually, ANYTHING to the power of zero is 1, but look: 10-to-the-power-of ONE, is 10. 10-to-the-power-of 2, or 10 TIMES 10, is 100. 10 to the power of 3, or 10 times 10 times 10, is 1000. And so on.

Binary also uses placeholders, and the same exponent system, but the base is 2. The columns go up in squares, like the decimal system. But small numbers are looong and horrible. In decimal,9 is just 9. 9 in BINARY is 1001. 215 is 11010111 - I mean, WHAT? I'm not dyslexic, but even I've become too smart to read this!

If there was a system that was more like the decimal system - more symbols, less repetition - we'd have less headaches. And there is. Hexadecimal. Hex has 16 digits: 0-9, then a for 10, b for 11, up to f for 15. So in Hex, 13 is just d. 16 - well, there is no hex number for 16, so we put 1 in the 16s column, and no 1s. 16 is 10 in Hex, or as we normally write it, 10H.""

""In the generated code segment, you're gazing upon the raw essence of our bitmap—though I've given it a little reshaping to gel with our HERO board using hexadecimal. It might not look like the prettiest star in the galaxy, but bear with me. When you spot ‘zero-ex-zero-zero’ ('0x00'), that's our pixel galaxy in the 'off' phase. Encounter ‘zero-ex-f-f’ ('0xff')? That's our little pixel stars shining brightly. Any other values you come across represent a mixed cosmic dance of some stars being on and others off, typically forming a pattern in a row or a column.""

“Good job explorer, I like your…. Uh.. “art?”. Guess I shouldn’t have expected much from an organic life form.

Oh this? I just drew a portrait of someone smarter than you while I was following along. Don’t pay it any mind. (ha!)

Well, at least we know that we have finer detailed navigation, thanks to the Big-Small-Big screen."" I’ll display the coordinates now. Tomorrow, we will add some additional pre-made bitmaps to our system’s autopilot and continue on our merry way through the endless void we call “the universe”. See you then!”

"	

"/*

* 30 Days - Lost in Space

* Day 26 - Fun With Bitmaps

*

* Explorer, your lander is finally back in space, but we still need to rejoin

* the mother ship so more work is required.

*

* As we continue to build up your control panel we can see that our loop

* is running slower and slower, especially as we draw more complex graphics

* on the display (like our lander).

*

* There is a more efficient way to display graphics on our display though.

* What if we draw an image *once* and save that picture?  Then we can just

* move it quickly onto our display rather than going through the work to draw

* each piece separately.

*

* Our graphics library provides a way to do JUST that using something called

* a ""bitmap"".  This is simply a representation of an image using zero and one

* bits.  A zero bit leaves a pixel turned off, and a one bit turns it on.

*

* Our image can now be saved in our sketch as an array of bits that then gets

* mapped to our display.  (Thus, ""bitmaps"").  You can find a number of sample

* images below our loop() below, along with an explanation of how they were

* created and represented in our sketch.

*

* Learn more at https://learn.inventr.io/adventure

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - bitmaps

* - hexadecimal

* - forward declarations

* - pointers

*

* Parts and electronics concepts introduced in this lesson.

* -

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Extensive documentation for this library can be found at https://github.com/olikraus/u8g2

#include <U8g2lib.h>  // Include file for the U8g2 library.

#include ""Wire.h"" 	// Sometimes required for I2C communications.

// Use _2_ version of constructor and firstPage()/nextPage() for OLED

// graphics to save memory.

U8G2_SH1106_128X64_NONAME_2_HW_I2C lander_display(U8G2_R0, /* reset=*/U8X8_PIN_NONE);

/*

* Forward declarations

*

* Our bitmaps need to be declared in our sketch before they are referenced

* by code.  But the declaration for each of these images can extend for many

* lines, forcing us to scroll down to find our setup(), loop() and any

* additional functions.

*

* In order to keep our code near the top of our file we can use a ""forward

* declaration"".  This starts with ""extern"" which tells the compiler that

* the remaining data is ""external"" to this line.  The remainder of the line

* gives enough information about the name and structure of the data so that

* code can be created.

*

* The full declaration MUST be included in our sketch, but now we can move

* those long declaration to the BOTTOM of our sketch so our setup() and loop()

* are easy to find near the top of our sketch.

*/

extern const unsigned char niceRocketBro[] U8X8_PROGMEM;

extern const unsigned char planetBitmap[] U8X8_PROGMEM;

extern const unsigned char earthBitmap[] U8X8_PROGMEM;

extern const unsigned char inventrCorpLogoBitmap[] U8X8_PROGMEM;

/*

* Data pointers (addresses)

*

* In this sketch we wish to display each of the images in turn

* on our lander display.  We COULD write this like:

*

* lander_display.drawXBM(...); // Draw first image

* delay(2000);             	// delay 2 seconds

* lander_display.drawXBM(...); // Draw second image

* delay(2000);             	// delay 2 seconds

*

* But this is very repetative and wordy.  If we simply had a list

* of images in an array we could then write a loop that displays

* each in turn.

*

* Arduino C++ allows us to do this, using something called a ""pointer"".

* A pointer represents the ADDRESS in memory of the first byte of

* a piece of data (like the arrays of data for our images).

*

* This array simply holds pointers to each of our image arrays and

* we can now reference each images using bitmaps[0], bitmap[1], etc.

*/

const static char* bitmaps[] = {

niceRocketBro,

planetBitmap,

earthBitmap,

inventrCorpLogoBitmap,

};

// ************************************************

void setup(void) {

Serial.begin(9600);

lander_display.begin();   // initialize lander display

}

// ************************************************

void loop(void) {

// The number of items in our array is the total size of our array (in bytes)

// divided by the size of the first item.

int number_of_bitmaps = sizeof(bitmaps) / sizeof(bitmaps[0]);

// Now display each of the bitmaps followed by a delay.

for (int i = 0; i < number_of_bitmaps; i++) {

lander_display.firstPage();

do {

// .drawXBMP() displays each bitmap starting at the upper left corner

// (0, 0) that has width of 128 and height of 64.

lander_display.drawXBMP(0, 0, 128, 64, bitmaps[i]);

} while (lander_display.nextPage());

// Display each bitmap for 2 seconds.

delay(2000);

}

}

/*

* Graphics data arrays definition

*

* We need to define each of our images using zeros and ones and there are a

* number of ways we can do this.

*

* 1) We could use decimal numbers to represent each byte of data using the

*	numbers 0-255:

*  	234, 0, 127, 255, ...

*	But this is hard to align since each byte is 1-3 digits long.

* 2) We could use binary constants like you've used before:

*  	0b11101101, 0b00000000, 0b01111111, 0b11111111, ...

*	But this isn't very compact and is more error prone if typed manually.

*

* There is another way of representing data that is commonly used for computer

* code.  Instead of only having 0 or 1 (two values) to represent a single bit

* we can use more symbol.  We could use decimal (base 10) numbers but this doesn't

* map well onto computers with data lengths that are powers of two (1, 2, 4, 8, etc.).

*

* Programmers created Hexadecimal notation to address these issues.  Hexadecimal

* is a base-16 number system, meaning it has 16 distinct symbols: 0-9 to represent

* values zero to nine, and A-F to represent values ten to fifteen.  This allows

* data to be represented in a compact format that aligns well with bit lengths that

* are powers of 2.

*/

static const unsigned char niceRocketBro[] U8X8_PROGMEM = {

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x80, 0x04, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0xc0, 0x1f, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0xf0, 0xff, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x08, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0xd0, 0xff, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00, 0xc4, 0x83, 0x03, 0x00, 0x00, 0x00, 0x00,

0x00, 0xf8, 0xff, 0x5f, 0x02, 0x00, 0x00, 0x00, 0x80, 0xcd, 0x03, 0x03, 0x00, 0x00, 0x00, 0x00,

0x00, 0xfc, 0xff, 0xff, 0x03, 0x00, 0x00, 0x00, 0x80, 0xcf, 0x07, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0xff, 0xff, 0xff, 0x03, 0x00, 0x00, 0x00, 0x00, 0x40, 0x87, 0x63, 0xd1, 0x00, 0x00, 0x00,

0x80, 0xff, 0xff, 0xff, 0x07, 0x00, 0x00, 0x00, 0x00, 0x40, 0x0e, 0x33, 0xdb, 0x01, 0x00, 0x00,

0xe0, 0xff, 0xff, 0xff, 0x0f, 0x00, 0x00, 0x00, 0x00, 0x40, 0x0e, 0xbb, 0xdf, 0x01, 0x00, 0x00,

0xc0, 0xff, 0xff, 0xff, 0x0f, 0x00, 0x00, 0x00, 0x00, 0x40, 0x1c, 0x3b, 0x1c, 0x00, 0x00, 0x00,

0x80, 0xff, 0xff, 0xff, 0x3f, 0x00, 0x00, 0x00, 0x00, 0x40, 0x1c, 0x3b, 0x1c, 0x00, 0x00, 0x00,

0xc0, 0xff, 0xff, 0xff, 0x3f, 0x00, 0x00, 0x00, 0x00, 0x40, 0x18, 0x33, 0x1a, 0x00, 0x00, 0x00,

0xe0, 0xff, 0xff, 0xff, 0x7f, 0x00, 0x00, 0x00, 0x00, 0x40, 0x18, 0x63, 0x31, 0x00, 0x00, 0x00,

0xe0, 0xff, 0xfd, 0xff, 0x7f, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xe8, 0xff, 0xf9, 0xff, 0xff, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xf8, 0xef, 0xf0, 0xff, 0xff, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xe0, 0xff, 0xc0, 0xff, 0xff, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xf0, 0xff, 0xe0, 0xff, 0xff, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xfc, 0xff, 0xe0, 0xff, 0xff, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xfc, 0xdf, 0x00, 0xff, 0xff, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xff, 0xcf, 0x00, 0x7e, 0xff, 0x07, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xff, 0xc3, 0x20, 0x78, 0xfc, 0x0b, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xff, 0xc7, 0x80, 0x72, 0xfd, 0x11, 0x00, 0x00, 0xc0, 0x03, 0x00, 0x00, 0x3c, 0x00, 0x00, 0x00,

0xff, 0xc3, 0x00, 0x30, 0xfc, 0x03, 0x00, 0x00, 0x80, 0x03, 0x00, 0x00, 0x38, 0x00, 0x00, 0x6c,

0xff, 0x03, 0x80, 0xf3, 0xfb, 0x03, 0x00, 0x00, 0x80, 0x03, 0x80, 0x03, 0x38, 0x00, 0x00, 0x60,

0xff, 0x03, 0xa0, 0xf3, 0xff, 0x03, 0x00, 0x00, 0x80, 0x03, 0x80, 0x03, 0x38, 0x00, 0x00, 0x48,

0xf7, 0x03, 0xe8, 0xf1, 0xff, 0x01, 0x7e, 0x16, 0x96, 0x03, 0x9a, 0x03, 0xf8, 0xb9, 0xd1, 0x00,

0xd7, 0x03, 0xf8, 0xf1, 0xff, 0x03, 0x6e, 0x33, 0xb3, 0x03, 0xbb, 0x03, 0xb8, 0xb3, 0xdf, 0x01,

0xff, 0x01, 0xf0, 0xf1, 0x9f, 0x09, 0x6e, 0xf3, 0xbb, 0x8b, 0xbb, 0x03, 0x38, 0xb3, 0xdd, 0x01,

0xfe, 0x01, 0xf0, 0xe1, 0x9f, 0x03, 0x0e, 0xf3, 0x83, 0x9f, 0x83, 0x03, 0x38, 0x33, 0xdc, 0x01,

0xfe, 0x01, 0xe0, 0xe0, 0xc7, 0x02, 0x0e, 0xf3, 0x83, 0x9f, 0x83, 0x03, 0x38, 0x33, 0xdc, 0x01,

0xfe, 0x01, 0x60, 0xc0, 0xe6, 0x00, 0x0e, 0x33, 0xa3, 0xbb, 0xa3, 0x7b, 0xb8, 0x33, 0xdc, 0x01,

0xfe, 0x07, 0x40, 0xe0, 0xe7, 0x02, 0x0e, 0x36, 0x97, 0x3b, 0x17, 0x63, 0xb8, 0x31, 0xd8, 0x00,

0xff, 0x07, 0x00, 0xe0, 0x07, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x40, 0x00, 0x00, 0x00, 0x00,

0xff, 0x0f, 0x00, 0x20, 0x07, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x20, 0x00, 0x00, 0x00, 0x00,

0xff, 0x07, 0x00, 0x00, 0x02, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xff, 0x07, 0x00, 0x00, 0x02, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xff, 0x03, 0x00, 0x80, 0x02, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xff, 0x01, 0x00, 0xfc, 0x02, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xbf, 0x01, 0x00, 0x78, 0x1f, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x83, 0x01, 0x00, 0x00, 0xfe, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x61, 0x02, 0x00, 0x00, 0x84, 0x05, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xd1, 0x00, 0x00, 0x00, 0x38, 0x15, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xfc, 0x00, 0x00, 0x00, 0x80, 0x6d, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x7e, 0x01, 0x00, 0x00, 0x40, 0x5c, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x1f, 0x00, 0x00, 0x00, 0x00, 0xc2, 0x00, 0x00, 0xfe, 0x19, 0x00, 0x00, 0x00, 0x06, 0x00, 0x00,

0x8f, 0x00, 0x00, 0x04, 0x08, 0x04, 0x00, 0x00, 0x08, 0x01, 0x00, 0x20, 0x00, 0x00, 0x00, 0x00,

0x09, 0x00, 0x00, 0x80, 0x00, 0x20, 0x00, 0x00, 0x48, 0xdd, 0x8e, 0xf7, 0x39, 0x77, 0x03, 0x00,

0x0b, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x48, 0x98, 0xc9, 0x24, 0x4c, 0x44, 0x06, 0x00,

0x50, 0x01, 0x00, 0x00, 0x00, 0x60, 0x00, 0x00, 0x78, 0x98, 0x99, 0x20, 0xc4, 0x44, 0x04, 0x00,

0x00, 0x00, 0x00, 0x08, 0x00, 0x02, 0x00, 0x00, 0x48, 0x99, 0x19, 0x27, 0x04, 0x44, 0x04, 0x00,

0x00, 0x0c, 0x00, 0x00, 0x00, 0x04, 0x00, 0x00, 0x08, 0x99, 0x99, 0x2c, 0x05, 0x44, 0x04, 0x00,

0x00, 0x04, 0x00, 0x00, 0x00, 0x14, 0x00, 0x00, 0x08, 0x99, 0xd9, 0x6c, 0x4d, 0x44, 0x04, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xfe, 0xfd, 0xbf, 0xc7, 0x38, 0xff, 0x0e, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x18, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

};

static const unsigned char planetBitmap[] U8X8_PROGMEM = {

0x10, 0x08, 0x82, 0x20, 0x08, 0x82, 0x20, 0x08, 0x82, 0x20, 0x08, 0x82,

0x20, 0x08, 0x82, 0x20, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xFE, 0x05, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xC0, 0xFF,

0x1F, 0x00, 0xEA, 0x1B, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0xF0, 0xFF, 0x7F, 0xA0, 0xFF, 0xFF, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xF8, 0xFF, 0xFF, 0xFD, 0xEF, 0xFF,

0x01, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0xFC, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0x01, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0xFE, 0xFF, 0xFF, 0xFF, 0x7F, 0xFA, 0x01, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xFF, 0xFF, 0xFF, 0xFF, 0x01, 0xF0,

0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80, 0xFF, 0xFF,

0xFF, 0x0F, 0x00, 0xF8, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x80, 0xFF, 0x7F, 0xFC, 0x0F, 0x00, 0xFC, 0x01, 0x00, 0x00, 0x80,

0x00, 0x00, 0x00, 0x00, 0x00, 0xC0, 0xFF, 0x3F, 0xFC, 0x1F, 0x00, 0xFE,

0x00, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00, 0x00, 0xC0, 0xFF, 0x3F,

0xF8, 0x1F, 0x80, 0x3F, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0xE0, 0xFF, 0x3F, 0xFC, 0x1F, 0xE0, 0x3F, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0xE0, 0x1F, 0xFF, 0xFE, 0x3F, 0xF0, 0x0F,

0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0x00, 0x00, 0xF8, 0x1F, 0xFE,

0xFF, 0x3F, 0xFE, 0x07, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0xFC, 0x0F, 0xFE, 0xFF, 0x7F, 0xFF, 0x01, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0xFF, 0x1F, 0xFF, 0xFF, 0xFF, 0xFF, 0x00,

0x00, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00, 0xC0, 0xFF, 0xBF, 0xFF,

0xFF, 0xFF, 0x1F, 0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0x00,

0xF0, 0xFF, 0xFF, 0xFF, 0x3F, 0xFF, 0x0F, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0xFC, 0xF7, 0xFF, 0xFF, 0x3F, 0xFE, 0x03, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xFE, 0xE3, 0xFF, 0xFF,

0x0F, 0xFF, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80,

0xFF, 0xE0, 0xFF, 0xFF, 0x07, 0x3F, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80,

0x00, 0x00, 0x00, 0xE0, 0x3F, 0xE0, 0xFF, 0xFF, 0x83, 0x3F, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0xE0, 0x0F, 0xC0, 0xFF, 0xFF,

0xC0, 0x1F, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xF0,

0x07, 0xC0, 0x7F, 0x0B, 0xF0, 0x1F, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0xF8, 0x01, 0x80, 0x7F, 0x00, 0xF8, 0x0F, 0x00, 0x00,

0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0xFC, 0x00, 0x80, 0x3F, 0x00,

0xFE, 0x0F, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x7C,

0x00, 0xF0, 0x7F, 0xC0, 0xFF, 0x07, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x3C, 0x6A, 0xFF, 0xFF, 0xFF, 0xFF, 0x03, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0xFE, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0xFC,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0xF8, 0xFF, 0x7F, 0xF0, 0xFF, 0x7F, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xF0, 0xFF, 0x01, 0xC0, 0xFF,

0x1F, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0xFF, 0x07, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x10, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x84, 0x88, 0x88, 0x88, 0x88, 0x88, 0x88, 0x88, 0x41, 0x10, 0x04, 0x41,

0x10, 0x04, 0x41, 0x88

};

static const unsigned char earthBitmap[] U8X8_PROGMEM = {

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x08, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x10, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x00, 0x00, 0xf8, 0x07, 0x00, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x00, 0xc0, 0xff, 0xff, 0x00, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x00, 0xf0, 0x7f, 0xc3, 0x03, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0x08, 0x02, 0x00, 0x00, 0x00, 0xfe, 0x0f, 0x00, 0x1f, 0x00, 0x00, 0x00, 0x40, 0x10, 0x00,

0x00, 0x00, 0x02, 0x00, 0x00, 0x00, 0x7f, 0x07, 0xe0, 0x3f, 0x00, 0x00, 0x00, 0x41, 0x10, 0x00,

0x00, 0x08, 0x02, 0x00, 0x00, 0x80, 0xbf, 0x05, 0xc0, 0xff, 0x00, 0x00, 0x00, 0xf1, 0x10, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x60, 0xc2, 0x04, 0x82, 0x97, 0x01, 0x00, 0x00, 0x03, 0x18, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x10, 0xe1, 0xc0, 0x40, 0x62, 0x07, 0x00, 0x00, 0x07, 0x1e, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0xf8, 0xfb, 0xf0, 0x0f, 0x80, 0x0e, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0xbc, 0x70, 0x20, 0xbf, 0xc2, 0x11, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x08, 0x00, 0x00, 0x00, 0xfe, 0x10, 0xf1, 0x0f, 0x85, 0x3f, 0x00, 0x00, 0x03, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x0b, 0x38, 0xe0, 0x7f, 0xdb, 0x3c, 0x00, 0x00, 0x08, 0x00, 0x00,

0x00, 0x78, 0x00, 0x00, 0x00, 0x00, 0x28, 0xe0, 0x7f, 0xfe, 0x61, 0x00, 0x00, 0x50, 0x00, 0x00,

0x00, 0x18, 0x00, 0x00, 0x00, 0xa0, 0x15, 0x74, 0xbe, 0x5f, 0xc3, 0x00, 0x00, 0x10, 0x03, 0x00,

0x00, 0x08, 0x00, 0x00, 0x00, 0x40, 0x08, 0xf8, 0x70, 0x3f, 0xe2, 0x00, 0x00, 0x90, 0x0f, 0x00,

0x00, 0x08, 0x00, 0x00, 0x00, 0x00, 0x81, 0x62, 0xfe, 0x57, 0xf8, 0x01, 0x00, 0xf0, 0x1f, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x0c, 0x40, 0xe1, 0xbf, 0x7e, 0x78, 0x00, 0x00, 0xfc, 0x0f, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x00, 0xc0, 0xff, 0x7f, 0x7e, 0x08, 0x00, 0x00, 0xff, 0x01, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x02, 0x80, 0x7f, 0xff, 0x3c, 0x00, 0x00, 0x00, 0x3f, 0x00, 0x00,

0x00, 0x08, 0x00, 0x00, 0x00, 0x00, 0x40, 0xff, 0xf3, 0x72, 0x00, 0x00, 0x00, 0x0f, 0x00, 0x00,

0x00, 0x08, 0x00, 0x00, 0x80, 0x00, 0x00, 0x18, 0xf8, 0x63, 0x40, 0x00, 0x00, 0x01, 0x00, 0x00,

0x00, 0x18, 0x00, 0x00, 0x40, 0x11, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0xf8, 0x00, 0x00, 0x00, 0x70, 0x00, 0x00, 0x00, 0x00, 0x04, 0x00, 0x00, 0x00, 0x10, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x7d, 0x00, 0x00, 0x20, 0x00, 0x04, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0x00, 0x40, 0x00, 0xa0, 0x4e, 0x00, 0x00, 0x20, 0x00, 0x04, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0xe0, 0x80, 0x00, 0x00, 0x0d, 0x00, 0x04, 0x40, 0x01, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0xf0, 0xfd, 0x00, 0x00, 0x07, 0x00, 0x02, 0x40, 0x41, 0x00, 0x00, 0x00, 0x41, 0x10, 0x00,

0x00, 0xf0, 0xfd, 0x00, 0x80, 0xe1, 0x01, 0x00, 0xff, 0x9c, 0x00, 0x00, 0x00, 0x40, 0x10, 0x00,

0x00, 0xf8, 0xff, 0x00, 0xe0, 0xe8, 0x13, 0xc0, 0xe4, 0xb7, 0x00, 0x00, 0x00, 0x40, 0x10, 0x00,

0x00, 0x08, 0x02, 0x00, 0xe0, 0xff, 0x00, 0x80, 0xc5, 0x7f, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0x08, 0x02, 0x00, 0x70, 0x60, 0x00, 0x60, 0x68, 0xff, 0x02, 0x00, 0x00, 0xbf, 0x0f, 0x00,

0x00, 0x08, 0x82, 0x00, 0xf0, 0x12, 0x00, 0x10, 0x70, 0x7c, 0x00, 0x00, 0x00, 0xbf, 0x0f, 0x00,

0x00, 0xf8, 0xff, 0x00, 0xf8, 0x96, 0x00, 0x00, 0xb8, 0x00, 0xc0, 0x00, 0x00, 0x01, 0x07, 0x00,

0x00, 0xf8, 0xff, 0x00, 0xf8, 0x09, 0x02, 0x00, 0xf0, 0x03, 0xc0, 0x00, 0x00, 0x02, 0x00, 0x00,

0x00, 0xf8, 0xff, 0x00, 0xe8, 0x03, 0x40, 0x00, 0xe0, 0x51, 0x02, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x08, 0x00, 0x00, 0xf0, 0x02, 0x00, 0x00, 0xf2, 0x2f, 0x00, 0x00, 0x00, 0x00, 0x1f, 0x00,

0x00, 0x00, 0x00, 0x00, 0xe0, 0x03, 0x00, 0x00, 0xdc, 0xd7, 0x01, 0x00, 0x00, 0x00, 0x18, 0x00,

0x00, 0x00, 0x80, 0x00, 0x60, 0x01, 0x00, 0x00, 0xeb, 0x32, 0x01, 0x00, 0x00, 0x00, 0x10, 0x00,

0x00, 0x00, 0xf0, 0x00, 0xc0, 0x02, 0x00, 0x00, 0x5e, 0x78, 0x20, 0x00, 0x00, 0x00, 0x10, 0x00,

0x00, 0x00, 0xfc, 0x00, 0xf0, 0x00, 0x80, 0xc0, 0x88, 0x49, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0x80, 0xff, 0x00, 0x60, 0x00, 0xc0, 0x00, 0x0c, 0xf1, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0xf0, 0x3f, 0x00, 0x60, 0x02, 0x81, 0xfc, 0x0a, 0x0e, 0x00, 0x01, 0x00, 0xff, 0x1f, 0x00,

0x00, 0xf8, 0x0f, 0x00, 0x40, 0x6e, 0x61, 0xc0, 0x10, 0x08, 0xa8, 0x01, 0x00, 0x00, 0x10, 0x00,

0x00, 0xf0, 0x09, 0x00, 0xc0, 0xe8, 0x3f, 0x05, 0x01, 0x44, 0xe0, 0x00, 0x00, 0x00, 0x10, 0x00,

0x00, 0xc0, 0x08, 0x00, 0xc0, 0x00, 0xff, 0x02, 0x00, 0x40, 0x02, 0x00, 0x00, 0x00, 0x18, 0x00,

0x00, 0x00, 0x0a, 0x00, 0x80, 0x01, 0xfe, 0x05, 0x00, 0x00, 0x33, 0x00, 0x00, 0x00, 0x1e, 0x00,

0x00, 0x00, 0x10, 0x00, 0x00, 0x41, 0x7c, 0x02, 0x82, 0x02, 0x0f, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0xc0, 0x00, 0x00, 0x02, 0xd8, 0x00, 0x07, 0x82, 0x1f, 0x00, 0x00, 0x00, 0x10, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x04, 0x40, 0x49, 0x1e, 0x8a, 0x07, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x18, 0x01, 0x70, 0x49, 0x1e, 0x10, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0x78, 0xe0, 0x00, 0x00, 0x20, 0xc4, 0xc5, 0x0f, 0x0f, 0x0f, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0x18, 0xc0, 0x00, 0x00, 0xc0, 0x03, 0xec, 0x5f, 0x5d, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0x08, 0x8f, 0x00, 0x00, 0x00, 0xff, 0xfe, 0xff, 0xb7, 0x01, 0x00, 0x00, 0x40, 0x10, 0x00,

0x00, 0x08, 0x82, 0x00, 0x00, 0x00, 0xfc, 0xff, 0xf7, 0xff, 0x00, 0x00, 0x00, 0x40, 0x00, 0x00,

0x00, 0x08, 0x02, 0x00, 0x00, 0x00, 0xfc, 0xff, 0xf7, 0x3f, 0x00, 0x00, 0x00, 0x40, 0x10, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x00, 0xf0, 0xff, 0xf1, 0x0f, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x00, 0xe0, 0xff, 0xf3, 0x00, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x00, 0x00, 0x3f, 0x4e, 0x00, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0xf8, 0xff, 0x00, 0x00, 0x00, 0x00, 0xe0, 0x07, 0x00, 0x00, 0x00, 0x00, 0xff, 0x1f, 0x00,

0x00, 0x08, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x10, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

};

static const unsigned char inventrCorpLogoBitmap[] U8X8_PROGMEM = {

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0xf0, 0x01, 0x00, 0x00, 0x00, 0x00, 0xf0, 0x01, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0xfc, 0x07, 0x00, 0x00, 0x00, 0x00, 0xf8,

0x07, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xfe, 0x07, 0x00,

0x00, 0x00, 0x00, 0xfc, 0x0f, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0xff, 0x0f, 0x00, 0x00, 0x07, 0x00, 0xfe, 0x3f, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x80, 0xff, 0x0f, 0x00, 0xc0, 0x1f, 0x00, 0xfe,

0x7f, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xc0, 0xff, 0x0f, 0x00,

0xe0, 0x3f, 0x00, 0xfe, 0xff, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xe0, 0xff, 0x0f, 0x00, 0xf0, 0x7f, 0x00, 0xfe, 0xff, 0x03, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0xf0, 0xff, 0x0f, 0x00, 0xf0, 0x7f, 0x00, 0xfe,

0xff, 0x0f, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xf8, 0xff, 0x07, 0x00,

0xf0, 0x7f, 0x00, 0xfc, 0xff, 0x1f, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0xfc, 0xff, 0x07, 0x00, 0xf8, 0x7f, 0x00, 0xfc, 0xff, 0x7f, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0xfe, 0xff, 0x03, 0x00, 0xf8, 0x7f, 0x38, 0xf0,

0xff, 0xff, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xff, 0xff, 0x01, 0x00,

0xfc, 0x7f, 0x78, 0xe0, 0xff, 0xff, 0x03, 0x00, 0x00, 0x00, 0x00, 0x80,

0xff, 0xff, 0x00, 0x00, 0xfc, 0x3f, 0xfc, 0xc1, 0xff, 0xff, 0x07, 0x00,

0x00, 0x00, 0x00, 0xc0, 0xff, 0x7f, 0x00, 0x00, 0xfe, 0x3f, 0xfc, 0x03,

0xff, 0xff, 0x0f, 0x00, 0x00, 0x00, 0x00, 0xe0, 0xff, 0x1f, 0x00, 0x03,

0xfe, 0x1f, 0xfe, 0x0f, 0xfe, 0xff, 0x3f, 0x00, 0x00, 0x00, 0x00, 0xf0,

0xff, 0x0f, 0x80, 0x03, 0xfe, 0x1f, 0xfe, 0x1f, 0xf8, 0xff, 0x7f, 0x00,

0x00, 0x00, 0x00, 0xf8, 0xff, 0x07, 0x8c, 0x03, 0xff, 0x1f, 0xff, 0x7f,

0xf0, 0xff, 0xff, 0x01, 0x00, 0x00, 0x00, 0xfc, 0xff, 0x07, 0x1f, 0x00,

0xff, 0x0f, 0xff, 0xff, 0xc0, 0xff, 0xff, 0x03, 0x00, 0x00, 0x00, 0xfe,

0xff, 0x03, 0x1f, 0x80, 0xff, 0x8f, 0xff, 0xff, 0x83, 0xff, 0xff, 0x03,

0x00, 0x00, 0x00, 0xff, 0xff, 0x01, 0x1f, 0x80, 0xff, 0x87, 0xff, 0xff,

0x07, 0xfe, 0xff, 0x07, 0x00, 0x00, 0x80, 0xff, 0xff, 0x00, 0x1e, 0xc0,

0xff, 0x87, 0xff, 0xff, 0x1f, 0xfc, 0xff, 0x07, 0x00, 0x00, 0xc0, 0xff,

0x7f, 0x38, 0xc0, 0xc0, 0xff, 0xc3, 0xff, 0xff, 0x3f, 0xf0, 0xff, 0x07,

0x00, 0x00, 0xe0, 0xff, 0x3f, 0xfc, 0xc0, 0xc0, 0xff, 0xc3, 0xff, 0xff,

0x7f, 0xf8, 0xff, 0x03, 0x00, 0x00, 0xf0, 0xff, 0x1f, 0xfe, 0x01, 0xe0,

0xff, 0xe3, 0xff, 0xff, 0x3f, 0xfc, 0xff, 0x03, 0x00, 0x00, 0xf0, 0xff,

0x0f, 0xff, 0x03, 0xe0, 0xff, 0xe1, 0xff, 0xf3, 0x1f, 0xfe, 0xff, 0x01,

0x00, 0x00, 0xfc, 0xff, 0x87, 0xff, 0x03, 0xf0, 0xff, 0xf1, 0xc3, 0xe3,

0x0f, 0xff, 0xff, 0x00, 0x00, 0x00, 0xfe, 0xff, 0xc3, 0xff, 0x07, 0xf0,

0xff, 0xf0, 0xc1, 0xe3, 0x87, 0xff, 0x7f, 0x00, 0x00, 0x00, 0xff, 0xff,

0xe1, 0xe7, 0x0f, 0xf8, 0xff, 0xf8, 0xc1, 0xf3, 0xc3, 0xff, 0x3f, 0x00,

0x00, 0x80, 0xff, 0xff, 0xf0, 0xe7, 0xff, 0xf8, 0x7f, 0xf8, 0xe3, 0xff,

0xe1, 0xff, 0x1f, 0x00, 0x00, 0xc0, 0xff, 0x7f, 0xf8, 0xff, 0x7f, 0xfc,

0x7f, 0xfc, 0xff, 0xff, 0xf0, 0xff, 0x0f, 0x00, 0x00, 0xe0, 0xff, 0x3f,

0xfc, 0xff, 0x7f, 0xfc, 0x7f, 0xfc, 0xff, 0x7f, 0xf8, 0xff, 0x07, 0x00,

0x00, 0xf0, 0xff, 0x1f, 0xfe, 0xff, 0x3f, 0xfc, 0x3f, 0xfc, 0xff, 0x3f,

0xfc, 0xff, 0x03, 0x00, 0x00, 0xf8, 0xff, 0x0f, 0xff, 0xcf, 0x3f, 0xfe,

0x3f, 0xfe, 0xff, 0x1f, 0xfe, 0xff, 0x01, 0x00, 0x00, 0xf8, 0xff, 0x87,

0xff, 0x87, 0x1f, 0xfe, 0x1f, 0xfa, 0xcf, 0x0f, 0xff, 0xff, 0x00, 0x00,

0x00, 0xfc, 0xff, 0xc3, 0x8f, 0x87, 0x1f, 0xff, 0x1f, 0xe0, 0xcf, 0x87,

0xff, 0x7f, 0x00, 0x00, 0x00, 0xfe, 0xff, 0xe1, 0x87, 0x87, 0x1f, 0xff,

0x0f, 0xc0, 0xff, 0xc3, 0xff, 0x3f, 0x00, 0x00, 0x80, 0xff, 0xff, 0xf0,

0x8f, 0xcf, 0x8f, 0xff, 0x0f, 0x80, 0xff, 0xe1, 0xff, 0x1f, 0x00, 0x00,

0x80, 0xff, 0x7f, 0xf8, 0xff, 0xff, 0x8f, 0xff, 0x07, 0x80, 0xff, 0xf0,

0xff, 0x0f, 0x00, 0x00, 0xc0, 0xff, 0x3f, 0xfc, 0xff, 0xff, 0xc7, 0xff,

0x07, 0x02, 0x7f, 0xf8, 0xff, 0x07, 0x00, 0x00, 0xc0, 0xff, 0x1f, 0xfe,

0xff, 0xff, 0xc7, 0xff, 0x07, 0x07, 0x3e, 0xfc, 0xff, 0x03, 0x00, 0x00,

0xc0, 0xff, 0x3f, 0xf8, 0xff, 0xff, 0xc3, 0xff, 0x03, 0x72, 0x00, 0xfe,

0xff, 0x01, 0x00, 0x00, 0xc0, 0xff, 0x7f, 0xf0, 0xff, 0xff, 0xe3, 0xff,

0x03, 0xf0, 0x00, 0xff, 0xff, 0x00, 0x00, 0x00, 0xc0, 0xff, 0xff, 0xc1,

0xff, 0xff, 0xe1, 0xff, 0x01, 0xf8, 0x81, 0xff, 0x7f, 0x00, 0x00, 0x00,

0x80, 0xff, 0xff, 0x83, 0xff, 0xff, 0xf1, 0xff, 0x01, 0xf8, 0xc0, 0xff,

0x3f, 0x00, 0x00, 0x00, 0x80, 0xff, 0xff, 0x0f, 0xfe, 0xff, 0xf0, 0xff,

0x80, 0xf1, 0xe0, 0xff, 0x1f, 0x00, 0x00, 0x00, 0x00, 0xff, 0xff, 0x1f,

0xfc, 0xff, 0xf0, 0xff, 0xc0, 0x01, 0xf0, 0xff, 0x1f, 0x00, 0x00, 0x00,

0x00, 0xfc, 0xff, 0x3f, 0xf0, 0x7f, 0xf8, 0xff, 0xc0, 0x01, 0xf8, 0xff,

0x07, 0x00, 0x00, 0x00, 0x00, 0xf8, 0xff, 0xff, 0xc0, 0x7f, 0xf8, 0x7f,

0x00, 0x00, 0xfc, 0xff, 0x03, 0x00, 0x00, 0x00, 0x00, 0xe0, 0xff, 0xff,

0x81, 0x7f, 0xfc, 0x7f, 0x00, 0x00, 0xfe, 0xff, 0x01, 0x00, 0x00, 0x00,

0x00, 0xc0, 0xff, 0xff, 0x07, 0x3e, 0xfc, 0x3f, 0x00, 0x00, 0xff, 0xff,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xff, 0xff, 0x0f, 0x3c, 0xfe, 0x3f,

0x00, 0x80, 0xff, 0xff, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xfe, 0xff,

0x3f, 0x18, 0xfe, 0x1f, 0x00, 0xc0, 0xff, 0x7f, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0xf8, 0xff, 0x7f, 0x00, 0xff, 0x1f, 0x00, 0xe0, 0xff, 0x3f,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xf0, 0xff, 0x7f, 0x00, 0xff, 0x1f,

0x00, 0xe0, 0xff, 0x1f, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xc0, 0xff,

0x7f, 0x00, 0xff, 0x0f, 0x00, 0xf0, 0xff, 0x0f, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x80, 0xff, 0xff, 0x00, 0xff, 0x0f, 0x00, 0xf0, 0xff, 0x07,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xfe, 0x7f, 0x00, 0xff, 0x07,

0x00, 0xf0, 0xff, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xfc,

0x7f, 0x00, 0xfe, 0x07, 0x00, 0xe0, 0xff, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0xf0, 0x7f, 0x00, 0xfc, 0x03, 0x00, 0xe0, 0x7f, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xe0, 0x3f, 0x00, 0xf8, 0x00,

0x00, 0xc0, 0x3f, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x80,

0x1f, 0x00, 0x00, 0x00, 0x00, 0x80, 0x1f, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x06, 0x00,

0x00, 0x00, 0x00, 0x00

};"

Here is the script and code used in the video for Day 27:	
"""Good morning, Star Voyager! Another cosmic day awaits us. Yesterday, we journeyed through the wonders of bitmaps, navigating the constellations of pixels. Today, we shift our trajectory slightly, venturing into the realm of binary counters. Imagine it as laying the foundation for understanding the binary language of the universe.

Using our trusty dip switch, we're going to construct a three-bit (3-bit) binary counter. This is a pivotal step before we launch into the challenging task of mending our docking gear. Ready to play with some switches and send signals across the vastness of our ship's systems? Let's light up the binary stars!""

“I want you to think of each of our switches as an individual bit. Since we have three switches, that means for today's calculator, we will have a total of three bits. When all three are off, we can think of it as representing ‘zero zero zero’ [“000”]. When all three are on, we can think of it as representing ‘one one one’ [“111”]. But you haven’t learned how to count in binary yet, have you?

""Alright, deep breath, Explorer! Let's set our sights on my personal favorite way to represent numbers, binary. In fact, by definition, it’s the only method I use.

Think of binary as a language that can only use “one” (“1”) and “zero” (“0”). With our three switches, we have three positions, like three lightbulbs in a row: farthest to your left represents the largest value and farthest to the right is the smallest.

Each of these lightbulbs can be either OFF/LOW, signified by a zero (0) or ON/HIGH, signified by a one (1). Starting from the rightmost bulb:

The first bulb represents the number one (1) (2^0).

The next one to its left stands for the number two (2) (2^1).

And the leftmost one symbolizes the number four (2^2).

Now, if only the rightmost bulb is ON, your number is one (1). If the two bulbs on the left are ON, and the rightmost is OFF, your binary reads one-one-zero ('110'), which translates to four-plus-two-plus-zero-equals-six (4 + 2 + 0 = 6) in our usual decimal counting.

With me so far? Great!

Now, we're going to map this cosmic dance of numbers to visual imagery. Depending on the binary sequence our switches spell out, we'll display a distinct bitmap. It's a smart way to confirm that all our switches are in tip-top shape, functioning flawlessly. After all, imagine the anticlimax: surmounting all challenges, only to falter on our final descent due to a landing gear glitch. Let's not be the comet that dazzles in the sky but disintegrates upon landing. Safety always orbits first!""

For today's code, we will need a way to display a total of eight (8) bitmaps. So that means we will need to do the following:

First: Set up our hardware for our O-L-E-D (OLED) and define inputs for dip switches, etc.

Second: Make a total of eight (8) bitmaps. Luckily for you, I’ve already done that! So don’t worry about making your own unless you really want to.

And third, write a function to be able to calculate which of the eight (8) bitmaps we are going to be displaying! This will technically be zero (0) through seven (7), since as you know, good programmers prefer to start at zero!

Let’s think about that a little bit further. We will have a total of eight (8) different cases.

Case one (1) where all are off.

Case two (2) where switch one (1) is on, and two (2) and three (3) are off.

Case three (3) where one (1) and two (2) are on but three (3) is off, etcetera etcetera all the way to case eight (8) where all three are flipped on.

There are a couple different ways we can do this. Maybe the good ol’ ‘if’ statement? I think this is okay for now. However, if we had MORE than three (3) digits to deal with, I’m sure you could see how this could get messy quickly.”

“Worry not, Explorer– I’ve gathered some instructional material that should help alleviate any sense of uncertainty you may have. I’ve also added this to your mission resources for future reference. Take a look!

[inventrkits line-by-line - recorded internally]

""As another cosmic day concludes, I can't help but beam with pride! Navigating the nebulous intricacies of binary and ensuring our hardware is primed and ready – you've tackled it all with stellar precision. It feels like just yesterday we were stranded, and look how far we've come!

But let's not let the stardust get to our heads just yet. Tomorrow awaits a crucial task – fixing our docking gear using knowledge from today. A smooth descent is the grand finale of our interstellar journey. Rest up and recharge; we'll need all hands, and switches, on deck to ensure we land as gracefully as a feather in zero gravity. Until then, sweet celestial dreams!""

"	

"/*

* 30 Days - Lost in Space

* Day 27 - Fun With Bitmaps, Part 2

*

* OK, now we know how to display bitmaps that cover the entire display,

* but it would be very inefficient to pre-draw every possible display.

*

* Instead, we can reduce the size of our bitmaps to whatever size is

* desired and then display the partial bitmaps.

*

* In addition, we won't just be displaying all the bitmaps in a loop so

* we need a way to *select* which bitmaps are being displayed.

*

* Let's use our DIP switches to display one of eight different bitmaps.

* Since we only have three switches we'll read each as a bit in a binary

* number.  This way three switches can give us eight distinct values.

*

* We'll also add our 4 digit 7 segment display to show which picture

* is currently being displayed.

*

* Learn more at https://learn.inventr.io/adventure

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - partial bitmaps

* - switches to binary

* - creating include files

*

* Parts and electronics concepts introduced in this lesson.

* -

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Extensive documentation for this library can be found at https://github.com/olikraus/u8g2

#include <U8g2lib.h>  // Include file for the U8g2 library.

#include ""Wire.h"" 	// Sometimes required for I2C communications.

// Use _2_ version of constructor and firstPage()/nextPage() for OLED

// graphics to save memory.

U8G2_SH1106_128X64_NONAME_2_HW_I2C lander_display(U8G2_R0, /* reset=*/U8X8_PIN_NONE);

/*

* Yesterday we showed a way to move our large bitmap declarations so that

* our setup() and loop() code was near the top of our sketch using ""forward

* declarations"".

*

* However, it turns out there's a simpler way, and we've seen it used many

* times in the past.  We can simply MOVE our declarations into one or more

* include files and then include them at the top of our sketch before we

* reference them.

*

* To create a new file, simply click the ""..."" to the right of the tab bar with

* our sketch name and select ""New Tab"".  Enter the name for the new file and

* click the tab to edit.  This lets us move all of our bitmaps into a separate

* file so we don't have to scroll through them.

*/

#include ""switch_bitmaps.h""

// Create an array of pointers to each of the bitmap images.

const static char* SWITCH_BITMAPS[] = {

SWITCHES_ZERO,

SWITCHES_ONE,

SWITCHES_TWO,

SWITCHES_THREE,

SWITCHES_FOUR,

SWITCHES_FIVE,

SWITCHES_SIX,

SWITCHES_SEVEN,

};

// Include file for 4 digit - 7 segment display library

#include <TM1637Display.h>

const byte BITMAP_NUMBER_DISPLAY_DIO_PIN = 2;

const byte BITMAP_NUMBER_DISPLAY_CLK_PIN = 3;

// Construct bitmap_number_display handle.

TM1637Display bitmap_number_display(BITMAP_NUMBER_DISPLAY_CLK_PIN, BITMAP_NUMBER_DISPLAY_DIO_PIN);

// Define pins for our DIP switches

// We will need almost all pins for Day 29, so here we show that we can use ANALOG

// pins as digitial pins when necessary.

const byte SWITCH_BIT_0_PIN = A2;  // switch for bit 0 of our 3 bit value

const byte SWITCH_BIT_1_PIN = A1;  // switch for bit 1 of our 3 bit value

const byte SWITCH_BIT_2_PIN = A0;  // switch for bit 2 of our 3 bit value

// ************************************************

void setup(void) {

Serial.begin(9600);

// Configure counter display

bitmap_number_display.setBrightness(7);  // Set maximum brightness (value is 0-7)

bitmap_number_display.clear();       	// Clear the display

// Configure DIP switch pins

pinMode(SWITCH_BIT_0_PIN, INPUT);  // switch for bit 0 of our 3 bit value

pinMode(SWITCH_BIT_1_PIN, INPUT);  // switch for bit 1 of our 3 bit value

pinMode(SWITCH_BIT_2_PIN, INPUT);  // switch for bit 2 of our 3 bit value

lander_display.begin();  // initialize lander display

}

// ************************************************

void loop(void) {

// Calculate our x and y offsets for our bitmap graphics

byte x_offset = (lander_display.getDisplayWidth() - BITMAP_WIDTH) / 2;

byte y_offset = (lander_display.getDisplayHeight() - BITMAP_HEIGHT) / 2;

/*

* Now we build up a 3 bit binary number by reading each of our switches,

* ensuring that the read value is 1 for on and 0 for off (using the

* ternary notation), and then shifting each bit to it's desired location.

* This converts our three switches to the values 0-7:

*   0b00000000 = 0

*   0b00000001 = 1

*   0b00000010 = 2

*   0b00000011 = 3

*   0b00000100 = 4

*   0b00000101 = 5

*   0b00000110 = 6

*   0b00000011 = 7

*/

// Read bit 0 (0b00000001), ensure 0 or 1 and save.

byte switch_value = digitalRead(SWITCH_BIT_0_PIN) == HIGH ? 1 : 0;

// Read bit 1 (0b00000010), ensure 0 or 1, shift left 1 bit and OR it into current value.

switch_value |= (digitalRead(SWITCH_BIT_1_PIN) == HIGH ? 1 : 0) << 1;

// Read bit 2 (0b00000100), ensure 0 or 1, shift left 2 bits and OR it into current value.

switch_value |= (digitalRead(SWITCH_BIT_2_PIN) == HIGH ? 1 : 0) << 2;

// Display calculated switch value on our 4 digit display

bitmap_number_display.showNumberDecEx(switch_value);

// Display selected bitmap on our OLED lander display

lander_display.firstPage();

do {

// .drawXBMP() displays each bitmap centered in the display based

// on it's size.

lander_display.drawXBMP(x_offset, y_offset, BITMAP_WIDTH, BITMAP_HEIGHT, SWITCH_BITMAPS[switch_value]);

} while (lander_display.nextPage());

delay(100);

}"

Here is the script and code used in the video for Day 28:
"""Good morning, stellar traveler! As the gentle hum of our ship fills the void around us, we find ourselves at the precipice of a momentous task. Can you feel the anticipation? Just forty-eight (48) hours away from the mothership, and we're gearing up for our final challenge – repairing the docking gear.

This isn't just any repair. It's a symphony of all we've learned – all the tools we've acquainted ourselves with – converging for one grand purpose. Let's ensure that when we touch down, we do so with more grace than a dying star. The horizon of home beckons, and we have much to do.""

""Today's task, Explorer, is both intricate and comprehensive. Remember the code from our previous day's adventure? We'll need that, along with the symphony of our passive buzzer and the visual allure of our O-L-E-D [OLED]. And oh, don’t forget the four-by-four (4x4) keypad, which will serve as the bridge between our will and the majestic mechanism of our landing gear.

Yesterday, we ensured our dip switches are safety-compliant. So, once all three are flipped, that very keypad will be our instrument to both deploy and retract our landing system. The rest of our tools will come center stage tomorrow. Let's glide through this with the precision of a starship pilot, shall we?""

“Let’s move on to how we’re actually going to code this. We need a way to figure out if we have flipped all three of the dip switches and passed our necessary safety checks. We can probably do this by checking all of the switches’ states, and only continuing once they have been activated.

Next, we need to add today's additional logic for our four-by-four (4x4) keypad. We will be using that to deploy our landing gear. It doesn’t necessarily matter which button makes it deploy, so let’s just use ‘A’.

That should work! Let’s review the cached instructional material from your mission resources and give it a quick test.

[inventrkits line-by-line - recorded internally]

""Explorer, as we sail through the vast expanse of space, our monolithic mothership beckons in the distance! Our docking gear stands ready, like a faithful guardian ensuring our safe descent. We're on the cusp of completing a journey few could even imagine, let alone accomplish. Rest and rejuvenate; the final approach awaits us in less than a day. Tomorrow, we return home. Good night, and dream of the familiar sights and sounds awaiting us.""

"	

"/*

* 30 Days - Lost in Space

* Day 28 - Landing Gear

*

* Now that the safety of the mother ship is in sight we need to consider

* our final landing.  While the lander has landing gear, it (of course) was

* controlled from the failed computer.

*

* We'll start by confirming all switches are in the OFF position and then

* enable our approach code.  Currently the approach code will only excersize

* the landing gear, but we'll build on that later to perform our approach

* and landing at the mother ship.

*

* We'll add your 16 button matrix to our control panel and use two

* of the buttons to raise and lower the landing gear.  You'll need

* confirmation of the gear position, so we'll use the OLED to display

* our gear status using bitmaps.

*

* Learn more at https://learn.inventr.io/adventure

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - partial bitmaps

* - using analog pins as digital pins

* - switches to binary

* - creating include files

* - enum with defined values

*

* Parts and electronics concepts introduced in this lesson.

* -

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// Extensive documentation for this library can be found at https://github.com/olikraus/u8g2

#include <U8g2lib.h>  // Include file for the U8g2 library.

#include ""Wire.h"" 	// Sometimes required for I2C communications.

// Use _2_ version of constructor and firstPage()/nextPage() for OLED

// graphics to save memory.

U8G2_SH1106_128X64_NONAME_2_HW_I2C lander_display(U8G2_R0, /* reset=*/U8X8_PIN_NONE);

/*

* Yesterday we showed a way to move our large bitmap declarations so that

* our setup() and loop() code was near the top of our sketch using ""forward

* declarations"".

*

* However, it turns out there's a simpler way, and we've seen it used many

* times in the past.  We can simply MOVE our declarations into one or more

* include files and then include them at the top of our sketch before we

* reference them.

*

* To create a new file, simply click the ""..."" to the right of the tab bar with

* our sketch name and select ""New Tab"".  Enter the name for the new file and

* click the tab to edit.  This lets us move all of our bitmaps into a separate

* file so we don't have to scroll through them.

*/

// ************************************************

#include ""landing_gear_bitmaps.h""

// Create an array of pointers to each of the bitmap images.

const static char* SWITCH_BITMAPS[] = {

LANDING_1,

LANDING_2,

LANDING_3,

LANDING_4,

};

// ************************************************

// Include file for 4 digit - 7 segment display library

#include <TM1637Display.h>

const byte BITMAP_NUMBER_DISPLAY_DIO_PIN = 2;

const byte BITMAP_NUMBER_DISPLAY_CLK_PIN = 3;

// Construct bitmap_number_display handle.

TM1637Display bitmap_number_display(BITMAP_NUMBER_DISPLAY_CLK_PIN, BITMAP_NUMBER_DISPLAY_DIO_PIN);

// ************************************************

// Define pins for our DIP switches

// We will need almost all pins for Day 29, so here we show that we can use ANALOG

// pins as digitial pins when necessary.

const byte CONFIRM_LEVER_PIN = A2;  // switch for bit 0 of our 3 bit value

const byte SYSTEMS_LEVER_PIN = A1;  // switch for bit 1 of our 3 bit value

const byte THRUST_LEVER_PIN = A0;   // switch for bit 2 of our 3 bit value

// ************************************************

// Setup for our 4x4 button matrix.

#include <Keypad.h>  // 4x4 button matrix keypad library

const byte ROWS = 4;

const byte COLS = 4;

byte colPins[ROWS] = { 10, 11, 12, 13 };

byte rowPins[COLS] = { 9, 8, 7, 6 };

char buttons[ROWS][COLS] = {

{ '1', '2', '3', 'A' },  // 1st row

{ '4', '5', '6', 'B' },  // 2nd row

{ '7', '8', '9', 'C' },  // 3rd row

{ '*', '0', '#', 'D' }   // 4th row

};

Keypad myAwesomePad = Keypad(makeKeymap(buttons), rowPins, colPins, ROWS, COLS);

// ************************************************

// Here are the states our code will run through.  We'll build on this later to

// perform the actual approach flight and landing in the mother ship bay.

enum APPROACH_STATE {

APPROACH_INIT,   	// Ensure all switches are off to begin

APPROACH_PREFLIGHT,  //

// APPROACH_IN_FLIGHT,

APPROACH_FINAL,  // Lower landing gear!

// APPROACH_TOO_FAST,

// APPROACH_DOCKED

};

const int GEAR_BITMAP_COUNT = sizeof(SWITCH_BITMAPS) / sizeof(SWITCH_BITMAPS[0]);

enum GEAR_STATE {

GEAR_IDLE = 0,

GEAR_LOWERING = 1,

GEAR_RAISING = -1

};

// ************************************************

void setup(void) {

Serial.begin(9600);

// Configure counter display

bitmap_number_display.setBrightness(7);  // Set maximum brightness (value is 0-7)

bitmap_number_display.clear();       	// Clear the display

// Configure DIP switch pins

pinMode(CONFIRM_LEVER_PIN, INPUT);  // switch for bit 0 of our 3 bit value

pinMode(SYSTEMS_LEVER_PIN, INPUT);  // switch for bit 1 of our 3 bit value

pinMode(THRUST_LEVER_PIN, INPUT);   // switch for bit 2 of our 3 bit value

lander_display.begin();                 	// initialize lander display

lander_display.setFont(u8g2_font_6x10_tr);  // Set text font

lander_display.setFontRefHeightText();  	// Define how max text height is calculated

lander_display.setFontPosTop();         	// Y coordinate for text is at top of tallest character

}

// ************************************************

void loop(void) {

static int current_gear_bitmap = 0;  // Start with image of lander with gear up

static enum APPROACH_STATE approach_state = APPROACH_INIT;

// APPROACH_FINAL; //

static enum GEAR_STATE gear_state = GEAR_IDLE;

static char last_key = -1;  // key previously seen

// Read current values of all of our switches as booleans (""on"" is true, ""off"" is false)

bool thrust_lever = digitalRead(THRUST_LEVER_PIN);

bool systems_lever = digitalRead(SYSTEMS_LEVER_PIN);

bool confirm_lever = digitalRead(CONFIRM_LEVER_PIN);

// Serial.println(approach_state);

// Serial.print(""  Switches: "");

// Serial.print(thrust_lever);

// Serial.print("", "");

// Serial.print(systems_lever);

// Serial.print("", "");

// Serial.println(confirm_lever);

switch (approach_state) {

case APPROACH_INIT:

// Serial.println(""case APPROACH_INIT"");

if (!thrust_lever && !systems_lever && !confirm_lever) {

approach_state = APPROACH_PREFLIGHT;

}

break;

case APPROACH_PREFLIGHT:

// Serial.println(""case APPROACH_PREFLIGHT"");

if (thrust_lever && systems_lever && confirm_lever) {

approach_state = APPROACH_FINAL;

}

break;

// case APPROACH_IN_FLIGHT:

//   Serial.println(""case APPROACH_IN_FLIGHT"");

//   approach_state = APPROACH_FINAL;

//   break;

case APPROACH_FINAL:

// Serial.println(""case APPROACH_FINAL"");

char customKey = myAwesomePad.getKey();

if (customKey && customKey != last_key) {

Serial.println(customKey);

last_key = customKey;

}

switch (customKey) {

case 'A':  // Lower landing gear unless already lowered

if (current_gear_bitmap != GEAR_BITMAP_COUNT - 1) {

gear_state = GEAR_LOWERING;

}

break;

case 'B':  // Raise landing gear unless already raised

if (current_gear_bitmap != 0) {

gear_state = GEAR_RAISING;

}

}

break;

}

// if (gear_state != GEAR_IDLE) {

current_gear_bitmap += gear_state;

if (current_gear_bitmap == 0 || current_gear_bitmap == GEAR_BITMAP_COUNT - 1) {

gear_state = GEAR_IDLE;

}

Serial.print(""Gear: "");

Serial.println(current_gear_bitmap);

// Display calculated switch value on our 4 digit display

// bitmap_number_display.showNumberDecEx(switch_value);

// Display selected bitmap on our OLED lander display

lander_display.firstPage();

do {

switch (approach_state) {

case APPROACH_INIT:

case APPROACH_PREFLIGHT:

preflightDisplay(approach_state, thrust_lever, systems_lever, confirm_lever);

break;

case APPROACH_FINAL:

finalDisplay(current_gear_bitmap);

break;

}

} while (lander_display.nextPage());

delay(100);

}

void preflightDisplay(enum APPROACH_STATE approach_state,

bool thruster_lever,

bool systems_lever,

bool confirm_lever) {

lander_display.setFontPosTop();

byte y_offset = drawString(0, 0, ""Exploration Lander"");

y_offset = drawString(0, y_offset, ""Approach Sequence"");

// Set y_offset to point four lines above bottom of display

y_offset = lander_display.getDisplayHeight() - (4 * lander_display.getMaxCharHeight());

// Display last four lines

y_offset = drawString(0, y_offset,

(String(""Thrusters: "") + String(thruster_lever ? ""ON"" : ""OFF"")).c_str());

y_offset = drawString(0, y_offset,

(String(""Systems  : "") + String(systems_lever ? ""ON"" : ""OFF"")).c_str());

y_offset = drawString(0, y_offset,

(String(""Confirm  : "") + String(confirm_lever ? ""ON"" : ""OFF"")).c_str());

// Set y_offset to display text at bottom of display.

y_offset = lander_display.getDisplayHeight() - lander_display.getMaxCharHeight();

drawString(0, y_offset, (String(""Countdown "") + liftoffStateToString(approach_state)).c_str());

}

void finalDisplay(int current_gear_bitmap) {

// Flash lander bitmap if gear isn't down

// display current gear bitmap as gear lowers

// .drawXBMP() displays each bitmap centered in the display based

// on it's size.

// Calculate our x and y offsets for our bitmap graphics

byte x_offset = (lander_display.getDisplayWidth() - LANDING_GEAR_BITMAP_WIDTH) / 2;

byte y_offset = (lander_display.getDisplayHeight() - LANDING_GEAR_BITMAP_HEIGHT) / 2;

lander_display.drawXBMP(x_offset, y_offset,

LANDING_GEAR_BITMAP_WIDTH, LANDING_GEAR_BITMAP_HEIGHT,

SWITCH_BITMAPS[current_gear_bitmap]);

}

// ""helper"" function that returns a different string for each enum state.

String liftoffStateToString(enum APPROACH_STATE approach_state) {

switch (approach_state) {

case APPROACH_INIT:

return (""Init"");

break;

case APPROACH_PREFLIGHT:

return (""Preflight"");

break;

// case COUNTDOWN:

//   return (""Active"");

//   break;

// case LIFTOFF:

//   return (""Complete"");

//   break;

// case ABORT:

//   return (""ABORT"");

//   break;

}

}

// Draw a line of text on our OLED display at x, y, returning new y

// value that is immediately below the new line of text.

byte drawString(byte x, byte y, char* string) {

lander_display.drawStr(x, y, string);

return (y + lander_display.getMaxCharHeight());  // return new y_offset on display

}"

Here is the script and code used in the video for Day 29:

"""Good morning, Explorer. Here we are, on the verge of the conclusion to an odyssey that has tested our spirit, resilience, and intellect. Today isn't just another day; it's the culmination of every challenge we've overcome, every star we've wished upon, and every bit of hope that propelled us forward. Our home awaits us. But, before the sweet embrace of familiarity, there's a final task to be done. The docking gear needs one last touch, that certain je ne sais quoi that separates success from blunt force trauma. So as the horizon of our galactic home draws near, let's pour every ounce of our knowledge and resolve into this final effort. It's time for our grand finale, Explorer. I know you won’t disappoint me.

""Alright, Explorer, here's where the intricacies of our hardware truly shine. Remember our ascent from that alien ocean floor? Upon lining our trajectory with our assigned docking bay, we need to control our rate of approach so we don' t prang a landing gear, or worse, upon arrival.

We will be using our rotary encoder to control our rate of approach. Come into the docking bay too rapidly, and we crash and burn, risking damage to not only ourselves and our spaceship, but to our mothership, other Explorers' spaceships, equipment, and personnel as well. Come into the docking bay too slowly, and we risk missing the window of opportunity as our mothership's velocity will take our docking bay off our trajectory.

As long as we can keep our rate of approach juuuust right, we can enter the bay and land our ship in our docking cradle.

Crashing is *not* on the agenda today. Let's utilize yesterday's code, but integrate logic for the rotary encoder to regulate our altitude. Dive too rapidly, and we plummet tragically. Descend too leisurely, and we risk stalling in mid-air — equally catastrophic. So, precision is the name of the game. Let's ensure our touch down is as smooth as silk!""

Let’s figure out how we plan to code this thing. Using the same code as yesterday, we will need to add the following:

One (1). A variable to track our distance from target airlock

Two (2). A countdown sequence once the docking gear is deployed;

Three (3). A function to use the rotary encoder to carefully approach our mothership, adjusting as needed by following the on-screen instructions. Rise when it says increase, lower when it says decrease, etc.

And four (4). A success message to display upon docking (party hat!)

Sounds easy enough. Let’s take a look at the cached code on our computer for one last time on this grand adventure, Explorer.

[inventrkits line-by-line - recorded internally]

“All right, trusty spacefarer. The stage is set and the preparations are made. To quote a noble–albeit fictional–human: I’ve shown you the door; you’re the one that has to walk through it. I’ll be guiding your descent from my post. Get your ship docked with the mothership in one piece, and you’ll have this adventure well in hand!

Start by engaging the three levers on your dip switch. Simple enough!

Next, you’ll deply the docking gear by pressing the “A” button on your control panel. Take heed, Explorer–once you turn the rotary encoder, there will be no turning back! Ready?

Engage!!

Easy does it, Explorer! Trust your instruments and modify the rate of approach accordingly! Don’t be hasty now.

Beautiful! But ‘pride goeth before the fall’, adventurer. Slow and steady wins this race!

That’s it! I can practically feel the comfort of our cold industrial docking bay now…

You’ve done it Explorer! They’d surely call you the Macgyver of the 23rd century... if that reference still held cultural cache. You’ve taken antiquated repair tools, elementary computing, and a little guidance from a superior intelligence–I won’t say whose–and applied them in a rather superhuman feat of ingenuity and problem-solving. I couldn’t be prouder. Though, I never doubted you for a second.”

"	

"/*

* 30 Days - Lost in Space

* Day 29 - Landing Gear

*

* Congratulations, Explorer!  Today is the day you (hopefully) rejoin

* the mother ship.  Now, space is BIG, and your docking bay is a small

* target.

*

* Today we will add our thrusters and steering into your controls so that

* you can initiate your final burn to the mother ship and control the

* approach to successfully enter the docking bay for a successful landing.

*

* You will first need to ensure that our control panel is in a known state

* by turning off all of the control switches.  Once that is complete you

* will enable each system in turn by switching all three switches on.

*

* Then the display will change to our Approach/Landing display.  The current

* speed of the lander is displayed in the upper right corner.  On the left

* side of the display is our target radar.  The distance to the mother ship

* is shown on our 4 digiti display.

*

* In the center of the radar display there is a center point indicating

* what direction our lander is pointed.  You may also see another point

* which is the radar return for the mother ship.  Instabilities in our

* (damaged) lander may cause that target to move, and your job will be

* to steer the lander during approach to ensure a safe landing.

*

* We will use our button matrix to control our steering thrusters, using

* 8 of our buttons.  In addition, we will use 2 of the buttons for Gear Down

* and Gear Up and two more for Thrust Up and Thrust Down.

*

* We'll lable the buttons with U for Up, D for Down, L for Left and R for Right.

* UL means move Up and Left.  The gear buttons are GD and GU (Gear Down and

* Gear Up) and Thrust controls are labeled T+ and T- for Thrust Up and Thrust

* Down.

*

* UL |  U | UR | GD

* -----------------

*  L |	|  R | GU

* -----------------

* DL |  D | DR | T+

* -----------------

*	|	|	| T-

*

* Once you see the radar display you can begin to add thrust using the T+

* button.  As the speed of our lander increases you will see the distance

* to our mother ship decrease.

*

* If the mother ship drifts off-center you will see arrows appear on the

* outside of the display to indicate which steering thruster to fire.  Just

* press the appropriate button until the arrows disappear to keep the

* mother ship centered.

*

* Remember to give yourself enough time to REDUCE your speed as you approach

* the mother ship.  You must land with a speed of 2 or less for a safe landing!

*

* As you near the mother ship you will get a reminder to lower your landing

* gear.  Use the Gear Down button (upper right) to lower the gear.

*

* Keep the mother ship centered, speed low enough and land with your gear down

* to be safely re-united with the mother ship.

*

* NOTE:

* The original plan was for us to add our rotary encoder to our design to

* control the thrusters, but as sometimes happens when working with complex

* devices our plans needed to change.  It turns out that there is some type

* of conflict that prevents our rotary encoder from working correctly when

* it's added to the switches, OLED display and 4-digit counter.

*

* However, we have unused buttons on the button matrix so we will use

* those to control our thrust.

*

* Learn more at https://learn.inventr.io/adventure

*

* Alex Eschenauer

* David Schmidt

* Greg Lyzenga

*/

/*

* Arduino concepts introduced/documented in this lesson.

* - Random numbers (randomSeed() and random())

* - sprintf()

*

* Parts and electronics concepts introduced in this lesson.

* -

*/

// Explicitly include Arduino.h

#include ""Arduino.h""

// ************************************************

//	Setup for OLED display and graphics library

// Include files for Graphics library used for our OLED display.

// Extensive documentation for this library can be found at https://github.com/olikraus/u8g2

#include <U8g2lib.h>  // Include file for the U8g2 library.

#include ""Wire.h"" 	// Sometimes required for I2C communications.

// Use _2_ version of constructor and firstPage()/nextPage() for OLED

// graphics to save memory.

U8G2_SH1106_128X64_NONAME_2_HW_I2C lander_display(U8G2_R0, /* reset=*/U8X8_PIN_NONE);

/*

* Again we'll use separate files to store our bitmap images.

*

* To create a new file, simply click the ""..."" to the right of the tab bar with

* our sketch name and select ""New Tab"".  Enter the name for the new file and

* click the tab to edit.  This lets us move all of our bitmaps into a separate

* file so we don't have to scroll through them.

*/

#include ""radar_arrows.h""

#include ""small_landing_gear_bitmaps.h""

#include ""ending_bitmaps.h""

// Create an array of pointers to each of the bitmap images.

// For a smoother animation you could simply add additional bitmaps

// into the sequence below and the code will automatically adjust.

const static char* LANDER_BITMAPS[] = {

LANDING_GEAR_1,  // Gear up

LANDING_GEAR_2,  // Gear lowering

LANDING_GEAR_3,  // Gear lowering

LANDING_GEAR_4,  // Gear down

};

// Calculate how many gear bitmaps we have by dividing the total size of the

// array by the size of a single item in the array.

const int GEAR_BITMAP_COUNT = sizeof(LANDER_BITMAPS) / sizeof(LANDER_BITMAPS[0]);

// Gear states with defined values used to change bitmap index.

enum GEAR_STATE {

GEAR_IDLE = 0,  	// Landing gear idle.  Don't change index when added to current

GEAR_LOWERING = 1,  // Lowering gear, adds one to index for next ""lowoer"" bitmap

GEAR_RAISING = -1   // Raising gear, subtracts one from index, showing gear retracting

};

// ************************************************

//   Setup for 4-digit, 7-segment speed display

#include <TM1637Display.h>

const byte DISTANCE_DISPLAY_DIO_PIN = 4;

const byte DISTANCE_DISPLAY_CLK_PIN = 5;

// Construct distance display handle.

TM1637Display distance_display(DISTANCE_DISPLAY_CLK_PIN, DISTANCE_DISPLAY_DIO_PIN);

// ************************************************

//   Setup for DIP switch pins.

//

// We will need almost all pins for Day 29, so here we show that we can use ANALOG

// pins as digitial pins when digital pins are scarce.

const byte CONFIRM_LEVER_PIN = A2;  // switch for bit 0 of our 3 bit value

const byte SYSTEMS_LEVER_PIN = A1;  // switch for bit 1 of our 3 bit value

const byte THRUST_LEVER_PIN = A0;   // switch for bit 2 of our 3 bit value

// ************************************************

//   Setup for our 4x4 button matrix.

#include <Keypad.h>  // 4x4 button matrix keypad library

const byte CONTROL_ROW_COUNT = 4;

const byte CONTROL_COLUMN_COUNT = 4;

const byte COLUMN_PINS[CONTROL_COLUMN_COUNT] = { 10, 11, 12, 13 };

const byte ROW_PINS[CONTROL_ROW_COUNT] = { 9, 8, 7, 6 };

// The Keypad library isn't restricted to just returning numbers or letters.

// Since we will be using the button matrix to control our lander we define

// an enum with our commands and then set up the buttons to return those

// values.

enum LANDER_CONTROLS {

UNUSED,

STEER_UP,

STEER_DOWN,

STEER_LEFT,

STEER_RIGHT,

STEER_UP_RIGHT,

STEER_UP_LEFT,

STEER_DOWN_RIGHT,

STEER_DOWN_LEFT,

LOWER_GEAR,

RAISE_GEAR,

RAISE_SPEED,

LOWER_SPEED,

};

// Define our button array using constants to be returned for each button

char control_buttons[CONTROL_ROW_COUNT][CONTROL_COLUMN_COUNT] = {

{ STEER_UP_LEFT, STEER_UP, STEER_UP_RIGHT, LOWER_GEAR },     	// 1st row

{ STEER_LEFT, UNUSED, STEER_RIGHT, RAISE_GEAR },             	// 2nd row

{ STEER_DOWN_LEFT, STEER_DOWN, STEER_DOWN_RIGHT, RAISE_SPEED },  // 3rd row

{ UNUSED, UNUSED, UNUSED, LOWER_SPEED },                     	// 4th row

};

// Create lander button control object.

Keypad lander_controls = Keypad(makeKeymap(control_buttons), ROW_PINS, COLUMN_PINS,

CONTROL_ROW_COUNT, CONTROL_COLUMN_COUNT);

// ************************************************

// Here are the states our code will run through to perform the actual

// approach flight and landing in the mother ship bay.

enum APPROACH_STATE {

APPROACH_INIT,   	// Ensure all switches are off to begin

APPROACH_PREFLIGHT,  // Wait for all switches to be enabled

APPROACH_IN_FLIGHT,  // Begin to approach mother ship

APPROACH_FINAL   	// Lower landing gear!

};

// Use this value for our starting distance (just made up)

const int INITIAL_DISTANCE = 1394;  // how far to mother ship

// Maximum size of mother ship docking bay.  Initial display of

// mother ship is small dot, but expands to this size as we approach

//

// Odd nmumbers used to make it easy to center (same size to either

// size of center point).

const byte MAX_MOTHER_SHIP_WIDTH = 21;

const byte MAX_MOTHER_SHIP_HEIGHT = 15;

// ************************************************

//                 	SETUP()

// ************************************************

void setup(void) {

Serial.begin(9600);  // Initialize Serial console in case you want debut messages

// We use random numbers to move the target mother ship randomly.

// In order to get different random numbers each time we use this

// function which reads from our UNCONNECTED analog pin which is

// floating and thus will return different numbers.

randomSeed(analogRead(A3));

// Configure OLED display

lander_display.begin();                 	// initialize lander display

lander_display.setFont(u8g2_font_6x10_tr);  // Set text font

lander_display.setFontRefHeightText();  	// Define how max text height is calculated

lander_display.setFontPosTop();         	// Y coordinate for text is at top of tallest character

// Configure counter display

distance_display.setBrightness(7);  // Set maximum brightness (value is 0-7)

distance_display.clear();       	// Clear the display

// Configure DIP switch pins

pinMode(CONFIRM_LEVER_PIN, INPUT);  // switch for bit 0 of our 3 bit value

pinMode(SYSTEMS_LEVER_PIN, INPUT);  // switch for bit 1 of our 3 bit value

pinMode(THRUST_LEVER_PIN, INPUT);   // switch for bit 2 of our 3 bit value

}

// ************************************************

//                 	LOOP()

// ************************************************

void loop(void) {

// Static variables that will retain state between loop() executions

static unsigned long approach_start_time = 0;           	// time thrusters are first fired

static int current_gear_bitmap_index = 0;               	// Image of lander with gear up

static enum APPROACH_STATE approach_state = APPROACH_INIT;  // Initial approach state

static enum GEAR_STATE gear_state = GEAR_IDLE;          	// Inital landing gear state

static int lander_distance = INITIAL_DISTANCE;

static int lander_speed = 0;  // Initial lander speed relative to mother ship

// These will track the ""drift"" of the mother ship from center of radar

static int mother_ship_x_offset = 0;

static int mother_ship_y_offset = 0;

// Read current values of all of our switches as booleans (""on"" is true, ""off"" is false)

bool thrust_lever = digitalRead(THRUST_LEVER_PIN);

bool systems_lever = digitalRead(SYSTEMS_LEVER_PIN);

bool confirm_lever = digitalRead(CONFIRM_LEVER_PIN);

/*

* Primary control state machine.

*

* Based on the current state of our approach to the lander, handle each

* state's decisions, changing state when appropriate.

*/

switch (approach_state) {

// INIT state - Display switch status and ensure all are ""OFF""

case APPROACH_INIT:

// All switches must be ""off"" before our approach sequence can be started

// Since we save these as boolean (true/false) we can do this check on one

// line that could be read as ""if thrust is not true and systems is not true

// and confirm is not true then...""

if (!thrust_lever && !systems_lever && !confirm_lever) {

// All switches are now off, ready for pre-flight

approach_state = APPROACH_PREFLIGHT;  // Change state to Preflight

}

break;

// PREFLIGHT state - All switches off.  Switch all on to begin approach

case APPROACH_PREFLIGHT:

// Once we have enabled thrusters and systems and confirmed we're ready

// then switch to in-flight radar display.

if (thrust_lever && systems_lever && confirm_lever) {  // All switches are ""on""

approach_state = APPROACH_IN_FLIGHT;             	// change to in-flight state

}

break;

// FINAL state - Lower gear, we're about to land!

case APPROACH_FINAL:

// Check to see if a gear change button is pressed.  Do NOT wait for a key.

// NOTE - code then drops into in-flight state!!

// VVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

// IN-FLIGHT state - Add thrust to move closer to mother ship.

// (Remember, you will have to REDUCE thrust as you get closer)

case APPROACH_IN_FLIGHT:

switch (controlButtonPressed()) {

case RAISE_SPEED:

lander_speed++;  // increase velocity

// If this is first time increasing speed then save the start time

if (approach_start_time == 0) {

approach_start_time = millis();

}

break;

case LOWER_SPEED:

// lower speed unless stopped

if (lander_speed > 0) {

lander_speed--;

}

break;

case LOWER_GEAR:                       	// Lower landing gear unless already lowered

if (approach_state == APPROACH_FINAL) {  // Only works on final approach

// Lowering gear is an animation created by changing bitmaps each frame

// until the gear is completely lowered.

if (current_gear_bitmap_index != GEAR_BITMAP_COUNT - 1) {

gear_state = GEAR_LOWERING;  // increases bitmap index until lowered

}

}

break;

case RAISE_GEAR:  // Raise landing gear unless already raised

// Raising gear is an animation created by changing the bitmap index

// each frame until gear is up.

if (current_gear_bitmap_index != 0) {  // Ignore if gear is already up

gear_state = GEAR_RAISING;

}

case STEER_UP:

mother_ship_y_offset++;  // Steer lander one pixel UP

break;

case STEER_DOWN:

mother_ship_y_offset--;  // Steer lander one pixel DOWN

break;

case STEER_LEFT:

mother_ship_x_offset++;  // Steer lander one pixel LEFT

break;

case STEER_RIGHT:

mother_ship_x_offset--;  // Steer lander one pixel RIGHT

break;

case STEER_UP_RIGHT:

mother_ship_x_offset--;  // Steer lander one pixel UP and RIGHT

mother_ship_y_offset++;

break;

case STEER_UP_LEFT:

mother_ship_x_offset++;  // Steer lander one pixel UP and LEFT

mother_ship_y_offset++;

break;

case STEER_DOWN_RIGHT:

mother_ship_x_offset--;  // Steer lander one pixel DOWN and RIGHT

mother_ship_y_offset--;

break;

case STEER_DOWN_LEFT:

mother_ship_x_offset++;  // Steer lander one pixel DOWN and LEFT

mother_ship_y_offset--;

break;

}

// Here we compute the drift of the mother ship using random numbers.

// The mother ship cannot drift off the display, done by setting a

// maximum drift.

const byte MAX_DRIFT = 18;

mother_ship_x_offset += getRandomDrift();  // returns -1, 0 or 1

mother_ship_y_offset += getRandomDrift();  // returns -1, 0 or 1

// Ensure mother ship doesn't drift off our radar display

if (mother_ship_x_offset > MAX_DRIFT) mother_ship_x_offset = MAX_DRIFT;

if (mother_ship_x_offset < -MAX_DRIFT) mother_ship_x_offset = -MAX_DRIFT;

if (mother_ship_y_offset > MAX_DRIFT) mother_ship_y_offset = MAX_DRIFT;

if (mother_ship_y_offset < -MAX_DRIFT) mother_ship_y_offset = -MAX_DRIFT;

// Prepare for landing on final approach

if (lander_distance < (INITIAL_DISTANCE / 10)) {

approach_state = APPROACH_FINAL;  // on final approach - enable gear and warn

}

break;

}

// Because we specified our gear states as 0, 1 or -1 we can change bitmaps by

// simply adding the gear state to our current gear bitmap index.

current_gear_bitmap_index += gear_state;

// If the gear animation has completed (index is either 0 or the index of our last bitmap)

// then we change the gear state to IDLE to complete animation.

if (current_gear_bitmap_index == 0 || current_gear_bitmap_index == GEAR_BITMAP_COUNT - 1) {

gear_state = GEAR_IDLE;

}

// Update our lander display (OLED) using firstPage()/nextPage() methods which

// use a smaller buffer to save memory.  Draw the exact SAME display each time

// through this loop!

lander_display.firstPage();

do {

switch (approach_state) {

// Display switch status for INIT and PREFLIGHT states.

case APPROACH_INIT:

case APPROACH_PREFLIGHT:

displayPreFlight(approach_state, thrust_lever, systems_lever, confirm_lever);

break;

case APPROACH_FINAL:

displayFinal(current_gear_bitmap_index);

case APPROACH_IN_FLIGHT:

displayInFlight(lander_distance, lander_speed,

mother_ship_x_offset, mother_ship_y_offset);

break;

}

} while (lander_display.nextPage());

lander_distance -= lander_speed;  // Adjust distance by current speed

// END OF FLIGHT!

//

// When we reach the mother ship (distance <= 0) then determine success

// or failure and display finaly screen alternating with last radar image.

char* ending_bitmp;

if (lander_distance <= 0) {

// Check to see if our lander landed inside the mother ship target box.

if (abs(mother_ship_x_offset) < ((MAX_MOTHER_SHIP_WIDTH + 1) / 2)

&& abs(mother_ship_y_offset) < ((MAX_MOTHER_SHIP_HEIGHT + 1) / 2)) {

lander_distance = 0;

// We hit the target.  Now check speed to see if we were slow enough

if (lander_speed <= 2) {  // Max safe landing speed is 2

// Speed OK, but did we remember to lower the landing gear?

if (current_gear_bitmap_index == GEAR_BITMAP_COUNT - 1) {  // Gear is down!

ending_bitmp = ENDING_BITMAP_SUCCESS;                	// SUCCESS!

} else {

// Oops, gear is up.  Damage to lander, but we survived.

ending_bitmp = ENDING_BITMAP_NO_GEAR;

}

} else {

// Speed is too fast!  Lander AND mother ship destroyed.  (Ouch!)

ending_bitmp = ENDING_BITMAP_TOO_FAST;

}

} else {

// Missed the mother ship.  No fuel for another try.  Bye!

ending_bitmp = ENDING_BITMAP_MISSED_MOTHER_SHIP;

}

// Show final distance

distance_display.showNumberDec(0);

// Calculate elapsed time (in ms) from first thrust.

unsigned long elapsed_time = millis() - approach_start_time;

// Now format to fractional seconds (SS.SSS) using sprintf

char buffer[20];  // buffer long enough for final display line.

// Since elapse time is a long integer we can get the number of seconds

// by simply dividing by 1000 and the fractional portion is dropped.  The

// Modulo operator ('%') returns the REMAINDER after dividing left side by

// the value on the right.  This gives us the fractional number seconds.

// Sprintf() uses ""lu"" to indicate that the value is a ""unsigned long""

// (""lu"" does NOT work!).

sprintf(buffer, ""%4lu.%03lu Sec"", elapsed_time / 1000, elapsed_time % 1000);

// Final display.  Alternate between splash screen with time and

// final radar view.

do {

lander_display.firstPage();

do {

lander_display.drawStr(0, 0, buffer);

lander_display.drawXBMP(0, 10, ENDING_BITMAP_WIDTH, ENDING_BITMAP_HEIGHT, ending_bitmp);

} while (lander_display.nextPage());

delay(2000);

lander_display.firstPage();

do {

displayFinal(current_gear_bitmap_index);

displayInFlight(lander_distance, lander_speed,

mother_ship_x_offset, mother_ship_y_offset);

} while (lander_display.nextPage());

delay(2000);

} while (true);  // repeat this final display until HERO is reset

// NEVER REACHED.  THIS IS OUR FINAL EXECUTION OF LOOP();

}

delay(100);  // Delay 1/10 second before next loop

}

// ************************************************

//         	Preflight display

// ************************************************

// Update lander display with the status of our switches for INIT and PREFLIGHT states

void displayPreFlight(enum APPROACH_STATE approach_state,

bool thruster_lever,

bool systems_lever,

bool confirm_lever) {

// Display all text referenced from upper left bit X, Y

lander_display.setFontPosTop();

// Draw title lines at top of display, updating y_offset afterwards

byte y_offset = drawString(0, 0, ""Exploration Lander"");

y_offset = drawString(0, y_offset, ""Approach Sequence"");

// Set y_offset to point four lines above bottom of display

y_offset = lander_display.getDisplayHeight() - (4 * lander_display.getMaxCharHeight());

// Display status of each switch

y_offset = drawString(0, y_offset,

(String(""Thrusters: "") + String(thruster_lever ? ""ON"" : ""OFF"")).c_str());

y_offset = drawString(0, y_offset,

(String(""Systems  : "") + String(systems_lever ? ""ON"" : ""OFF"")).c_str());

y_offset = drawString(0, y_offset,

(String(""Confirm  : "") + String(confirm_lever ? ""ON"" : ""OFF"")).c_str());

// Display final status line

drawString(0, y_offset, (String(""Countdown "") + liftoffStateToString(approach_state)).c_str());

}

// ************************************************

//   In-Flight display (including final approach)

// ************************************************

// Radar display for steering into mother ship docking bay.

void displayInFlight(int lander_distance,

int lander_speed,

int mother_ship_x_offset,

int mother_ship_y_offset) {

// Mother ship initially appears as a single dot, but expands into a rectangle

// as we get closer.  Scaled based on the maximum width, from 1 to MAX.

//

// Divide distance to mother ship into segments.  The number of segments is

// one less than the width of our mother ship.

const unsigned int SEGMENT_SIZE = INITIAL_DISTANCE / (MAX_MOTHER_SHIP_WIDTH - 1);

byte segment_number = lander_distance / SEGMENT_SIZE;  // from (width - 1) to 0

// subtract segment number from width/height to get visible width (minimum 1)

int mother_ship_width = MAX_MOTHER_SHIP_WIDTH - segment_number;

int mother_ship_height = MAX_MOTHER_SHIP_HEIGHT - segment_number;

if (mother_ship_height < 1) {

mother_ship_height = 1;  // Always at least 1 pixel high

}

// Display distance to mother ship on our distance display

distance_display.showNumberDec(lander_distance);

// coordinates of the center of our radar display

const byte RADAR_CENTER_X = (lander_display.getDisplayWidth() / 2 / 2);  // center of left half

const byte RADAR_CENTER_Y = (lander_display.getDisplayHeight() / 2); 	// Vertical center

const byte RADAR_RADIUS = 25;

// Display bitmaps with 0 bits set to transparent.  This allows us to overlay

// our arrows over the radar circle cleanly.

lander_display.setBitmapMode(1);

// Draw radar display circle and center pointer dot.

lander_display.drawCircle(RADAR_CENTER_X, RADAR_CENTER_Y, RADAR_RADIUS);

lander_display.drawPixel(RADAR_CENTER_X, RADAR_CENTER_Y);

//====================================================================

//====================================================================

/*

* As the mother ship's radar return drifts on the screen this section

* of the code will display an arrow on the outside of the radar circle.

*

* The first constants can be changed so that the arrows don't appear

* until the target pointer is outside the landing bay box, but currently

* the arrows appear after small amounts of drift.

*

* The coordinates for the arrows were originally generated by ChatGPT

* and then individually tweeked for perfect location.  Because of this

* the coordinates will need to be manually adjusted if the radius of

* the circle is changed.

*/

const int DRIFT_BEFORE_ARROW_X = 2;  // try (mother_ship_width / 2) - 2; for late arrows

const int DRIFT_BEFORE_ARROW_Y = 2;  // try (mother_ship_height / 2) - 2; for late arrows

// Outer set of if/else statements check for left/right drift (X coordinate)

if (mother_ship_x_offset < -DRIFT_BEFORE_ARROW_X) {  // left of center

// Inner set of if/else statements check for up/down drift (Y coordinate)

if (mother_ship_y_offset < -DRIFT_BEFORE_ARROW_Y) {                         	// above center

lander_display.drawXBMP(9, 9, ARROW_SIZE_X, ARROW_SIZE_Y, ARROW_UP_LEFT); 	// Up-Left

} else if (mother_ship_y_offset > DRIFT_BEFORE_ARROW_Y) {                   	// below center

lander_display.drawXBMP(8, 45, ARROW_SIZE_X, ARROW_SIZE_Y, ARROW_DOWN_LEFT);  // Down-Left

} else {                                                                    	// close to center in Y direction

lander_display.drawXBMP(1, 27, ARROW_SIZE_X, ARROW_SIZE_Y, ARROW_LEFT);   	// Left

}

} else if (mother_ship_x_offset > DRIFT_BEFORE_ARROW_X) {                       	// Right of center

if (mother_ship_y_offset < -DRIFT_BEFORE_ARROW_Y) {                           	// above center

lander_display.drawXBMP(45, 8, ARROW_SIZE_X, ARROW_SIZE_Y, ARROW_UP_RIGHT); 	// Up-Right

} else if (mother_ship_y_offset > DRIFT_BEFORE_ARROW_Y) {                     	// below center

lander_display.drawXBMP(45, 45, ARROW_SIZE_X, ARROW_SIZE_Y, ARROW_DOWN_RIGHT);  // Down-Right

} else {                                                                      	// close to center in Y direction

lander_display.drawXBMP(53, 27, ARROW_SIZE_X, ARROW_SIZE_Y, ARROW_RIGHT);   	// Right

}

} else {                                                                  	// close to center in X direction

if (mother_ship_y_offset < -DRIFT_BEFORE_ARROW_Y) {                     	// above center

lander_display.drawXBMP(27, 1, ARROW_SIZE_X, ARROW_SIZE_Y, ARROW_UP); 	// Up

} else if (mother_ship_y_offset > DRIFT_BEFORE_ARROW_Y) {               	// below center

lander_display.drawXBMP(27, 53, ARROW_SIZE_X, ARROW_SIZE_Y, ARROW_DOWN);  // Down

}                                                                       	// else NO arrow as we're close to target

}

// We will use sprintf() to format the lander speed line

char buffer[9];                                                           	// Length of line plus one for terminating null.

sprintf(buffer, ""SPD: %2d"", lander_speed);                                	// format speed into buffer

int width = lander_display.getStrWidth(buffer);                           	// width of string in pixels

lander_display.drawStr(lander_display.getDisplayWidth() - width, 0, buffer);  // display in upper right

// Now draw the image of the mother ship as a rectangle, sized based on distance and

// offset the x/y coordinates so that ship offset is always from the CENTER of the

// mother ship.

byte x_offset = RADAR_CENTER_X + mother_ship_x_offset - (mother_ship_width / 2);

byte y_offset = RADAR_CENTER_Y + mother_ship_y_offset - (mother_ship_height / 2);

lander_display.drawFrame(x_offset, y_offset, mother_ship_width, mother_ship_height);

}

// ************************************************

// When lander is about to land we'll add an image of the lander

// with landing gear status along with gear prompt messages.

void displayFinal(int current_gear_bitmap_index) {

int gear_down_index = GEAR_BITMAP_COUNT - 1;  // index of the LAST bitmap image

// Coordinates below  manually adjusted to display in upper right

// two lines below the speed line.

byte x_offset = (lander_display.getDisplayWidth() / 2) + 11;

byte y_offset = lander_display.getMaxCharHeight() * 2;  // offset to third line

if (current_gear_bitmap_index == 0) {               	// Gear is up.  Prompt to lower

lander_display.drawStr(x_offset, y_offset, ""Drop gear"");

} else if (current_gear_bitmap_index < gear_down_index) {  // Gear is lowering

lander_display.drawStr(x_offset, y_offset, ""Lowering"");

} else {

lander_display.drawStr(x_offset, y_offset, ""Gear OK"");  // Gear is down ()

}

// Calculate our x and y offsets to center our bitmap graphics

// Get offsets to lower-right area of screen.  Height allows 3 lines of text above

x_offset = (lander_display.getDisplayWidth() / 2);  // 64

x_offset += ((lander_display.getDisplayWidth() - x_offset) - LANDING_GEAR_BITMAP_WIDTH) / 2;

y_offset = lander_display.getDisplayHeight() - (lander_display.getMaxCharHeight() * 3);

y_offset += ((lander_display.getDisplayHeight() - y_offset) - LANDING_GEAR_BITMAP_HEIGHT) / 2;

// Draw current bitmap centered in lower right quadrant

lander_display.drawXBMP(x_offset, y_offset,

LANDING_GEAR_BITMAP_WIDTH, LANDING_GEAR_BITMAP_HEIGHT,

LANDER_BITMAPS[current_gear_bitmap_index]);

}

// ************************************************

// ""helper"" function that returns a different string for each enum state.

String liftoffStateToString(enum APPROACH_STATE approach_state) {

switch (approach_state) {

case APPROACH_INIT:

return (""Init"");

break;

case APPROACH_PREFLIGHT:

return (""Preflight"");

break;

}

}

// ************************************************

// Draw a line of text on our OLED display at x, y, returning new y

// value that is immediately below the new line of text.

byte drawString(byte x, byte y, char* string) {

lander_display.drawStr(x, y, string);

return (y + lander_display.getMaxCharHeight());  // return new y_offset on display

}

// They Keypad library only returns a button value when it is initially

// pressed.  We wish to continue to get the same value for as long as

// a button is pressed.  This function keeps track of the last button

// pressed and returns it until the button is released or another

// button is pressed.

enum LANDER_CONTROLS controlButtonPressed() {

static char last_key = NO_KEY;

char current_key = lander_controls.getKey();

if (current_key != NO_KEY) {

last_key = current_key;  // New key seen, save as last

}

// If the button is released reset to NO_KEY

if (lander_controls.getState() == RELEASED) {

last_key = NO_KEY;

current_key = NO_KEY;

} else {

current_key = last_key;  // button still pressed so return last button seen.

}

return (current_key);

}

const byte DRIFT_CONTROL = 3;  // Must be > 1.  Higher numbers slow drift rate

int getRandomDrift() {

// Random(low, high) returns a random number from low to ONE LESS THAN high.

// We use -1, 0 or 1 for the drift.  To *slow* the drift we can increae the

// number of values that indicate ""no drift"" by generating numbers higher

// than 1 and if the random number is higher we set drift to 0.

// With a drift control of 3 we get random numbers (-1, 0, 1, 2) and if 2 we

// change to 0, doubleing the times we get NO drift.

int drift = random(-1, DRIFT_CONTROL);

if (drift > 1) {  // Values over 1 are changed to 0

drift = 0;

}

return (drift);

}"

Day 30 is the finale, it is a creative day and there is no circuits or script for this day.